3GPP TSG-RAN WG3 Meeting #128	R3-253661

Malta, MT, May 19th – 23rd, 2025

Agenda item:	13.3

Source: 	LG Electronics Inc.

Title: 	(TP for NR_Mob_Ph4 TS 38.473) Discussions on the remaining aspects of Conditional Intra-CU LTM

Document for:	Other

# Introduction

For Rel-19 Conditional Intra-CU LTM, RAN3-127bis (Wuhan) has progressed as follows:

RAN3-127bis (Wuhan, April 2025)

For CLTM focus firstly on scenarios without CU-UP change.

The gNB-DU signals the generated L1 execution conditions over F1AP for CLTM via a new IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages. Details FFS to be continued.

Reuse the ACCESS SUCCESS message from Candidate DU to CU to inform the cell change.

For Inter-DU CLTM, reuse the UE CONTEXT MODIFICATION REQUEST message from CU to Source DU to inform the cell change and stop data transmission.

For CLTM, the existing LTM network signaling for LTM Cell Switch Completion phase is reused.

FFS whether a cell can be simultaneously configured for LTM and CLTM.

FFS whether its needed that for each candidate cell, the CU indicates to the source DU and the candidate DUs in the LTM Configuration ID Mapping List the need of generating L1 execution condition.

Whether needs to indicate from CU to source DU and candidate DU when the cell is configured with L3 CLTM, separately from L1 CLTM.

In this contribution, we discuss the remaining issues for completion of Intra-CU LTM designs for C-LTM and proposes the TP for 38.473 BL CR [1].

# Discussion

## Remaining issues for Intra-CU Conditional LTM preparation

RAN3 already agreed to leverage the existing F1AP signaling framework designed at Rel-18 LTM as the foundation for Rel-19 C-LTM. Given this situation, we believe there is no reason not to harmonize C-LTM with the existing LTM and allow flexibility to configure a UE such that traditional LTM is applied for one candidate cell, while C-LTM operates for another candidate cell, all through a single RRC configuration upfront.

With that, regarding the following FFS:

FFS whether a cell can be simultaneously configured for LTM and CLTM.

From the RAN2 RRC signaling structure [2], it is indeed true that the design permits a candidate cell to be simultaneously configured for both traditional network-triggered LTM and Conditional LTM (C-LTM). Specifically, a UE may be configured with C-LTM for a candidate cell via the ltm-ExecutionCondition-r19 field in the candidate cell’s LTM-Candidate-r18, while the same cell can also configure traditional LTM and L1 or L3 measurement reporting through LTM-Candidate-r18 > ltm-CandidateConfig-r18 (= RRCReconfiguration). This shows that RRC design supports simultaneous configuration of LTM and C-LTM for a given candidate cell, and it is up to NW to configure them accordingly (either LTM only, or C-LTM only, or both).

Moreover, note that the existing LTM Indicator IE (set to “true”) has been used for DU to be aware of traditional LTM preparation, and the CSI Resource Configuration IE was a differentiator on whether DU needs to generate the CSI-RS reporting configuration for the UE’s L1 measurement reporting or not. Now as RAN2 already agreed, C-LTM execution can also be L1-based or L3-based. Also, according to the RAN2’s RRC design [2], L1-based C-LTM execution condition leverages the existing CSI-RS report configuration ID, which seems to mean that it requires DU to know the CSI-RS resources to be measured and evaluated, and CSI-RS report configuration shall also be generated and configured to the UE:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### –	LTM-ExecutionConditionList

The IE LTM-ExecutionConditionList is used to configure LTM cell switch conditions.

LTM-ExecutionConditionList information element

-- ASN1START

-- TAG-LTM-EXECUTIONCONDITIONLIST-START

LTM-ExecutionConditionList-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-ExecutionCondition-r19

LTM-ExecutionCondition-r19 ::= SEQUENCE {

ltm-CandidateId-r19                            LTM-CandidateId-r18,

executionCondition-r19                         CHOICE {

l1-Conditions-r19                              LTM-CSI-ReportConfigId-r18,

l3-Conditions-r19                              SEQUENCE (SIZE (1..2)) OF MeasId

}                                                                                                    OPTIONAL,   -- Need R

...

}

-- TAG-LTM-EXECUTIONCONDITIONLIST-STOP

-- ASN1STOP

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Furthermore, even for C-LTM, NW can still configure L1 or L3 measurement reports to the UE, to trigger early TA acquisition. This means that, in case of L1-based measurement reports, the existing mechanism to request DU to generate CSI-RS reporting configuration (by providing the CSI-RS resource configuration) is better to be maintained and leveraged for C-LTM as well.

RAN2-128 (Orlando) agreements on C-LTM:

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

Considering all these observations, we think the most straightforward and modular way to cover all the possible combinations (while keeping the existing behavior by the LTM Indicator IE set to “true”) is to

- Expend the codepoints of the existing LTM Indicator IE for the possibilities of “C-LTM only” and “LTM and C-LTM together” in high-level;

- Independently control DU to generate CSI-RS reporting configuration by supplying the CSI-RS Resource Configuration or not (same as LTM);

- Define additional IE to independently request DU to generate and supply L1-based execution conditions (applicable when the expanded LTM Indicator IE is set to either “C-LTM only” or “LTM and C-LTM together”)

Proposal 1: As allowed by RAN2’s RRC signalling design, RAN3 to allow flexibility for CU to prepare both traditional LTM and C-LTM even for the same candidate cell.

Proposal 2: To cover all the possible combinations and follow RAN2’ RRC signalling design for L1-based C-LTM execution condition leveraging the existing CSI-RS report configuration ID (while keeping the existing behavior by the F1AP LTM Indicator IE set to “true”),

- Expend the codepoints of the existing LTM Indicator IE for the possibilities of “C-LTM only” and “LTM and C-LTM together” in high-level;

- Independently control DU to generate CSI-RS reporting configuration by supplying the CSI-RS Resource Configuration or not (same as LTM);

- Define additional IE to independently request DU to generate and supply L1-based execution conditions (applicable when the expanded LTM Indicator IE is set to either “C-LTM only” or “LTM and C-LTM together”)

Moreover, regarding the following FFS:

FFS whether its needed that for each candidate cell, the CU indicates to the source DU and the candidate DUs in the LTM Configuration ID Mapping List the need of generating L1 execution condition.

In order for DU to generate L1-based execution condition (requested by CU), it needs to know for which target candidate cells it needs to generate the execution conditions. Originally, we proposed to leverage the LTM Configuration ID Mapping List IE as a way to indicate such. However, if LTM and C-LTM can be configured simultaneously, this meaning of this IE shall not be differently interpreted for LTM and C-LTM. It is better to define a separate IE to control explicitly the target candidate cells for which the execution conditions are generated. But for this IE, the same structure as the LTM Configuration ID Mapping List can be re-used.

Proposal 3: Considering that LTM and C-LTM can be configured simultaneously for the same candidate cell, do not use the existing optional LTM Configuration ID Mapping List IE as a way to inform DU of which target cells to generate L1 execution conditions.

Proposal 4: Instead, define a new IE for CU to request a DU to supply L1 execution conditions, with directly indicating which cells to generate (by referring to 9.3.1.294 LTM Configuration ID Mapping List).

## L1 execution condition encoding

The gNB-DU signals the generated L1 execution conditions over F1AP for CLTM via a new IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages. Details FFS to be continued.

According to RAN2’s RRC design [2], the serving cell and each candidate cell’s LTM configuration includes a list of execution conditions toward (other) candidate cells as a list:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### –	LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...,

[[

ltm-NoSecurityChangeID-r19                     LTM-NoSecurityChangeId-r19                            OPTIONAL,    -- Need M

ltm-ExecutionCondition-r19                     SetupRelease {LTM-ExecutionConditionList-r19}         OPTIONAL     -- Need M

]]

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

LTM-NoSecurityChangeId-r19 ::= INTEGER (1..maxNrofLTM-Configs-plus1-r18)

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

ltmExecutionCondition This field can only be included within an ltm-Config associated with the MCG.

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID.

ltm-NoSecurityChangeID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID.

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. This field is absent if tag2 is present for this LTM candidate configuration.

#### –	LTM-Config

The IE LTM-Config is used to provide LTM configurations.

LTM-Config information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Cond NR-DC

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...,

[[

ltm-ServingCellNoSecurityChangeID-r19       LTM-NoSecurityChangeId-r19                                          OPTIONAL,   -- Need N

ltm-ServingCellExecutionCondition-r19       SetupRelease {LTM-ExecutionConditionList-r19}                       OPTIONAL    -- Need M

]]

}

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

LTM-Config field descriptions

ltm-ServingCellExecutionCondition This field can can only be included within an ltm-Config associated with the MCG.

| Conditional Presence | Explanation |
| --- | --- |
| LTM-MCG | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |
| NR-DC | If ltm-ConfigNRDC is configured, this field is optionally present, Need M, within ltm-ConfigNRDC and absent within ltm-Config. If ltm-ConfigNRDC is not configured, this field is optionally present, Need M, within ltm-Config. |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### –	LTM-ExecutionConditionList

The IE LTM-ExecutionConditionList is used to configure LTM cell switch conditions.

LTM-ExecutionConditionList information element

-- ASN1START

-- TAG-LTM-EXECUTIONCONDITIONLIST-START

LTM-ExecutionConditionList-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-ExecutionCondition-r19

LTM-ExecutionCondition-r19 ::= SEQUENCE {

ltm-CandidateId-r19                            LTM-CandidateId-r18,

executionCondition-r19                         CHOICE {

l1-Conditions-r19                              LTM-CSI-ReportConfigId-r18,

l3-Conditions-r19                              SEQUENCE (SIZE (1..2)) OF MeasId

}                                                                                                    OPTIONAL,   -- Need R

...

}

-- TAG-LTM-EXECUTIONCONDITIONLIST-STOP

-- ASN1STOP

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

And, as RAN2-128 agreed as follow:

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

This means that the C-LTM configuration of each candidate cell also includes execution condition generated by the initial source cell. To cope with this, at the last meeting RAN3 discussed and agreed for DU to supply the generated L1 execution conditions (each toward another candidate cell) as a list, so that CU can accordingly re-compile and re-compose the LTM-ExecutionConditionList for the current serving cell and each candidate cell. This was reflected into the F1AP BL CR [1] as follows.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 9.3.1.XXX		Conditional LTM Execution Condition List

This IE indicates the list of conditional LTM execution conditions to be used by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Conditional LTM Execution Condition Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Execution Condition | M |  | OCTET STRING (FFS) | Up to RAN2 | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

As a result, for each item in the L1 execution condition list supplied from DU, LTM-CSI-ReportConfigId-r18 shall be referenced for the OCTET STRING.

Proposal 5: For each item in the L1 execution condition list supplied from DU, OCTET STRING to refer LTM-CSI-ReportConfigId-r18 according to RAN2’s RRC design.

## Inter-DU C-LTM execution

As illustrated in Figure, 1, the previous serving DU, which is unaware that an Inter-DU C-LTM has been executed by the UE, requires additional notification from the CU to properly stop data transmission for the UE, revert the previous serving cell to a prepared state (if it remains one of the configured LTM candidate cells), and inform any DL packets not successfully delivered to the UE for retransmissions by the CU:

Figure 1: Inter-DU C-LTM execution procedure

Regarding this, RAN3-127bis agreed the following:

For Inter-DU CLTM, reuse the UE CONTEXT MODIFICATION REQUEST message from CU to Source DU to inform the cell change and stop data transmission.

For this purpose, at Rel-18 S-CPAC, the Transmission Action Indicator IE in UE CTXT MOD REQ has been used. This IE can be re-used and extended for C-LTM as follows:

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC or Conditional LTM and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

Proposal 6: To inform the previous serving DU and stop data transmission after Inter-DU C-LTM execution happens, re-use the existing Transmission Action Indicator IE (used for S-CPAC) in UE CTXT MOD REQ message.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

## Remaining issues for Intra-CU Conditional LTM preparation

Proposal 1: As allowed by RAN2’s RRC signalling design, RAN3 to allow flexibility for CU to prepare both traditional LTM and C-LTM even for the same candidate cell.

Proposal 2: To cover all the possible combinations and follow RAN2’ RRC signalling design for L1-based C-LTM execution condition leveraging the existing CSI-RS report configuration ID (while keeping the existing behavior by the F1AP LTM Indicator IE set to “true”),

- Expend the codepoints of the existing LTM Indicator IE for the possibilities of “C-LTM only” and “LTM and C-LTM together” in high-level;

- Independently control DU to generate CSI-RS reporting configuration by supplying the CSI-RS Resource Configuration or not (same as LTM);

- Define additional IE to independently request DU to generate and supply L1-based execution conditions (applicable when the expanded LTM Indicator IE is set to either “C-LTM only” or “LTM and C-LTM together”)

Proposal 3: Considering that LTM and C-LTM can be configured simultaneously for the same candidate cell, do not use the existing optional LTM Configuration ID Mapping List IE as a way to inform DU of which target cells to generate L1 execution conditions.

Proposal 4: Instead, define a new IE for CU to request a DU to supply L1 execution conditions, with directly indicating which cells to generate (by referring to 9.3.1.294 LTM Configuration ID Mapping List).

## L1 execution condition encoding

Proposal 5: For each item in the L1 execution condition list supplied from DU, OCTET STRING to refer LTM-CSI-ReportConfigId-r18 according to RAN2’s RRC design.

## Inter-DU C-LTM execution

Proposal 6: To inform the previous serving DU and stop data transmission after Inter-DU C-LTM execution happens, re-use the existing Transmission Action Indicator IE (used for S-CPAC) in UE CTXT MOD REQ message.

The corresponding F1AP TP is provided in Section 5.

# Reference

[1] R3-253149, “(BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM”, Huawei, Nokia, Samsung, Google, NEC, China Telecom, Ericsson, LG Electronics, CATT, Ofinno, ZTE, Lenovo, Qualcomm Incorporated

[2] R2-2504125, “Running RRC CR for inter-CU and conditional LTM”, Ericsson

# TP for TS 38.473 BL CR (R3-253149)

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 8.3	UE Context Management procedures

### 8.3.1	UE Context Setup

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.1.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the LTM Indicator IE set to "true" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the LTM Indicator IE set to "C-LTM-only" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns Conditional LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the LTM Indicator IE set to "LTM-and-C-LTM" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns to configure LTM and Conditional LTM simultaneously for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, provide the lower layer configuration in the Reference Configuration Information IE in the LTM ConfigurationIE in the UE CONTEXT SETUP RESPONSE message for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig IE for the requested LTM candidate cell.

If the L1 Execution Condition Request IE is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the Conditional LTM execution conditions from the requested LTM candidate cell identified by the included SpCell ID IE toward the indicated LTM candidate cells, and provide the L1 Execution Condition List IE in the UE CONTEXT SETUP RESPONSE message.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation), in the UE CONTEXT SETUP RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the LTM Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the candidate gNB-DU.

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.4	UE Context Modification (gNB-CU initiated)

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.3.4.2	Successful Operation

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the LTM Indicator IE set to "true" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the LTM Indicator IE set to "C-LTM-only" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns Conditional LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the LTM Indicator IE set to " LTM-and-C-LTM " is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns to configure LTM and Conditional LTM simultaneously for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the Reference Configuration Information IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message to provide lower layer configuration for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the requested LTM candidate cell identified by the SpCell ID IE.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the serving cell.

If the L1 Execution Condition Request IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the Conditional LTM execution conditions from the requested LTM candidate cell identified by the included SpCell ID IE toward the indicated LTM candidate cells, and provide the L1 Execution Condition List IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the L1 Execution Condition Request IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the Conditional LTM execution conditions from the serving cell toward the indicated LTM candidate cells, and provide the L1 Execution Condition List IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation) in the UE CONTEXT MODIFICATION RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the Early Sync Candidate Cell Information List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Candidate Cell Information List IE for some candidate cell, the gNB-DU shall, if supported, take them into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the Early Sync Serving Cell Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Serving Cell Information IE, the gNB-DU shall, if supported, take it into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the LTM CFRA Resource Config List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [16].

If the LTM Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the gNB-DU.

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the LTM Reset Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch as specified in TS38.331 [8].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the LTM Security Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store it and take it into account for supporting the UE’s AS security continuation during an inter-CU LTM cell switch and act as specified in TS 38.401 [4] and TS 38.321 [16].

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC or Conditional LTM and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | reject |
| gNB-CU UE F1AP ID |  | M |  |  |  | 9.3.1.4 |  |  |  | YES |  | reject |
| gNB-DU UE F1AP ID |  | O |  |  |  | 9.3.1.5 |  |  |  | YES |  | ignore |
| SpCell ID |  | M |  |  |  | NR CGI 9.3.1.12 |  | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. |  | YES |  | reject |
| ServCellIndex |  | M |  |  |  | INTEGER (0..31,...) |  |  |  | YES |  | reject |
| SpCell UL Configured |  | O |  |  |  | Cell UL Configured 9.3.1.33 |  |  |  | YES |  | ignore |
| CU to DU RRC Information |  | M |  |  |  | 9.3.1.25 |  |  |  | YES |  | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |
| LTM Information Setup |  |  | 0..1 |  |  |  |  |  | YES |  | reject |  |
| >LTM Indicator | M |  |  |  | ENUMERATED (true, …, C-LTM-only, LTM-and-C-LTM) |  |  |  | - |  |  |  |
| >Reference Configuration | O |  |  |  | 9.3.1.292 |  |  |  | - |  |  |  |
| >CSI Resource Configuration | O |  |  |  | 9.3.1.330 |  |  |  | - |  |  |  |
| >L1 Execution Condition Request | O |  |  |  | 9.3.1.294 |  | Can be included only when the LTM Indicator IE is set to “C-LTM-only” or “LTM-and-C-LTM”. |  | - |  |  |  |
| LTM Configuration ID Mapping List | O |  |  |  | 9.3.1.294 |  |  |  | YES |  | reject |  |
| Early Sync Information Request |  |  | 0..1 |  |  |  |  |  | YES |  | ignore |  |
| >Request for RACH Configuration | M |  |  |  | ENUMERATED (true, …) |  |  |  | - |  |  |  |
| >LTM gNB-DUs List |  |  | 1 |  |  |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). |  | YES |  | reject |  |
| >>LTM gNB-DUs Item IEs |  |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |  | - |  |  |  |
| >>>LTM gNB-DU ID | M |  |  |  | gNB-DU ID 9.3.1.9 |  |  |  | - |  |  |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell. | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >L1 Execution Condition List | O |  | 9.3.1.XXX | Includes the generated L1 Execution Conditions for Conditional LTM. | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …, C-LTM-only, LTM-and-C-LTM) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >L1 Execution Condition Request | O |  | 9.3.1.294 | Can be included only when the LTM Indicator IE is set to “C-LTM-only” or “LTM-and-C-LTM”. | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |
| LTM Security Information | O |  | 9.3.1.XX |  | YES | reject |

#### 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >L1 Execution Condition List | O |  | 9.3.1.XXX | Includes the generated L1 Execution Conditions for Conditional LTM. | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

/*********************Next change***********************/

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 9.3.1	Radio Network Layer Related IEs

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.1.XXX		Conditional LTM Execution Condition List

This IE indicates the list of conditional LTM execution conditions to be used by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Conditional LTM Execution Condition Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Execution Condition | M |  | OCTET STRING | Includes the LTM-CSI-ReportConfigId-r18, as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# Annex A (RAN3 agreement for Conditional Intra-CU LTM)

## RAN3-127bis (Wuhan, April 2025)

For CLTM focus firstly on scenarios without CU-UP change.

The gNB-DU signals the generated L1 execution conditions over F1AP for CLTM via a new IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages. Details FFS to be continued.

Reuse the ACCESS SUCCESS message from Candidate DU to CU to inform the cell change.

For Inter-DU CLTM, reuse the UE CONTEXT MODIFICATION REQUEST message from CU to Source DU to inform the cell change and stop data transmission.

For CLTM, the existing LTM network signaling for LTM Cell Switch Completion phase is reused.

FFS whether a cell can be simultaneously configured for LTM and CLTM.

FFS whether its needed that for each candidate cell, the CU indicates to the source DU and the candidate DUs in the LTM Configuration ID Mapping List the need of generating L1 execution condition.

Whether needs to indicate from CU to source DU and candidate DU when the cell is configured with L3 CLTM, separately from L1 CLTM.

## RAN3-127 (Athens, February 2025)

The CU decides to configure C-LTM.

The CU determines which type of C-LTM to initiate: L1 event-triggered C-LTM or L3 event-triggered C-LTM.

Reuse the current LTM preparation procedure (i.e. UE Context Setup/Modification procedure) for DU to generate the C-LTM L1 execution condition.

UE Context Setup procedure for the candidate DU to generate the subsequent C-LTM L1 execution condition.

UE Context Modification procedure for the source DU to generate the C-LTM L1 execution condition.

Indicator for C-LTM shall be added in the UE CONTEXT SETUP/MODIFICATION REQUEST message. FFS for the detail.

For L1 measurement based C-LTM, to support Early Sync, the existing F1: DU-CU/ CU-DU TA Information Transfer is reused.

# Annex B (RAN2 agreement for Conditional Intra-CU LTM)

## RAN2-129bis (Wuhan, April 2025)

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA.

Define a per-band capability for L1 execution condition, e.g. cltm-ExecutionConditionL1-r19 is defined to indicate whether the UE supports L1 execution condition for CLTM and subsequent CLTM.

Define a per-band capability for L3 execution condition, e.g. cltm-ExecutionConditionL3-r19 is defined to indicate whether the UE supports L3 execution condition for CLTM and subsequent CLTM and whether the UE supports 2 trigger events for same execution condition.

A UE that supports conditional LTM should indicate the support for at least one of cltm-ExecutionConditionL3-r19 or cltm-ExecutionConditionL1-r19.

When a UE indicates support for both conditional LTM and ltm-RACH-LessCG-r18, it implies that the UE supports RACH-less conditional LTM with a configured grant. Whether/how to update the field description of ltm-RACH-LessCG-r18 can be addressed in the running CR review.

Rely on the R18 capability (i.e., ue-TA-Measurement-r18) to indicate whether UE supports UE-based TA measurement for C-LTM. There is no need to define a separate capability for this purpose.

Define a new per UE capability for UE support of early TA MAC CE reception for CLTM and also the max number of maintaining TA values. The value range is (1~8).

Rely on the R18 capability (i.e., ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18) to indicate whether the UE supports MAC-CE activated joint/separate LTM TCI states for CLTM.

## RAN2-129 (Athens, February 2025)

Agreements on MAC issues:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

- Absolute TA value is included in the MAC CE for CLTM early TA acquisition.

- The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE.

- RAN2 assumes the CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE and in the candidate cell TCI state Activation/Deactivation MAC CE, pending RAN1 confirmation.

- The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

- The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

- The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam.

- There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell.

- Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader.

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

## RAN2-128 (Orlando, November 2024)

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

## RAN2-127bis (Hefei, October 2024)

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.