3GPP TSG-RAN WG3 #128	R3-253675

St. Julian - Malta, 19 – 23 May. 2025

Agenda Item:	18.2. Support LP-WUS Indicating Paging Monitoring

Source:	NTT DOCOMO INC.

Title:	Further discussion on LP-WUS

Document for:	Discussion, agreement

Introduction

In past meetings, RAN3 achieved following:

[RAN3#125]

- New IEs should be introduced to support LP-WUS on NG, Xn and F1.

[RAN3#125bis]

- The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

- The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

- The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

- Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

- The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

[RAN3#126]

- WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

- It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

- NG-RAN node can acquire the total number of CN assigned subgrouping via OAM.

- WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

[RAN3#127]

- For the following issues, including Extended UE Identity Index、The maximum value of CN subgroup ID、Emergency call back、the terminology of “LP-WUS”、 the LP-WUS node capability need more inputs from other WGs.

[RAN3#127bis]

- The maximum number of subgroups is decided to 31.

- Turn the working assumption into agreement and remove the FFS on the presence of the LP-WUS Subgrouping Support Indication IE per cell in the F1AP Paging message.

- RAN3 will not send an LS to SA3 regarding this issue.

- Whether Extended UE Identity Index or Hashed UE_ID is used and how many bits are needed for UE_ID is pending on RAN2.

This document summaries current situations and discussions.

Discussion

Stage 3

At the previous meeting, it was agreed that the maximum number of subgroups should be 31. In the existing PEIPS Assistance Information, subgroups are defined as 0.. 7. Based on LS [1] from RAN2, the minimum number of subgroup IDs is 0. Therefore, CN subgroup IDs in NG/Xn/F1 stage3 should be defined as 0.. 30.

Proposal 1:	LP-WUS CN Subgroup ID within LP-WUSPS Assistance Information should be defined as INTEGER (0..30, …) in NGAP/XnAP/F1AP.

UE Identity Index

According to the LS [1] from RAN2, the Subgroup ID for LP-WUS specified in 38.304 is calculated using the 5G-S-TMSI mod X as well as PEI, and this X is up to 1048576 (1024*4*8*32), so it is necessary to introduce new IE carrying UE Identity Index of at least 20bit length into NGAP/XnAP/F1AP. The following 2 methods were discussed in the offline email discussion.

Opt1: Introduce new UE ID related IE with 20 bit

Opt2: Introduce new UE ID related IE with 5bits and receiving node concatenate it with legacy 16 bits Extended UE Identity Index.

While both seem to work, we do not see the benefit of forcing the receiving node to concatenate two IEs, which is not normal way in RAN3 specs, to avoid an overhead due to 16bit overlap when both legacy and new IEs are sent. Therefore, we propose just introducing a new 20bit IE, i.e. Opt1.

Proposal 2:	Introduce new IE for UE Identity Index with 20bits, which is used to calculate LP-WUS subgroup ID.

The behavior of nodes that send/receive this new IE was also discussed in the offline email discussion. Here, whether or not new IE is necessary or understandable for receiving node depends on 1) whether LP-WUS is used with PEI/eDRX and 2) whether the target RAN node sends paging to the UE using LP-WUS. On the other hand, basically, if a receiving node receives an IE that is not necessary or understandable, it can be ignored in the 3GPP specification, so even if new IE is sent unnecessarily, there is no fail case. Therefore, we do not think that it is necessary to force the sending node to decide whether or not new IE is necessary in the spec.

Proposal 3:	The behavior of nodes receiving/sending the new IE for UE Identity Index should be clarified. FFS on specification impact.

Proposal 4:	The AMF includes the new IE for UE Identity Index in the NGAP Paging message whenever the UE supports LP-WUS.

Proposal 5:	The gNB-CU includes the new IE for UE Identity Index in the XnAP RAN paging message or F1AP paging message whenever the paging message received from the AMF includes it.

Legacy gNB mixed environment issue

In the LS [2] we received from SA2 at the RAN3#127 meeting, it was pointed out that in an environment where legacy gNBs and LP-WUS capable gNBs are mixed, if a UE registers with 5GC under legacy gNB and then moves to LP-WUS cell, the UE is in low-power state, but the 5GC and the RAN node which receives the paging message does not know that the UE is in low-power state, so WUS transmission does not occur and paging may not reach the UE. This problem is because legacy gNB cannot report a newer release of UE RadioCapability for Paging to 5GC via INM.

This issue was discussed at the previous RAN2 meeting and we received LS [3]. The following part is related to this issue.

- Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced.   This is applicable only to new Rel-19 and future capabilities added to paging capabilities.  This will only work for gNB that support this Rel-19 “container”

In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

Based on the above, RAN2 acknowledges that when using LP-WUS within pre-release 19 gNBs and release 19 gNBs mixed environment, paging may not be reached. At the same time, no solution is introduced for this issue. So basically, this LS indicates that RAN3 does not need to discuss a new solution.

However, we do not think it is a good way to specify that there are implicit cases in the spec where paging may not work. Therefore, we think that this fact should be captured at stage2 level. On which spec we specify it can be further discussed.

Proposal 6:	The fact that when LP-WUS is used in legacy gNB mixed environment, paging to a UE in a low-power state in LP-WUS cells may not reach because UE RadioCapability for Paging including LP-WUS capability is not properly reported to 5GC should be noted in stage2 specification. FFS on which spec to specify it.

Emergency PDU session

In the previous meeting, regarding the use of LP-WUS when an emergency PDU session is established, reply LS [4] from SA2 was discussed. SA2 also agreed on CR [5], which removes the constraints on the operation of PEI and LP-WUS when establishing an emergency PDU session and RAN3 remove the note from stage2 as well. Based on these, our understanding of the operation when establishing an emergency PDU session is summarized as follows.

- gNB can recognize the presence of an emergency PDU session based on the appropriate ARP assignment by the operator.

- It is up to implementation whether gNB keeps the RRC_CONNECTED state when the emergency PDU session is established or sends the UE to the RRC_IDLE/INACTIVE state same as normal operation.

- If gNB sends UE to RRC_INACTIVE state when emergency PDU session is established, eDRX should not be configured. (captured as a note)

This is because different from eDRX, there is no way for NW to control UE behavior for PEI/LP-WUS in a UE-dedicated manner. Also in this case, if necessary, gNB can make UE maintain RRC_CONNECTED state to realize quick response for emergency call back and emergency re-call. Therefore, we think that the discussion on emergency PDU session fro LP-WUS can be closed.

Observation:	There is no issue on handling of LP-WUS during emergency PDU session.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	LP-WUS CN Subgroup ID within LP-WUSPS Assistance Information should be defined as INTEGER (0..30, …) in NGAP/XnAP/F1AP.

Proposal 2:	Introduce new IE for UE Identity Index with 20bits, which is used to calculate LP-WUS subgroup ID.

Proposal 3:	The behavior of nodes receiving/sending the new IE for UE Identity Index should be clarified. FFS on specification impact.

Proposal 4:	The AMF includes the new IE for UE Identity Index in the NGAP Paging message whenever the UE supports LP-WUS.

Proposal 5:	The gNB-CU includes the new IE for UE Identity Index in the XnAP RAN paging message or F1AP paging message whenever the paging message received from the AMF includes it.

Proposal 6:	The fact that when LP-WUS is used in legacy gNB mixed environment, paging to a UE in a low-power state in LP-WUS cells may not reach because UE RadioCapability for Paging including LP-WUS capability is not properly reported to 5GC should be noted in stage2 specification. FFS on which spec to specify it.

Observation:	There is no issue on handling of LP-WUS during emergency PDU session.

References

- R3-253026 “LS on LP-WUS subgrouping progress”, RAN2

- R3-250021 “LS Reply on LP-WUS subgrouping”, SA2

- R3-253012 “Reply LS on paging capability loss issue”, RAN2

- R3-251521 “Reply LS on emergency call back and paging”, SA2

- S2-2502428 “Removing unnecessary restrictions for emergency PDU sessions”, Qualcomm