3GPP TSG-RAN WG3 #128	R3-253678

St. Julian - Malta, 19 – 23 May. 2025

Agenda Item:	31.2. Enhancements

Source:	NTT DOCOMO INC., Nokia, Huawei, Vodafone, CATT, LG Electronics, NEC, Fujitsu, Qualcomm, SK Telecom, Deutsche Telekom, Rakuten

Title:	Enhancement of NR to LTE mobility restriction

Document for:	Discussion, approval

Introduction

In TS 38.413, Mobility Restriction List IE is specified, and this IE can be included in some messages (e.g. HANDOVER REQUEST) to indicate restriction information for the UE from AMF to gNB. This IE includes Forbidden TACs within Forbidden Area Information to which the UE is prohibited to access. However, this IE refers TAC for NR defined as 3 octets, hence current NGAP specification does not support to configure 2 octets TAC for E-UTRAN as a forbidden TAC. Also, TS 23.501 specifies the high level principle of Mobility Restriction.

In the last meeting, we submitted discussion paper [1][2] for this enhancement and following was captured:

E///: This is the discussion should be start in SA2, inter-RAT mobility can be satisfied without this enhancement. Same issue for Idle mode case to be checked by CT4.

ZTE: There is no such requirement in SA2 spec, similar case we had before, if this is the new requirement in SA2 then it should be triggered by SA2. No further enhancement is needed.

NTT Docomo: There is also no restriction in SA2 spec, ask SA2 first

HW: Checked internally, no restriction in SA2 spec

Nok: Support to send LS to SA2

CATT: Per TAC RAT mobility restriction

noted

This contribution discusses the enhancement to NGAP specification and LS to SA2 to realize above enhancement.

Discussion

A high level principle for Mobility Restriction function is specified in TS23.501. In RRC_IDLE/INACTIVE state, this function is executed by UE. In RRC_CONNECTED, this function is executed by RAN and CN. From operator perspective, this function is useful in some cases. The figure shows the example scenario. In some deployments, there is an area in which limited users can access the RAN and it is managed based on subscription information. This area is configured as forbidden area to which UE is restricted to access, and this area would be, for example, MPN (Mobile Private Network), military zones, or enterprise specific area. In case A, the forbidden area is neighboring to the non-forbidden area, and both are covered by LTE cells. This situation may occur in early phase of migration. In case B, the layout of the forbidden area and the non-forbidden area is the same as in case A, but the NR SA area has been deployed little bit, and there is a place where NR SA area and LTE area are adjacent. In case C, the forbidden area situation is the same, and NR SA has been fully deployed, and all areas are covered by NR SA cells. The handover from the non-forbidden area to the forbidden area is avoided by mobility restriction function provided by EPC in case A and 5GC in case C, respectively. On the other hand, in case B, the handover to the forbidden area cannot be avoided because 5GC cannot configure the mobility restriction to LTE cells based on existing specification. Proposed enhancement in this paper realizes mobility restriction even in such situation.

Fig. example scenario for forbidden area.

In RRC_CONNECTED, CR [3] proposes an enhancement to include 2 octet EPS TACs as forbidden areas in the Mobility Restriction List IE in NGAP to restrict access to specific EPS TACs. From the operator's point of view, there is a strong demand for the function which allows to restrict mobility from NR to specific EPS TACs in RRC_CONNECTED state based on user subscription information, so RAN3 should discuss this enhancement.

Proposal 1:	RAN3 agree to enhance NGAP mobility restriction to include EPS TAC as forbidden area.

The UE behavior in 5GS regarding mobility restriction is specified in TS24.501, where the forbidden area can include only 3 octet NR TAC. On the other hand, since the UE can simultaneously register with the EPS and the information related to mobility restriction to EPS TAC can be received directly from EPC, mobility toward specific EPS TAC can be restricted even if 5GC cannot configure 2 octet EPS TAC as a forbidden area.

The NW behavior related to the mobility restriction is basically based on the information exchanged in the Mobility Restriction List IE in the NGAP, but gNB cannot receive mobility restriction for EPS TAC from EPS. Also, in the TS23.501 referenced by this IE, there is no restriction on the octet length of the TAC that can be configured as a forbidden area.

Therefore, the mobility restriction function in the UE and the mobility restriction function in the NW are specified in different stage3 specifications, and the stage2 specification they refer to do not specify any restrictions on whether the forbidden area can be 2octet EPS TAC. Therefore, basically, RAN3 can support this enhancement without affecting the CN specifications.

Observation 1:	Mobility Restriction via NAS message is different functionality from that via NGAP because UE which receives NAS message can register both 5GC and EPC and receive mobility restriction from both, but gNB cannot receive it from EPC.

Observation 2:	There is no restriction for configuring EPS TAC as a forbidden area in TS 23.501 which NGAP specification refers to.

On the other hand, there is ambiguity as to whether AMF can obtain subscription information for a particular UE from UDM and obtain mobility restrictions with EPS TACs. In TS 29.503 sec. 6.1.6.2.4, which specifies the forbiddenAreas exchanged via SBI in 5GC, describes this IE as "List of forbidden areas in 5GS," and the array(Area) referred to by this IE is defined as 2 or 3 octets in TS 29.571 sec. 5.4.2. Our understanding is that NR TAC is defined as 3 octets in TS 23.003, so it can be implied as EPS TAC when forbiddenAreas is 2 octets and NR TAC when it is 3 octets. On the other hand, these specifications do not explicitly specify that EPS TAC can be configured for forbiddenAreas. If RAN3 companies have concerns about this ambiguity, we can send LS to SA2 and request clarification on the definition of forbiddenAreas.

Proposal 2:	Send LS to SA2 to check the understanding on mobility restriction functionality.

After receiving feedback from SA2, RAN3 can specify this enhancement in NGAP specification.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	RAN3 agree to enhance NGAP mobility restriction to include EPS TAC as forbidden area.

Observation 1:	Mobility Restriction via NAS message is different functionality from that via NGAP because UE which receives NAS message can register both 5GC and EPC and receive mobility restriction from both, but gNB cannot receive it from EPC.

Observation 2:	There is no restriction for configuring EPS TAC as a forbidden area in TS 23.501 which NGAP specification refers to.

Proposal 2:	Send LS to SA2 to check the understanding on mobility restriction functionality.

Reference

[1]	R3-252144, “NR to LTE mobility restriction”, NTT Docomo

[2]	R3-252145, “Enhancement of NR to LTE mobility restriction [Enhanced Mobility]” (CR to TS 38.413), NTT Docomo

[3]	R3-253679, “Enhancement of NR to LTE mobility restriction [Enhanced Mobility]” (CR to TS 38.413), NTT Docomo

Annex: LS to SA2, CC to CT4

3GPP TSG-RAN WG3 #128	R3-25xxxx

St. Julian - Malta, 19 – 23 May 2025

Title:	[Draft] LS on NR to LTE mobility restriction

Response to:	-

Release:	Rel-19

Work Item:	NR_newRAT-Core

Source:	NTT DOCOMO (to be RAN3)

To:	SA2

Cc:	CT4

Contact Person: 	Mio Nakamura

mio.nakamura.ue@nttdoomo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

In the current NGAP specification, only NR TAC can be configured as a forbidden area for mobility restriction. RAN3 discussed enhancement on the mobility restriction function to enable configuring EPS TAC as a forbidden area. On the other hand, we could not conclude whether this enhancement affects the core network related specifications because there is an ambiguity as to whether forbiddenAreas specified in 29.503 support specifying EPS TAC.

- Actions:

RAN3 kindly asks SA2 to provide feedback on whether the Mobility Restriction function specified in TS 23.501 and TS 29.503 allows to configure EPS TAC defined by 2 octet as forbidden area defined in TS 29.571. Also, if necessary, please provide references to other appropriate specifications for this function or introduce necessary clarification on those specifications.

- Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #129	25th – 29th August 2025		India

TSG-RAN3 Meeting #129bis	13th – 17th October 2025		Prague