3GPP TSG-RAN WG3 Meeting #128	R3-253687

Malta, MT, May 19th – 23th, 2025

Title: 	(TP to TS38.425) F1-U enhancement for UE delay

Source: 	CMCC

Agenda item: 	11.4

Document Type:	Other

- Introduction

In RAN3#127bis meeting, it has been agreed that:

Packet delay measured for UE performance feedback sent from CU-UP to CU-CP is per DRB level.

For Packet delay, whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, UL, DL, and periodicity?

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

Agree the TP to 37.480 and TP to 37.483 in this meeting for the new procedure over E1 interface.

- Discussion

Based on the WA, the current resource status initiation procedure and resource status report procedure will be reused to get the measured EC over F1 interface.

And based on the WA1, in case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19. So Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UL/DL UE throughput and UL/DL packet delay from CU-UP to CU-CP upon request.

However, For Packet delay, whether/how to enhance UE Context Setup/Modification procedure to introduce the configuration of data collection, e.g., start, stop, UL, DL, and periodicity?

In 127bis meeting, BL CR to 38.473 has been agreed to introduce the DATA Collection Request message over E1. And in the message IE, Reporting Periodicity for Data Collection is used to define the periodicity for reporting of requested objects. Also used as the averaging window length for all objects if supported.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average UE Throughput UL, Second bit = Average UE Throughput DL, Third Bit = Average Packet Delay, Other bits are ignored by the gNB-CU-UP. FFS on the coding | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

Now the reporting periodicity parameter is provisioned on CU-UP. But CU-UP waits the report from gNB-DU to get this measurement. And in current F1-U, gNB-DU report the “The ASSISTANCE INFORMATION DATA” frame with DL/UL delay. However how often and how to average the delay in gNB-DU is never defined, from our understanding, it is up to implementation.

From the above, we can see the periodicity doesn’t really work for reporting and average the delay. Without the unified periodicity, the UL/DL delay may differ a lot, and it is not reasonable be compare it as a UE performance feedback. So this periodicity shall be provisioned to gNB-DU and aligned up with CU-UP.

Currently in TS38.425:

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

Since it is a per UE parameter, the most straightforward way it to provision the periodicity parameter to gNB-DU as part of UE Context when UE CONTEXT SETUP REQUEST or UE CONTEXT MODIFICATION REQUEST message are sent from gNB-CU to gNB-DU via F1AP.

And when gNB-DU receive the periodicity for data collection, it should restore it and use it to report the UE’s UL/DL delay through The Assistance Information.

Proposal 1: It is proposed that gNB-DU shall be provisioned and aligned with Reporting Periodicity for Data Collection provisioned via E1 in DATA COLLECTION REQUEST message for each UE.

Proposal 2: It is proposed to use UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST via F1AP to provision the same Reporting Periodicity for Data Collection from gNB-CU to gNB-DU as via E1AP.

And to capture the previous agreements on UL/DL delay for split architecture, the following TP to TS38.425 is proposed as below.

Proposal 3: Capture the conclusion on UL/DL delay for split architecture in TP to TS 38.425.

- Summary

Proposal 1: It is proposed that gNB-DU shall be provisioned and aligned with Reporting Periodicity for Data Collection provisioned via E1 in DATA COLLECTION REQUEST message for each UE.

Proposal 2: It is proposed to use UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST via F1AP to provision the same Reporting Periodicity for Data Collection from gNB-CU to gNB-DU as via E1AP.

Proposal 3: Capture the conclusion on UL/DL delay for split architecture in TP to TS 38.425.

4. TP to TS 38.425

<< Start of the change>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[3]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[4]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[5]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[6]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 23.501: "System Architecture for the 5G System".

[9]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture description".

[10]                      3GPP TS 38.401: "NR; Radio Access Network (NG-RAN); Architecture description".

<< next change>>

5.4.3	Transfer of Assistance Information

5.4.3.1	Successful operation

NOTE 1:	In this section, PDCP duplication and delay measurement related information are not applicable to E-UTRA PDCP.

The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures.

An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only.

The Transfer of Assistance Information procedure may be invoked if

-	the corresponding node decides to send the Radio Quality Assistance Information and/or the PDCP duplication activation suggestion to the node hosting the NR PDCP entity for the concerned data radio bearer or,

-	the corresponding node decides to send the Radio Quality Assistance Information to the node hosting the NR PDCP entity for the concerned RLC entity.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to measure the UL/DL Delay for specific DRBs, and to send the UL/DL Delay results to the node hosting the NR PDP entity for the concerned data radio bearer.

The ASSISTANCE INFORMATION DATA frame may include one or more Radio Quality Assistance Information. The information shall consist of the information indicated in the Assistance Information Type.

The ASSISTANCE INFORMATION DATA shall be sent, if supported, when the corresponding node receives a DL USER DATA PDU including the Assistance Information Report Polling Flag set to 1.

The ASSISTANCE INFORMATION DATA frame may include the PDCP Duplication Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or not activate DL PDCP duplication. The node hosting the NR PDCP entity may take this information into account to take a decision on whether to activate or not activate PDCP duplication.

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay for the slice, or for the UE of RAN.

<< next change>>

5.5.3.48	UL Delay DU Result

Description: This field indicates UL delay measured at the corresponding node in milliseconds for the concerned DRB over Uu interface. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the total UL delay for the concerned DRB and report to the UPF for the purpose of QoS monitoring as specified in [8],or for the purpose of packet delay feedback as specified in [10]. This information element is not applicable to E-UTRA PDCP.

Value range: {0..232-1}.

Field length: 4 octets.

5.5.3.49	DL Delay DU Result

Description: This field indicates DL delay measured at the corresponding node in milliseconds for the concerned DRB over Uu interface. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the total DL delay for the concerned DRB and report to the UPF for the purpose of QoS monitoring as specified in [8],or for the purpose of packet delay feedback as specified in [10]. This information element is not applicable to E-UTRA PDCP.

Value range: {0..232-1}.

Field length: 4 octets.

<<end of change>>

- Reference

- R3-253073 (BL CR to 37.483) Support of enhancements on AI/ML for NG-RAN，Samsung

- TS 38.425, NR user plane protocol

- TS 38.473, F1AP

- RAN3_127bis_agenda_20250411_1400_EOM, chairnote