3GPP TSG RAN WG3 Meeting #128	R3-253702

St Julian's, Malta, May 19th - 23rd, 2025

Agenda Item:	21.3

Source: 	Meta

Title:  	Discussion on XR RAN Awareness and UL Rate Control

Document for:	Discussion and Decision

# Introduction

In RAN#107, the XR RAN Awareness and UL rate control objective is as follows [1]:

Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

In this paper, we discuss XR RAN Awareness and UL rate control.

# Discussion

Based on the RAN2 agreement, the rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

In the RAN3 #127-bis meeting, RAN3 discussed details on supporting per-QoS flow indication. The key observations from the RAN3 perspective are:

Observation 1: Per QoS flow rate indication can be supported when there is a one-to-one mapping between QoS flows and DRBs.

Observation 2: Per QoS flow rate indication can also be supported in scenarios where the CU and DU are not separated, as the QoS flow to DRB mapping information is available at the gNB to derive the recommended bit rate of the UL.

Observation 3: When the QoS flow to DRB mapping is not one-to-one and under the CU-DU split architecture, the F1 interface has an impact in supporting per QoS flow rate indication.

In the RAN2#129-bis meeting, RAN2 also continued discussing the XR rate control and made the following agreements:

Agreement: Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

Because the UL recommended bit rate is a PHY bit rate, it is natural that DU is the entity to monitor the network congestion conditions and estimate the recommended bitrate.

Proposal 1: The recommended bit rate is estimated at the DU.

With Option 1, the QoS to DRB mapping information needs to be sent from CU to DU. DU then derives the recommended bit rate on a per-QoS flow level based on the network congestion conditions. This option achieves simplicity and full flexibility of QoS flow level indication.

Observation 4: Option 1 achieves simplicity and full flexibility of QoS flow level indication.

With Option 2, some companies proposed signaling the QoS flow to DRB mapping and QoS flows rate ratios to the UE via RRC. However, this approach has several limitations and design flaws: The CU may not be aware of the rate ratios as the DU is responsible for estimating the PHY bit rate. The ratio may not be semi-static and can vary depending on channel and network conditions. This approach also incurs RRC signaling overhead and delays.

Observation 5: Option 2 has several limitations and design flaws.

Based on the above discussions, it is proposed that RAN3 work on the F1AP interface enhancement to support Option 1.

Proposal 2: RAN3 enhances the F1AP interface to support Option 1.

# Conclusion

In this paper, we discuss our views on XR RAN awareness and UL rate control.

Observation 1: Per QoS flow rate indication can be supported when there is a one-to-one mapping between QoS flows and DRBs.

Observation 2: Per QoS flow rate indication can also be supported in scenarios where the CU and DU are not separated, as the QoS flow to DRB mapping information is available at the gNB to derive the recommended bit rate of the UL.

Observation 3: When the QoS flow to DRB mapping is not one-to-one and under the CU-DU split architecture, the F1 interface has an impact in supporting per QoS flow rate indication.

Proposal 1: The recommended bit rate is estimated at the DU.

Observation 4: Option 1 achieves simplicity and full flexibility of QoS flow level indication.

Observation 5: Option 2 has several limitations and design flaws.

Proposal 2: RAN3 enhances the F1AP interface to support Option 1.

# References

[1] RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3