3GPP TSG-RAN WG3 Meeting #128	R3-253705

Malta, ML, 19-23 May, 2025

Agenda Item:	19.2

Source:		Huawei, China Telecom, China Unicom

Title:	(TP to BL CR for 38.401) introduction of Evolution of NR duplex operation Sub-band full duplex (SBFD)

Document for:	Discussions & Approval

- Introduction

This TP tries to capture the proposals in the discussion paper [1], listed as follows:

Proposal 4: To remove the precondition of CLI (inference) being detected before exchange the CLI related information.

- References

[1] R3-253703	Further discussion on remaining open issues, Huawei, China Telecom, China Unicom

3	Annex-1: TP to 38.401

=============================Start of change==============================

7.3	Cross-Link Interference Management

The Cross-Link Interference Management function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, the gNB-CU forwards the TDD DL/UL patterns received from neighboring nodes to each concerned gNB-DU. The gNB-DU reports the TDD DL/UL patterns of its serving cells to the gNB-CU if Cross-Link Interference is detected.

In Sub-band full duplex (SBFD) operation, Cross Link Interference (CLI) may also be present. In case of split gNB architecture, the gNB-DU reports CLI related information of its serving cells to the gNB-CU. The gNB-CU forwards CLI related information received from served gNB-DUs and from neighboring gNBs to each concerned gNB-DU.

=============================End of change==============================