3GPP TSG-RAN WG3 Meeting #128															R3-253713

St. Julian - Malta, 19th – 23rd May 2025

Agenda Item:	13.2

Source:	Ericsson, NTT DoCoMo, Lenovo, Jio Platforms, Verizon Wireless, Charter Communications

Title:	PRACH resources for RACH-less LTM

Document for:	Discussion, Agreement

# Introduction

In this contribution, we provide further discussion on the how to handle preamble index allocation for RACH-less LTM.

# Discussion

In inter-CU LTM, during the preparation phase, the source gNB requests PRACH resources from each candidate gNB, to be used later for Early UL sync. The candidate gNB allocate a pool of PRACH resources, and as long as they are kept available for the source gNB, the source gNB can send PDCCH orders for TA value acquisition. When a certain PRACH resource is used for one UE, the source gNB cannot use the same PRACH resource for another UE, i.e., once the source gNB1 has sent a PDCCH order targeting a certain PRACH resource #1 allocated by a candidate gNB2 to UE1, it cannot send another PDCCH order targeting the same PRACH resource #1 allocated by the same gNB2 to a UE2 until the TA value for UE1 is acquired. If that happens, the source gNB will not be able to discriminate whether the TA value it receives is for UE1 or UE2.

- Once the source gNB1 has sent a PDCCH order targeting a PRACH resource #1 allocated by a candidate gNB2 to UE1, it cannot send another PDCCH order targeting the same PRACH resource #1 allocated by the same gNB2 to a UE2 until the TA value for UE1 is acquired.

When an LTM execution is triggered, the source DU will send an LTM Cell Switch Command MAC CE to the UE, and RACH-less LTM will be attempted if the MAC CE will refer to PRACH resources previously allocated by the candidate cell, e.g., via the Random Access Preamble index. If, for any reason, the candidate gNB2 has released the PRACH resources previously allocated for LTM for the source gNB1, the LTM execution will fail.

- At LTM execution, if RACH-less LTM is attempted but the candidate gNB has released the PRACH resources, the LTM execution fails.

- If a candidate gNB releases PRACH resources initially provided to the source gNB as PRACH resources for LTM, and the source gNB is not informed, this will lead to failures in TA acquisition or LTM execution.

It is clear that RACH-less LTM is only possible if PRACH resources are available, and also that the amount of such resources is quite limited (even if the same Preamble Index is reused with multiple PRACH occasions). It is therefore  important to ensure that the amount of PRACH resources for LTM is kept under control. The availability of PRACH resources for LTM can be limited due to reasons not related to LTM. For example, a candidate gNB may need to use PRACH resources for non-LTM purposes and decide to deallocate some PRACH resources previously allocated for LTM.

In general, we think that, given the scarcity of PRACH resources, and to ensure that RACH-less LTM can be used effectively, a robust solution needs to consider both the source gNB and the candidate gNB “interests”. The source gNB “interest” is to use PRACH resources for RACH-less LTM. Ideally, requested PRACH resources from a candidate gNB should always be available, or otherwise Early UL sync or LTM execution will fail. The candidate gNB “interest” is instead, to reduce the amount of time during which PRACH resources allocated for LTM are not used. Obviously, the standard cannot restrict the gNB behaviour, so the candidate gNB, if needed, can simply release PRACH resources previously reserved for LTM, and use them for other purposes.

In our current signaling baseline, the two “interests” just mentioned are only partially supported. The source gNB can request PRACH resources to a candidate gNB, and a candidate gNB can still release them if needed, but in this case the source gNB will not know about it. We still believe that allocating PRACH resources, such as preamble index, on a per-UE basis would be the most effective way to support dynamic resource management and address scalability concerns. However, since this approach was not pursued, we are considering introducing a deallocation mechanism to help mitigate the issue. Therefore, to make the LTM solution more robust (and avoid unnecessary failures), we propose that the candidate gNB informs the source gNB when it deallocates Random Access Preamble index that were previously allocated for LTM.

- For inter-CU LTM, a candidate gNB informs the source gNB when Preamble Indexes assigned LTM are deallocated.

The same mechanism is also good to have for intra-CU LTM, the related proposal is presented below.

- For intra-CU LTM, a candidate gNB-DU informs the source gNB-DU when Preamble Indexes assigned for LTM are deallocated.

- Agree on the TPs for 38.423 and 38.473 in R3-253714 and R3-253715 that capture the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Observation 1	Once the source gNB1 has sent a PDCCH order targeting a PRACH resource #1 allocated by a candidate gNB2 to UE1, it cannot send another PDCCH order targeting the same PRACH resource #1 allocated by the same gNB2 to a UE2 until the TA value for UE1 is acquired.

Observation 2	At LTM execution, if RACH-less LTM is attempted but the candidate gNB has released the PRACH resources, the LTM execution fails.

Observation 3	If a candidate gNB releases PRACH resources initially provided to the source gNB as PRACH resources for LTM, and the source gNB is not informed, this will lead to failures in TA acquisition or LTM execution.

Proposal 1	For inter-CU LTM, a candidate gNB informs the source gNB when Preamble Indexes assigned LTM are deallocated.

Proposal 2	For intra-CU LTM, a candidate gNB-DU informs the source gNB-DU when Preamble Indexes assigned for LTM are deallocated.

Proposal 3	Agree on the TPs for 38.423 and 38.473 in R3-253714 and R3-253715 that capture the above proposals.