3GPP TSG-RAN WG3 Meeting #R3-128       	                                                         R3-253721

St Julian’s, Malta, 19-23 May, 2025

Agenda Item:	17.3

Source: 	Rakuten

Title:  	End-to-End Xn and F1 Signalling to support OD-SIB1

Document for:	Discussion and decision

# Introduction

We have the following agreements from the previous RAN3 meetings:

RAN3#125:

UL WUS configuration is decided by NES Cell DU.

UL WUS configuration transmission from NES Cell DU to NES Cell CU.

NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

RAN3#125-bis:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

RAN3#126:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

The procedure uses non UE-associated signaling.

Cell A stores the UL WUS configuration information after it has received it.

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

RAN3#127:

It is agreed that after Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

It is agreed from the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

RAN3#127-bis:

In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that:

- One Choice “Start with UL WUS Condiguration”

One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

# End-to-End Signalling at NES Cell and Cell A

Based on the above agreements, we will propose end-to-end signalling at the NES Cell and at Cell A to support the basic functionalities of OD-SIB1. These include:

- NES Activation: NES Cell transitioning from default periodic SIB1 transmission to OD-SIB1 mode

- NES Deactivation: NES Cell transitioning from OD-SIB1 mode to default periodic SIB1 transmission

- NES Update: NES Cell’s changed UL WUS Configuration updated by NES Cell to Cell A

- NES Support Withdrawn: Cell A stopping the transmission of NES Cell’s UL WUS Configuration in SIBx

There are other use cases which can be discussed once the signaling to support these have been agreed.

## NES Activation

Step 1: NES Cell gNB-DU decides to activate the NES mode

All the previous agreements on this topic have not discussed which entity of the NES Cell makes the decision of the SIB1 transmission mode to operate in. This could be either the periodic SIB1 transmission (default mode), or the OD-SIB1 transmission (NES mode).

There are several reasons which make the NES cell gNB-DU the preferred entity to decide whether the NES cell should transition to an energy saving mode. Firstly, for any cell, the gNB-DU is aware of the current traffic, including the RRM resources in use, number of UEs connected or camped to the cell and the type of bearers established for the UEs.  Secondly, the gNB-DU receives the UL WUS configuration from the UEs when the UE intends to trigger OD-SIB1 from the NES cell.  And lastly, as per TS38.473, gNB-DU is the network entity responsible for encoding and broadcasting SIB1 for a given cell.

Hence, we propose that the decision of whether the NES Cell should transition from default periodic SIB1 transmission to OD-SIB1 transmission and other way round should lie with the NES cell gNB-DU. This may be done in agreement with NES Cell gNB-CU.

We propose that the decision of whether the NES Cell should transition from default periodic SIB1 transmission to OD-SIB1 transmission and other way round should lie with the NES cell gNB-DU.

Step 2: NES Cell gNB-DU sends the necessary information to NES Cell gNB-CU over F1

The following has been agreed:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU.

And

- Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

From TS38.473, the Served Cell Information identifies the PCI of the NES Cell gNB for which the UL WUS Configuration might be shared.

And from R3-252386 as agreed in the previous meeting, the following has been added to the Served Cell Information IE.

The above agreements and references make it clear how the information to activate NES mode at the NES Cell is transmitted from the NES Cell gNB-DU to NES Cell gNB-CU.

Step 3: NES Cell gNB-CU sends the necessary information to Cell A gNB-CU over Xn

The following has been agreed in the last RAN3 meeting:

- In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that:

- One Choice “Start with UL WUS Condiguration”

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node1 (NES Cell gNB-CU) to provide UL WUS configuration information to NG-RAN node2 (Cell A gNB-CU) and request NG-RAN node2 to transmit UL WUS configuration information. If the “start with UL WUS Configuration” is included in the UL WUS CONFIGURATION PROVISION REQUEST message, the Cell A gNB shall reply with the UL WUS CONFIGURATION PROVISION RESPONSE message. The Cell A gNB stores the UL WUS configuration information.

One open point above is that this request for UL WUS Configuration Provision from NES Cell gNB to Cell A gNB does not include the PCI of the NES Cell gNB-DU which is captured in the Served Cell Information IE sent from the NES Cell gNB-DU to NES Cell gNB-CU. Therefore, we propose that the agreed procedure UL WUS CONFIGURATION PROVISION REQUEST captures the PCI of the cell for which this coordination is being done.

We propose that UL WUS CONFIGURATION PROVISION REQUEST message sent from NES Cell gNB-CU to the Cell A gNB-DU contains the PCI of the NES Cell gNB-DU which is proposing to transition from the default periodic SIB1 transmission to the OD-SIB1 transmission.

In the above message, it has been ambiguously mentioned that Cell A gNB shall reply with the UL WUS CONFIGURATION PROVISION RESPONSE message with success or failure. However, it has not yet been agreed in RAN3 regarding which entity in Cell A gNB makes this decision. We believe to keep the design simple and with minimal signaling, Cell A gNB-CU is best positioned to decide whether Cell A gNB may lend support to the NES Cell gNB by broadcasting its UL WUS Configuration in its SIBx. Cell A gNB-CU is also aware of the availability of the cells under it. For example, whether they are in energy saving mode or not.

We propose that the decision of whether the Cell A gNB should support the NES Cell gNB’s request of transmitting the NES Cell gNB’s UL WUS Configuration in its SIBx lies with the Cell A gNB-CU.

Step 4: Cell A gNB-CU sends the necessary information to Cell A gNB-DU over F1

The first open point in this step is which Cell A gNB-DU is the request for UL WUS Configuration provision support being sent to. We believe that this can be left to implementation which can derive its own logic or use OAM configuration for the same. The NES Cell gNB might not be aware of the availability of a particular Cell A at the time of request, hence we are not in agreement with some of the contributions from the earlier meetings which propose that the NES Cell gNB decides which Cell A gNB-DU should broadcast the NES Cell gNB’s UL WUS Configuration. Neither are we in agreement that all the DUs (cells) under Cell A gNB-CU should be asked to broadcast the NES Cell’s UL WUS Configuration. This can lead to unnecessary transmission of the UL WUS Configuration by cells which might not have significant overlapping coverage with the NES cell. And in a worst-case scenario, the NES Cell might end up having to transmit OD-SIB1 to UEs which have poor signal reception from the NES Cell, thereby minimizing the energy saving benefits of the NES Cell. Therefore, we propose that the choice of Cell A gNB-DU that gets the request to support the transmission of UL WUS Configuration of a NES Cell gNB lies with the Cell A gNB-CU.

RAN3 to agree that the choice of Cell A gNB-DU that gets the request to support the transmission of UL WUS Configuration of a NES Cell gNB lies with the Cell A gNB-CU. This can be left to the implementation which can derive its own logic or use OAM configuration for the same.

It has been agreed in RAN2#127bis that the UL WUS configuration of a NES Cell is provided to UEs via a new SIB in Cell A. This new SIB is herein referred to as “SIBx”. We propose that SIBx be encoded by the Cell A gNB-CU, just like the other System Information Blocks containing neighbour information. And this SIBx can then be included in the legacy transmission of gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU.

We propose that SIBx be encoded with the NES Cell gNB’s UL WUS Configuration by the Cell A gNB-CU and transmitted to Cell A gNB-DU by using the GNB-CU CONFIGURATION UPDATE message and gNB-CU System Information IE.

Once the Cell A gNB-DU starts broadcasting SIBx, the UEs in RRC IDLE/INACTIVE mode may acquire the SIB and get the UL WUS Configuration of the NES Cell gNB. This enables the UEs to trigger OD-SIB1 transmission on the NES Cell gNB which has stopped the default periodic transmission of SIB1.

## NES Deactivation

For deactivating the NES mode on the NES Cell gNB, the same steps as above are involved to execute the option to stop the transmission of UL WUS Configuration in Cell A gNB SIBx.

Step 1: NES Cell gNB-DU decides to deactivate the NES mode

Based on our Proposal 1, the deactivation of the NES mode (i.e. transitioning from OD-SIB1 to default periodic SIB1) mode lies with the NES Cell gNB-DU.

Step 2: NES Cell gNB-DU sends the necessary information to NES Cell gNB-CU over F1

From the previous two meetings, we have the following agreements on this point:

- It is agreed that NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

- One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”.

And from R3-252386 as agreed in the previous meeting, the following has been added to the Served Cell Information IE.

If the on-demand SIB1 IE is included and set to “Stop provision” in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, stop the coordination of on-demand SIB1 transmission for network energy saving.

Once the NES Cell gNB-DU transmits this IE in the GNB-DU CONFIGURATION UPDATE message to the NES Cell gNB-CU, it may immediately begin the default periodic SIB1 transmission. It need not wait for any acknowledgement from the NES Cell gNB-CU, in spite of using the Class 1 procedure.

We propose that once the NES Cell gNB-DU transmits the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message to the NES Cell gNB-CU, it may immediately begin the default periodic SIB1 transmission. It need not wait for any acknowledgement from the NES Cell gNB-CU, in spite of using the Class 1 procedure.

Step 3: NES Cell gNB-CU sends the necessary information to Cell A gNB-CU over Xn

The purpose of the UL WUS CONFIGURATION PROVISION procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit UL WUS configuration information. The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to stop the transmission of UL WUS configuration. Cell A gNB removes the previously received UL WUS Configuration of the NES Cell gNB.

Step 4: Cell A gNB-CU sends the necessary information to Cell A gNB-DU over F1

To stop the broadcasting of UL WUS Configuration of the NES Cell gNB in SIBx of Cell A gNB-DU, the Cell A gNB-CU must inform the Cell A gNB-DU of the same. In keeping in line with the design principles followed so far in the agreements in RAN3, we propose the use of gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU. The content of SIBx encoded by Cell A gNB-CU reflects the absence of NES Cell gNB’s UL WUS Configuration. If Cell A gNB is supporting other NES Cell gNB’ss NES mode, then Cell A gNB-DU may continue to broadcast SIBx.

We propose that to stop the broadcasting of UL WUS Configuration of the NES Cell gNB in SIBx of Cell A gNB-DU, the Cell A gNB-CU uses gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU. The content of SIBx encoded by Cell A gNB-CU reflects the absence of NES Cell gNB’s UL WUS Configuration.

- Observation 1 	If Cell A gNB is supporting other NES Cell gNB’s NES mode, then Cell A gNB-DU may continue to broadcast SIBx.

## NES Update

It is possible that the UL WUS Configuration of the NES Cell gNB changes. And if this happens while the NES Cell gNB-DU is in the NES mode, then the NES Cell gNB-CU and all the Cell A gNBs supporting the transmission of UL WUS Configuration need to be informed. Towards this we propose extending the earlier agreed messages with an “Update Provision” option.

The first IE to be updated is the Served Cell Information IE in the gNB-DU Configuration Update procedures sent from the NES Cell gNB-DU to the NES Cell gNB-CU over the F1 interface.

The second method to be updated is the UL WUS CONFIGURATION PROVISION REQUEST sent from the NES Cell gNB-CU to the Cell A gNB-CU over the Xn interface.

We propose that when the UL WUS Configuration changes in the NES Cell gNB, the NES Cell gNB-DU informs the NES Cell gNB-CU using an “Update Provision” in the Served Cell Information IE over the F1 interface.

We propose that when the UL WUS Configuration changes in the NES Cell gNB, the NES Cell gNB-CU informs the Cell A gNB-CU using an “Update” in the UL WUS CONFIGURATION PROVISION REQUEST over the Xn Interface.

Once the updated UL WUS Configuration has been accepted at the Cell A gNB-CU, it needs to inform the Cell A gNB-DU. We propose that to update the UL WUS Configuration of the NES Cell gNB, the Cell A gNB-CU uses gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU over the F1 interface. The content of SIBx encoded by Cell A gNB-CU reflects the update of NES Cell gNB’s UL WUS Configuration.

We propose that to update the UL WUS Configuration of the NES Cell gNB, the Cell A gNB-CU uses gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU over the F1 interface. The content of SIBx encoded by Cell A gNB-CU reflects the update of NES Cell gNB’s UL WUS Configuration.

It can be left to the internal implementation at the NES Cell gNB-DU to transition to the default periodic SIB1 transmission while the updated UL WUS Configuration change has been reflected in the SIBx at the Cell A gNB.

## NES Support Withdrawn

When Cell A wants to stop transmitting the UL WUS Configuration of the NES Cell, it may use the agreed Class 2 procedure to inform the NES Cell of the same. We have the following agreement from a previous meeting:

A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

The procedure uses non UE-associated signalling.

As per our Proposal 3, where the decision of whether the Cell A gNB should support the NES Cell gNB’s request of transmitting the NES Cell gNB’s UL WUS Configuration in its SIBx lies with the Cell A gNB-CU, we propose that the decision to stop the support of NES Cell gNB’s UL WUS Configuration transmission also lies with Cell A gNB-CU.

We propose that the decision to stop the support of NES Cell gNB’s UL WUS Configuration transmission also lies with Cell A gNB-CU.

To inform the Cell A gNB-DU of the same, the Cell A gNB-CU may follow the same process as in NES Deactivation process, where the encoded SIBx transmitted as part of gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU over the F1 interface will have null content for NES Cell gNB’s UL WUS Configuration, or absence of NES Cell gNB’s UL WUS Configuration. From this, the Cell A gNB-DU may infer that it need not broadcast SIBx for the NES Cell gNB. However, If Cell A gNB is supporting other NES Cell gNBs’ NES mode, then Cell A gNB-DU may continue to broadcast SIBx.

The Cell A gNB-CU may then use the new Class 2 procedure to inform the NES Cell gNB-CU of its decision to stop the support of UL WUS Configuration transmission in its SIBx. Once the NES Cell gNB-CU gets this information, it may decide to do nothing if there are other Cell As which continue to transmit the UL WUS Configuration of the NES Cell gNB-DU. However, if there are no other Cell As which support the NES mode, then the NES Cell gNB-CU uses another Class 2 procedure to inform the NES Cell gNB-DU of the Cell A’s decision to stop the transmission of UL WUS Configuration. On reception of this message, the NES Cell gNB-DU must stop the NES mode, and transition from OD-SIB1 transmission to default periodic SIB1 transmission.

We propose the use of a new class 2 procedure (yet to be defined) for NES Cell gNB-CU to inform NES Cell gNB-DU to transition from OD-SIB1 mode to the default periodic SIB1 transmission.

# Conclusion

We have the following observations and proposals:

- Proposal 1	We propose that the decision of whether the NES Cell should transition from default periodic SIB1 transmission to OD-SIB1 transmission and other way round should lie with the NES cell gNB-DU.

- Proposal 2	We propose that UL WUS CONFIGURATION PROVISION REQUEST message sent from NES Cell gNB-CU to the Cell A gNB-DU contains the PCI of the NES Cell gNB-DU which is proposing to transition from the default periodic SIB1 transmission to the OD-SIB1 transmission.

- Proposal 3	We propose that the decision of whether the Cell A gNB should support the NES Cell gNB’s request of transmitting the NES Cell gNB’s UL WUS Configuration in its SIBx lies with the Cell A gNB-CU.

- Proposal 4	RAN3 to agree that the choice of Cell A gNB-DU that gets the request to support the transmission of UL WUS Configuration of a NES Cell gNB lies with the Cell A gNB-CU. This can be left to the implementation which can derive its own logic or use OAM configuration for the same.

- Proposal 5	We propose that SIBx be encoded with the NES Cell gNB’s UL WUS Configuration by the Cell A gNB-CU and transmitted to Cell A gNB-DU by using the GNB-CU CONFIGURATION UPDATE message and gNB-CU System Information IE.

- Proposal 6	We propose that once the NES Cell gNB-DU transmits the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message to the NES Cell gNB-CU, it may immediately begin the default periodic SIB1 transmission. It need not wait for any acknowledgement from the NES Cell gNB-CU, in spite of using the Class 1 procedure.

- Proposal 7	We propose that to stop the broadcasting of UL WUS Configuration of the NES Cell gNB in SIBx of Cell A gNB-DU, the Cell A gNB-CU uses gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU. The content of SIBx encoded by Cell A gNB-CU reflects the absence of NES Cell gNB’s UL WUS Configuration.

- Observation 1 	If Cell A gNB is supporting other NES Cell gNB’s NES mode, then Cell A gNB-DU may continue to broadcast SIBx.

- Proposal 8	We propose that when the UL WUS Configuration changes in the NES Cell gNB, the NES Cell gNB-DU informs the NES Cell gNB-CU using an “Update Provision” in the Served Cell Information IE over the F1 interface.

- Proposal 9	We propose that when the UL WUS Configuration changes in the NES Cell gNB, the NES Cell gNB-CU informs the Cell A gNB-CU using an “Update” in the UL WUS CONFIGURATION PROVISION REQUEST over the Xn Interface.

- Proposal 10	We propose that to update the UL WUS Configuration of the NES Cell gNB, the Cell A gNB-CU uses gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message sent from Cell A gNB-CU to Cell A gNB-DU over the F1 interface. The content of SIBx encoded by Cell A gNB-CU reflects the update of NES Cell gNB’s UL WUS Configuration.

- Proposal 11	We propose that the decision to stop the support of NES Cell gNB’s UL WUS Configuration transmission also lies with Cell A gNB-CU.

- Proposal 12	We propose the use of a new class 2 procedure (yet to be defined) for NES Cell gNB-CU to inform NES Cell gNB-DU to transition from OD-SIB1 mode to the default periodic SIB1 transmission.

# Reference

Chairman’s Notes RAN3#125

Chairman’s Notes RAN3#125-bis

Chairman’s Notes RAN3#126

Chairman’s Notes RAN#127

Chairman’s Notes RAN#127-bis

R3-252386