3GPP TSG-RAN WG3 Meeting #128	R3-253734

Malta, MT, 19th May– 23rd  May 2025

Agenda Item:       10.2

Source:	ZTE Corporation

Title: 		(TP for SON BLCR for 38.300, 38.401, and 38.473) LTM failure and near failure caused by wrong beam

Document for:	Discussion and Approval

1	Introduction

RAN3 received an LS (R3-251514) on SON for LTM from RAN2 at the last meeting. In this LS, RAN2 requested RAN3 to define a network based solution to address near failure caused by BFR shortly after a successful LTM (Case 1), and asked whether this solution could also apply to LTM failure due to wrong beam (Case 2).

In this contribution, we discuss both cases, and provide the corresponding TPs for network solution in the Annex.

2	Discussion

2.1 Case1 (Near failure):

The BFR shortly after successful LTM cell switch to the wrong beam (“near failure”) is described in the following Figure 1.

Figure 1. A near-failure due to LTM to Right Cell and Wrong (or Sub-optimal) Beam.

What distinguishes LTM from legacy HO is that the network must determine not only the most suitable cell, but also the most appropriate beam for the UE to attach to. Let’s dive into the scenario in above figure.

-Step 1: Using RACH-less LTM, the UE is switched from Cell 1/Beam 1-1 to Cell 2/Beam 2-1.

-Step 2: The UE successfully accesses Cell 2/Beam 2-1 but experiences a beam failure shortly thereafter.

-Step 3: The UE initiates a BFR and successfully recovers the connection via Cell 2/Beam 2-2.

In this example, Cell 2/Beam 2-2 proves to be a better target than Cell 2/Beam 2-1. Consequently, although the source node chose the correct cell (i.e., Cell 2), it selected an unsuitable beam (Beam 2-1).

RAN3's progress on BFR shortly after successful LTM cell switch at last meeting:

RAN3 supports network-based solution for the case of BFR shortly after a successful LTM cell switch.

The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

Based on the above agreements, we suggest to capture following Stage 2 TP in 38.401

The target gNB-DU detects that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch. The target gNB-DU performs initial analysis and in case of the BFR caused by wrong beam selection at the source gNB-DU, it may send the recovery beam information to the source gNB-DU via the gNB-CU

Proposal 1: To capture RAN3's agreement on the reporting of recovery beam information to the source gNB-DU as Stage 2 specification in TS 38.401.

The recovery beam information can be represented by either a TCI state ID or a beam ID, e.g.,SSB index. Since candidate beams are provided to source gNB-DU by the target gNB-DU as TCI state IDs during the LTM preparation phase, it is proposed that the recovery beam information be conveyed using the TCI state ID.

Proposal 2: The reported recovery beam information is conveyed using the TCI state ID.

Based on different implementation considerations, after a successful LTM cell switch, both immediate and delayed reporting are considered feasible for MRO optimization. In the case of immediate reporting, the gNB-DU UE F1AP ID may be included along with the recovery beam information to enable the CU or source gNB-DU to identify the UE context. Or, in the case of delayed reporting, the C-RNTI may be included instead.

Proposal 3:  If target DU detects the BFR after the successful LTM cell switch, based on different implementation, both immediate and delayed reporting are considered feasible for MRO optimization.

Proposal 4:   For immediate reporting, the gNB-DU UE F1AP ID may be included to  identify the UE context; For delayed reporting, the C-RNTI may be included instead.

To facilitate the above  recovery beam information reporting, the target DU may send a message, such as a DU-CU ACCESS AND MOBILITY INDICATION message, to the CU to report relevant information of the LTM near failure event. The CU can then forward this information to the source DU via an ACCESS AND MOBILITY INDICATION message.

Proposal 5 :  The target gNB-DU sends a message, such as a DU-CU ACCESS AND MOBILITY INDICATION message, to the gNB-CU to report recovery beam information. The gNB-CU can then forward this information to the source gNB-DU via an ACCESS AND MOBILITY INDICATION message.

2.2 Case2 (failure):

If the beam info is wrongly configured or indicated,  there is one failure case about LTM to Right Cell, but Wrong Beam as shown in the following Figure 2.

Figure 2 A failure due to LTM to Right Cell and Wrong Beam.

Step 1. An LTM cell switch decision is triggered by network, and the UE is switched to the right cell but wrong beam). E.g., a UE was in cell1/beam1-1 is commanded to LTM cell switch to cell2/beam 2-1.

Step 2. The LTM cell switch failed during the LTM cell switch procedure, e.g., UE was not able to detect any scheduling on Beam 2-1.

Step 3. UE executes the cell selection procedure, still selecting to cell 2, and afterwards re-establishing the connection at the cell2/beam2-2.

RAN3's progress on LTM cell switch failure due to wrong beam at last meeting:

FFS if/how the solution for case 1 could also be used for the case of LTM cell switch failure due to wrong beam (case 2)?

From our perspective, both UE based and network based solutions are feasible for MRO optimization in the case of LTM cell switch failure due to wrong beam. However, since RAN2 has not discussed the UE based solution and considering the Release 19 timeline, RAN3 agrees to proceed with the network based solution.

Proposal 6 :  Considering the Release 19 timeline, RAN3 agrees to proceed with the network based solution for Case 2, as the UE based solution has not been discussed in RAN2.

From the target cell's perspective, upon receiving a CELL SWITCH NOTIFICATION from the source DU via the source CU, it should anticipate the UE's imminent arrival and expect a RACH-less access to the target cell . If the target cell subsequently observes the UE successfully performing a RACH-based access using a beam different from the target beam,  it may consider a RACH-less LTM failure caused by wrong beam.

Proposal 7 :   The target DU can identify an LTM cell switch failure caused by right cell but wrong beam, i.e., when it observes the UE successfully performing a RACH-based access at same target cell using a beam different from the indicated target beam.

Similarly to Case 1 of BFR, for Case 2 of beam failure, In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization. In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization.

Proposal 8 :   For Case 2, In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization. In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization.

In Case 1 of Beam Failure Recovery (BFR), the BFR is detected after a successful LTM cell switch. Therefore, it is proposed that the recovery beam information be reported to the CU via the DU–CU Access and Mobility Indication message. In contrast, in Case 2, the target cell observes the UE performing a RACH-based access on a beam different from the target beam during the LTM switch procedure. After the UE successfully reconnects the target cell via this different beam, the target DU shall send the ACCESS SUCCESS message to the CU. In case the source DU is responsible for MRO optimization, it is proposed that the target DU include the reconnect beam information in the ACCESS SUCCESS message. The CU can then forward the reconnect beam information to the source DU either immediately or with a delay via the ACCESS AND MOBILITY INDICATION message.

Proposal 9 :  In case the source DU is responsible for wrong beam selection, it is proposed that the target DU include the reconnect beam information in the ACCESS SUCCESS message to CU. The CU can then forward the reconnect beam information to the source DU either immediately or with a delay via the ACCESS AND MOBILITY INDICATION message.

Based on the above discussion, we suggest to capture following Stage 2 TP in 38.401

The target gNB-DU detects that the UE successfully performs a RACH-based access to the target cell during the LTM cell switch procedure, reconnecting to the same cell with a beam different from the target beam. The target gNB-DU performs initial analysis and in case of the RACH-less LTM failure caused by wrong beam selection at the source gNB-DU, it may send the reconnect beam information to the source gNB-DU via the gNB-CU.

Proposal 10: To capture the reporting of reconnect beam information to the source gNB-DU as Stage 2 specification in TS 38.401.

Based on the above discussion, we suggest to capture the LTM failure of  right cell but wrong beam as a failure case  in 38.300.

Intra-system Wrong Beam for RACH-less LTM: a RACH-less LTM cell switch failure occurs during the LTM cell switch procedure; The UE successful re-establishes the radio link connection in the same target cell but using a different beam.

Proposal 11: To capture LTM failure case caused by right cell but wrong beam as Stage 2 specification in TS 38.300.

Based on the above proposals for both cases, in the annex, we provide the corresponding stage 2 TPs for 38.401, 38.300, and stage 3 TP for TS 38.473 for the network based solution for both cases. Furthermore, it is proposed to reply to RAN2 that RAN3 has defined solutions for both cases. A draft reply LS is provided in the annex.

Proposal 12 :Reply to RAN2 that RAN3 has defined solutions for both cases. A draft reply LS is provided in the annex.

Proposal 13 :RAN3 to agree on the corresponding stage 2 TPs for TS 38.401 and TS 38.300, and stage 3 TP for TS 38.473, as provided in the annex.

3	Conclusion

For case 1 (The BFR case):

Proposal 1: To capture RAN3's agreement on the reporting of recovery beam information to the source gNB-DU as Stage 2 specification in TS 38.401.

Proposal 2: The reported recovery beam information is conveyed using the TCI state ID.

Proposal 3:  If target DU detects the BFR after the successful LTM cell switch, based on different implementation, both immediate and delayed reporting are considered feasible for MRO optimization.

Proposal 4:   For immediate reporting, the gNB-DU UE F1AP ID may be included to  identify the UE context; For delayed reporting, the C-RNTI may be included instead.

Proposal 5 :  The target gNB-DU sends a message, such as a DU-CU ACCESS AND MOBILITY INDICATION message, to the gNB-CU to report recovery beam information. The gNB-CU can then forward this information to the source gNB-DU via an ACCESS AND MOBILITY INDICATION message.

For case 2:

Proposal 6 :  Considering the Release 19 timeline, RAN3 agrees to proceed with the network based solution for Case 2, as the UE based solution has not been discussed in RAN2.

Proposal 7 :   The target DU can identify an LTM cell switch failure caused by right cell but wrong beam, i.e., when it observes the UE successfully performing a RACH-based access at same target cell using a beam different from the indicated target beam.

Proposal 8 :   For Case 2, In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization. In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization.

Proposal 9 :  In case the source DU is responsible for wrong beam selection, it is proposed that the target DU include the reconnect beam information in the ACCESS SUCCESS message to CU. The CU can then forward the reconnect beam information to the source DU either immediately or with a delay via the ACCESS AND MOBILITY INDICATION message.

Proposal 10: To capture the reporting of reconnect beam information to the source gNB-DU as Stage 2 specification in TS 38.401.

Proposal 11: To capture LTM failure case caused by right cell but wrong beam as Stage 2 specification in TS 38.300.

For both cases:

Proposal 12 :Reply to RAN2 that RAN3 has defined solutions for both cases. A draft reply LS is provided in the annex.

Proposal 13 :RAN3 to agree on the corresponding stage 2 TPs for TS 38.401 and TS 38.300, and stage 3 TP for TS 38.473, as provided in the annex.

Annex a: TP for BL CR for TS38.401

<< Start of Change >>

7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

The target gNB-DU detects that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch. The target gNB-DU performs initial analysis and in case of the BFR caused by wrong beam selection at the source gNB-DU, it may send the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU detects that the UE successfully performs a RACH-based access to the target cell during the LTM cell switch procedure, reconnecting to the same cell with a beam different from the target beam. The target gNB-DU performs initial analysis and in case of the RACH-less LTM failure caused by wrong beam selection at the source gNB-DU, it may send the reconnect beam information to the source gNB-DU via the gNB-CU.

<< End of Change >>

Annex b: TP for BL CR for TS38.300

<< First Change >>

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

-	Intra-system Wrong Beam for RACH-less LTM: a RACH-less LTM cell switch failure occurs during the LTM cell switch procedure; The UE successful re-establishes the radio link connection in the same target cell but using a different beam.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover, handover to wrong cell and wrong beam for RACH-less LTM are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO/LTM or CHO with candidate SCG(s) is configured but the CHO/LTM execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell/the cell UE attempts LTM recovery is the cell that served the UE at the last handover/LTM initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover/LTM cell switch for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery/the cell UE attempts LTM recovery is neither the cell that served the UE at the last handover/LTM initialisation nor the cell that served the UE where the RLF happened or the cell that the handover/LTM was initialized toward.

-	Intra-system Wrong Beam for RACH-less LTM:  there is a recent RACH-less LTM cell switch for the UE prior to the connection failure, and the successful re-connect cell is the same target cell of the last LTM cell switch but UE using a different beam from the one indicated in the LTM cell switch command by the last serving cell.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO/LTM execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of Too Late Handover when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may send a Handover Report over Xn to the candidate NG-RAN node(s) controlling the candidate PSCell(s) whose mobility configuration caused the failure.

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

<< End of Change >>

Annex c: TP for BLCR for TS38.473

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

8.3.8	Access Success

8.3.8.1	General

The purpose of the Access Success procedure is to enable the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover, conditional PSCell addition,conditional PSCell change,LTM, or subsequent CPAC. The procedure uses UE-associated signalling.

8.3.8.2	Successful Operation

Figure 8.3.8.2-1: Access Success procedure. Successful operation.

The gNB-DU initiates the procedure by sending a ACCESS SUCCESS message.

Upon reception of the ACCESS SUCCESS message, the gNB-CU shall consider that the UE successfully accessed the cell indicated by the included NR CGI IE in this gNB-DU and consider all the other CHO or conditional PSCell addition or conditional PSCell change preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU as cancelled. In case of subsequent mobility, the other preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU are kept.

If the ACCESS SUCCESS message contains the LTM Wrong Beam Information IE, the gNB-CU shall notify this information to the source gNB-DU of LTM cell switch.

Interaction with other procedure:

The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate gNB-DUs for this UE, if any.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGE >>>>>>>>>>>>>>>>>>>>

8.11.1	Access and Mobility Indication

8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Wrong Beam Information List IE the gNB-DU shall take it into account for optimisation of LTM cell switch related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

8.11.1.3	Abnormal Conditions

Not applicable.

8.11.2	DU-CU Access and Mobility Indication

8.11.2.1	General

This procedure is initiated by the gNB-DU to send the Access and Mobility related Information to the gNB-CU.

The procedure uses non-UE-associated signalling.

8.11.2.2	Successful Operation

Figure 8.11.2.2-1: DU-CU Access and Mobility Indication procedure. Successful operation

The DU-CU Access and Mobility Indication procedure is initiated by DU-CU ACCESS AND MOBILITY INDICATION message sent from the gNB-DU to the gNB-CU.

If the DU-CU ACCESS AND MOBILITY INDICATION message contains the DL LBT Failure Information List IE, the gNB-CU shall take it into account for optimisation of mobility parameters.

If the DU-CU ACCESS AND MOBILITY INDICATION message contains the LTM Wrong Beam Information List IE, the gNB-CU shall notify the relevant source gNB-DU(s) of LTM cell switch, the corresponding beam information.

8.11.2.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGE >>>>>>>>>>>>>>>>>>>>

9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover, conditional PSCell addition, conditional PSCell change, LTM, or subsequent CPAC.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |
| LTM Wrong Beam Information | O |  | 9.3.1.xxx | This IE includes only the reconnect beam information related to an LTM failure caused by right cell with wrong beam. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| LTM Wrong Beam Information List |  | 0..<maxnoofLTMWrongBeamInformation> |  |  | YES | ignore |
| > LTM Wrong Beam Information |  |  | 9.3.1.xxx |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |
| maxnoofLTMWrongBeamInformation | Maximum no. of LTM wrong beam information. Value is 64. |

9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| > DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| LTM Wrong Beam Information List |  | 0..<maxnoofLTMWrongBeamInformation> |  |  | YES | ignore |
| > LTM Wrong Beam Information |  |  | 9.3.1.xxx | This IE includes only the recovery beam information related to an LTM near failure caused by right cell with wrong beam. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |
| maxnoofLTMWrongBeamInformation | Maximum no. of LTM wrong beam information. Value is 64. |

9.3.1.xxx	LTM Wrong Beam Information

This information element is used to indicate the recovery beam information related to an LTM near failure, or the reconnect beam information related to an LTM failure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU. |
| Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. |
| UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. |
| Wrong Beam Type | M |  | ENUMERATED (near-failure-LTM-to-Right-Cell-with-Wrong-Beam, failure-LTM-to-Right-Cell-with-Wrong-Beam, ...) |  |

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>

Annex d: reply LS

3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

Malta, MT, 19th May– 23th  May 2025

Title:	[Draft] Reply LS on SON for LTM

Response to:	R2-2501378/R3-251514

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	ZTE Corporation (will be RAN3)

To:	RAN2

Cc:

Contact person:	dapeng Li

li.dapeng@zte.com.cn

Send any reply LS to:  3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: A

- Overall description

RAN3 thanks RAN2 for the LS on SON for LTM.

RAN3 has discussed both cases and agrees to proceed with network based solutions for both cases. RAN3 defined solutions are provided in the attached approved TPs.

- Actions

To RAN2:

ACTION: 	RAN3 kindly asks RAN2 to take the above information into consideration.

3	Dates of next RAN3 meetings

RAN3#129			25th - 29th Aug 2025 		India (TBC), IN