3GPP TSG-RAN WG3 Meeting #128	R3-253746

Malta, MT, 19-23, May, 2025

Agenda Item:	18.2

Source:	Ericsson

Title:	Response to R3-253255

Document for:	Response paper

# 1	Introduction

This paper is a response to the discussion paper in [1], where the proponents propose to capture a change to TS 38.300 that eDRX and LP-WUS may be used together.

# 2	Discussion

As indicated in [1], it is proposed to have a stage 2 TP to capture that the eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.

# 6. TP to BLCR for TS 38.300 <<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>> […] - When the RRC_IDLE eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before resuming/establishing an RRC connection; - The eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging. <<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

First, there is a problem with the wording: “may” only applies to the UE, i.e. from gNB’s perspective, LP-WUS shall be used when “configured and supported by the UE”. The gNB does not “may use” but configures.

Also, we remark there is no such clarification in Rel-17 for PEI and eDRX case. Therefore there is no need to capture such clarification for LPWUS. Finally, one can deduce that LPWUS and eDRX are supported from the UE_ID when the 5G-S-TMSI is mod X, where X is 131072, if eDRX is applied; otherwise, X is 32768.

Therefore, it is proposed that RAN3 does not capture the stage 2 clarification on LP-WUS co-existence with eDRX.

Proposal 1: RAN3 does not capture any stage 2 clarification on LP-WUS co-existence with eDRX.

Regarding the impacts of co-existence between LP-WUS and eDRX, when the UE requests to use LP-WUS with a long eDRX cycle, then LP-WUS should not be allowed with such long eDRX to avoid unnecessary LP-WUS transmissions and monitoring or entry/exit conditions during the UE long eDRX cycles. This will cause a lot of UE battery loss. To disable this, during the NAS negotiation signaling, the core network can omit the CN-assigned subgroup ID in the REGISTRATION ACCEPT message towards the UE when it includes the negotiated long eDRX cycle. However when UE is using UE-ID based subgrouping this option is missing.

Observation 1: Co-existence of LP-WUS with long eDRX may impact UE power saving gains due to frequent UE wake-ups during the long eDRX cycle monitoring the LP-WUS. The CN can prevent the UE from using LP-WUS by not including the CN-assigned LP-WUS subgroups in the NAS REGISTRATION ACCEPT message. However for UE-ID based LP-WUS, this option is missing.

A solution to be discussed is that the UE indicates to CN during the NAS negotiation phase that it expects UE-ID LP-WUS. Afterwards, depending on the value of the negotiated eDRX, if e.g. longer than 20 seconds, the CN indicates to RAN during the INITIAL CONTEXT SETUP REQUEST message (for RRC_INACTIVE RAN Paging) and during the PAGING message (for RRC_IDLE paging) that UE-ID based subgrouping is not allowed. An illustration of signalling is presented below:

Example of signalling UE-ID allowed status to NG-RAN during long eDRX configuration for RRC-IDLE and RRC_INACTIVE

Some impacts pertain to CT1 and RAN2 feedback is needed. It proposed to discuss this scenario in RAN3.

Proposal 2: RAN3 to discuss the scenario of CN indicating to NG-RAN whether UE-ID LPWUS should be allowed or not to prevent UE battery drain during LP-WUS monitoring and very long eDRX cycles.

Finally, it is also pertinent for RAN3 discussion to capture in TS 38.300 a NOTE on the non-homogenous deployment, e.g., that “it is up to the network implementation to avoid the paging loss issue for pre- and Rel-19 features” and send the LS to SA2 and RAN2 as proposed in [2].

Proposal 3: Capture in TS 38.300 a NOTE on the non-homogenous deployment, e.g., that “it is up to the network implementation to avoid the paging loss issue for pre- and Rel-19 features”.

# 3	Conclusion

Proposal 1: RAN3 does not capture any stage 2 clarification on LP-WUS co-existence with eDRX.

Observation 1: Co-existence of LP-WUS with long eDRX may impact UE power saving gains due to frequent UE wake-ups during the long eDRX cycle monitoring the LP-WUS. The CN can prevent the UE from using LP-WUS by not including the CN-assigned LP-WUS subgroups in the NAS REGISTRATION ACCEPT message. However for UE-ID based LP-WUS, this option is missing.

Proposal 2: RAN3 to discuss the scenario of CN indicating to NG-RAN whether UE-ID LPWUS should be allowed or not to prevent UE battery drain during LP-WUS monitoring and very long eDRX cycles.

Proposal 3: Capture in TS 38.300 a NOTE on the non-homogenous deployment, e.g., that “it is up to the network implementation to avoid the paging loss issue for pre- and Rel-19 features”.

# 4	References

- R3-253255 (TP to BLCR for TS 38.413, TS 38.423 and TS 38.300) Discussion on Low-power wake-up signal and receiver for NR (Huawei)

- R3-253643 [draft] Reply LS on LP-WUS subgrouping (Huawei)