3GPP TSG-RAN WG3 Meeting #128	R3-253747

St Julian’s, Malta, 19 – 23 May, 2025

Agenda item:	8.1

Source:	Nokia

Title:	Response to R3-253736, R3-253737

Document for:	Response

# 1	Introduction

Related to incoming LS from RAN2 in [1], which includes CR to TS 38.314 [2] technically endorsed by RAN2, we provide in this paper response to [3] and [4]:

- R3-253736 [3], which is a CR to TS 38.300 to reflect RAN2’s technically endorsed updates [2] of measurement of number of active UEs

- R3-253737 [4], which is a draft reply to the incoming LS from RAN2 [1]

RAN2 indicates in their LS [1] in action to SA5: “RAN2 would like to receive SA5 reply urgently (within the meeting week in May if possible) as RAN2 need it to confirm or revise the IPA CRs accordingly.” Even though RAN3 is only in cc of the LS, we believe RAN2 could also need feedback from RAN3 within the week of the present meeting.

# 2	Discussion

The LS [1] is the so far last LS of an LS exchange that has taken place over a few meeting cycles, and where RAN3 has been included in cc. In LS [5] received from SA5 at previous meeting, SA5 provides the following request to RAN2:

- SA5 would like RAN2 to specify metrics that can measure the number of RRC_INACTIVE UEs in a cell with ongoing SDT

SA5 motivates their request by indicating: “In the current 5G network, the number of UEs using SDT can be huge, especially in IoT scenarios, where the total amount of SDT data transmission can be very high, which may lead to potential network congestion or contribute to network congestion to some extent.” From the LS it can further be seen that the request from SA5 is linked to monitoring of e.g. the percentage of the network workload contributed by SDT transmission, and the SDT traffic volume of UEs in a cell.

Following SA5’s request, RAN2 has technically endorsed the following changes in TS 38.314:

As indicated in [4], RAN2’s changes may have RAN3 impact on the Mobility Load Balancing (MLB) feature:

“RAN3 would clarify that since Release 16, the number of active UEs in RRC_CONNECTED has been used as an evaluation metric for SON Load Balance features.This extension of measurements may impact load balance functionality, which was originally designed based solely on RRC_CONNECTED UEs.”

And a change related to TS 38.300 description of ‘Number of active UEs” is proposed in [3].

However, we would like to point out that currently XnAP refers to TS 38.314, and not to TS 38.300 for definition of Number of active UEs:

#### 9.2.2.62	Number of Active UEs

The Number of Active UEs IE indicates the mean number of active UEs as defined in TS 38.314 [42].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Mean number of Active UEs | M |  | INTEGER (0..16777215, ...) | As defined in TS 38.314 [42] and where value "1" is equivalent to 0.1 Active UEs, value "2" is equivalent to 0.2 Active UEs, value n is equivalent to n/10 Active UEs. |

In split architecture the Number of Active UEs is measured by the gNB-DU and reported in the F1AP Number of Active UEs IE (TS 38.473 clause 9.3.1.135, same definition as above).

The NG-RAN node may balance load between cells typically by handing over UEs in RRC_CONNECTED mode from a source cell with high load to a target cell with lower load. And the Number of Active UEs is one out of several metrics that enable the NG-RAN node to evaluate the load of neighbour nodes. As per current specification, only RRC_CONNECTED UEs are counted as Active UEs in current XnAP signalling. When it comes to load induced by UEs in RRC_IDLE (due to RRC connection) and RRC_INACTIVE (due to SDT), the main mechanism relies on centralized (OAM) adaptation of cell reselection parameters, e.g. frequency priorities. (TS 38.300 also describes adjustment of cell reselection parameters but only to reflect changes in handover settings: “Cell reselection configuration may be amended to reflect changes in the HO setting.”).

With the change in TS 38.314 it becomes less clear whether the XnAP Number of Active UEs IE will from Rel-19 continue to count only UEs in RRC_CONNECTED mode, or whether also UEs in RRC_INACTIVE are counted. We don’t exclude that MLB could be enhanced to also take into account load related to SDT, but we believe there are also arguments going in the direction that SDT-induced load should mainly be balanced by OAM using e.g. cell reselection parameters. At current stage, we therefore think that the XnAP Number of Active UEs IE, and hence MLB as specified by RAN3, should remain unaffected of upcoming change of TS 38.314.

Proposal 1: Mobility Load Balancing should at this stage remain unaffected of upcoming change of TS 38.314.

We would also like to point out that the Predicted Number of Active UEs is also part of predicted measurements included in the XnAP DATA COLLECTION UPDATE message, and relies on the Number of Active UEs IE. This was introduced as part of the AI/ML Load Balancing use case studied in Rel-17 (TR 37.817) and standardized in Rel-18. So additionally, AI/ML models in support of this use case would be impacted by any change of definition of this metric because there would be a need to predict the number of active UEs in RRC_INACTIVE in addition to the current prediction of active UEs in RRC_CONNECTED.

| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 2: NG-RAN support for AI/ML Load Balancing use case should at this stage remain unaffected of upcoming change of TS 38.314.

A possible way forward to achieve this could be, instead of the change proposed in [3], to clarify in XnAP that the currently exchanged Number of Active UEs metric is limited to number of UEs in RRC_CONNECTED mode.

#### 9.2.2.62	Number of Active UEs

The Number of Active UEs IE indicates the mean number of active UEs as defined in TS 38.314 [42].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Mean number of Active UEs | M |  | INTEGER (0..16777215, ...) | As defined in TS 38.314 [42] and where value "1" is equivalent to 0.1 Active UEs, value "2" is equivalent to 0.2 Active UEs, value n is equivalent to n/10 Active UEs. This value only includes the number of UEs in RRC_CONNECTED mode. |

Proposal 3: Clarify in XnAP, from Rel-19, that the currently exchanged Number of Active UEs metric is limited to number of UEs in RRC_CONNECTED mode.

Such update in RAN3’s specification would however require that TS 38.314 continues supporting filtering of the ‘number of active UEs’ measurement per RRC state, so that UEs in RRC_CONNECTED can be counted separately from UEs in RRC_INACTIVE. We believe this is RAN2’s intention by addition of the following note in their CR [2]:

NOTE:	Each measurement specified in section 4.2.1.3.2 through 4.2.1.3.9 is performed by the gNB for active UEs in RRC_CONNECTED and the UEs in RRC_INACTIVE with ongoing SDT procedure.

However, RAN3 could still explicitly indicate to RAN2 that such filtering needs to be enabled in their specifications, with the understanding that “Each measurement specified in section 4.2.1.3.2 through 4.2.1.3.9 is performed by the gNB separately for the active UEs in RRC_CONNECTED and the UEs in RRC_INACTIVE with ongoing SDT procedure”.

We also propose to inform RAN2 about RAN3’s agreements as per the proposals above.

Proposal 4: Inform RAN2 about RAN3’s agreements as per the proposals above.

# 3	Conclusion

We have made the following proposals:

Proposal 1: Mobility Load Balancing should at this stage remain unaffected of upcoming change of TS 38.314.

Proposal 2: NG-RAN support for AI/ML Load Balancing use case should at this stage remain unaffected of upcoming change of TS 38.314.

Proposal 3: Clarify in XnAP, from Rel-19, that the currently exchanged Number of Active UEs metric is limited to number of UEs in RRC_CONNECTED mode.

Proposal 4: Inform RAN2 about RAN3’s agreements as per the proposals above.

A corresponding draft LS is included in annex of this paper.

# References

[1]		R3-253025 - R2-2503181, Reply LS on Number of UEs in RRC_INACTIVE state with data transmission, RAN2

[2]	R2-2503035, Introduction of number of UEs in RRC_INACTIVE state with data transmission, TS 38.314 CR#0034r1 (Rel-19)

[3]	R3-253736, Clarification Number of active UEs in load balance

[4]	R3-253737, [draft] Reply LS on RRC_INACTIVE with ongoing SDT procedure

[5]		R3-251527 - S5-250827, Reply to RAN2 LS on Number of UEs in RRC_INACTIVE state with data transmission, SA5

# Annex

3GPP TSG-RAN WG3 Meeting #128	     R3-25xxxx

St Julian’s, Malta, 19 – 23 May, 2025

Title:	[Draft] Reply LS on Number of UEs in RRC_INACTIVE state with data transmission

Response to:	R3-253025/R2-2503181

Release:	Release 19

Work Item:	PM_KPI_5G_Ph4

Source:	Nokia [to be RAN3]

To:	RAN2

Cc:	SA5

Contact Person:

#### Name:	Håkon Helmers

E-mail Address:	hakon.helmers@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks RAN2 for their LS relative to update of TS 38.314 definition of number of active UEs (TS 38.314 CR#0034r1 in R2-2503035).

RAN3 would like to clarify that since Rel-16, the number of active UEs in RRC_CONNECTED mode is exchanged on the Xn interface and is used by the receiving NG-RAN node, among other measurements, to determine corrective action for Mobility Load Balancing. In split architecture the Number of Active UEs is measured by the gNB-DU and reported to the gNB-CU over the F1 interface. Since Rel-18, also predicted number of active UEs is exchanged on the Xn interface and used for the AI/ML Load Balancing use case. Both exchanged metrics rely on the definition provided in TS 38.314 clause 4.2.1.3.

RAN3 has agreed that Mobility Load Balancing should at this stage remain unaffected of upcoming change of TS 38.314. RAN3 has also agreed that NG-RAN support for AI/ML Load Balancing use case should at this stage remain unaffected of upcoming change of TS 38.314.

Based on this, RAN3 would like to inform that, depending on RAN2’s final choice for update of TS 38.314, RAN3 can clarify in TS 38.423 that the XnAP Number of Active UEs IE only includes UEs in RRC_CONNECTED mode. However, such update requires that RAN2’s specification continues supporting filtering of the ‘number of active UEs’ measurements per RRC state, so that UEs in RRC_CONNECTED can be counted separately from UEs in RRC_INACTIVE. As per the note introduced by CR#0034r1, RAN3 has the understanding that “Each measurement specified in section 4.2.1.3.2 through 4.2.1.3.9 is performed by the gNB separately for the active UEs in RRC_CONNECTED and the UEs in RRC_INACTIVE with ongoing SDT procedure”.

2. Actions:

To RAN2 : 	RAN3 respectfully requests RAN2 to take the above into account and further inform RAN3 about related updates of TS 38.314.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #129				25 - 29 August 2025	     		Bengaluru, India

TSG-RAN WG3 Meeting #129bis			13 – 17 October 2025              	Prague, CZ