3GPP TSG-RAN WG3 Meeting #128	R3-253836

Malta, MT, 19 - 23 May, 2025

Agenda item:	21.3

Source: 	Huawei, China Telecom, ZTE

Title: 	(TP for XR BL CR for TS37.340) Enhancement for BSSize/TTNB

Document for:	Discussion and Decision

# 1. Introduction

This paper provides TP for TS 37.340 to capture the forwarding for BSSize and TTNB in DC case.

# Annex A - TPs for BL CR for TS 37.340

CHANGE STARTS

## 13.x	eXtended Reality Services

### 13.x.1	Overview

The eXtended Reality Services as described in TS 38.300 [3] is extended to address the NR-DC operation.

### 13.x.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling.

The UE may report uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow via UE Assistance Information to the MN or the SN as configured. If the SN receives the burst arrival time, the SN may use it by considering the SFN offset of the MN.

When the PDCP hosting gNB receives the End of Data Burst Indication from the UPF, it always provides the End of Data Burst Indication to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

When the PDCP hosting gNB receives the Burst Size and/or the Time To Next Burst from the UPF, it always provides the Burst Size and/or the Time To Next Burst to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

CHANGE ENDS