3GPP TSG RAN WG3 Meeting # 128	R3-253845

# Malta, MT, 19 - 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1790 | rev | 8 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 36.423 for SON) Addition of SON enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Samsung, Lenovo, Huawei, Ericsson, ZTE, CMCC, Nokia, Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support SON feature in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce SCG Failure Information Report and SCG Failure Transfer in X2AP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support MRO for SCG failure in EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7, 8.1, 8.7.x (new), 8.7.y (new), 9.3.3, 9.3.4, 9.3.5, 9.3.7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS36.300 CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1 Submit to RAN3#125bis Rev.2 Merge agreed R3-245822 in RAN3#125bis Rev.3 Submit to RAN3#126 Rev.4 Submit to RAN3#127 Rev.5 Merge agreed R3-250851 in RAN3#127 Rev.6 Submit to RAN3#127bis Rev.7 Submit to RAN3#128 Rev.8 Update cover page |  |  |  |  |  |  |  |  |

///////////////////////////////////////////////////////////////////////start///////////////////////////////////////////////////////////////////////

7	Functions of X2AP

The X2AP protocol provides the following functions:

-	Mobility Management. This function allows the eNB to move the responsibility of a certain UE to another eNB. Forwarding of user plane data, Status Transfer and UE Context Release function are parts of the mobility management.

-	Dual Connectivity. This function allows the eNB to request another eNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	E-UTRA-NR Dual Connectivity. This function allows the eNB to request another en-gNB to provide radio resources for a certain UE while keeping responsibility for that UE.

-	Load Management. This function is used by eNBs to indicate resource status, overload and traffic load to each other.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Resetting the X2. This function is used to reset the X2 interface.

-	Setting up the X2. This function is used to exchange necessary data for the eNB or en-gNB for setup the X2 interface and implicitly perform an X2 Reset.

-	eNB Configuration Update. This function allows updating of application level data needed for two eNBs to interoperate correctly over the X2 interface.

-	Mobility Parameters Management. This function allows the eNB to coordinate adaptation of mobility parameter settings with a peer eNB.

-	Mobility Robustness Optimisation. This function allows reporting of information related to mobility failure events.

-	Energy Saving. This function allows decreasing energy consumption by enabling indication of cell activation/deactivation over the X2 interface.

-	X2 Release. This function allows an eNB to be aware that the signalling connection to a peer eNB is unavailable.

-	Message Transfer. This function allows indirect transport of X2AP messages to a peer eNB.

-	Registration. This function allows registration of eNB in case indirect transport of X2AP messages is supported.

-	Removing the X2. This function allows removing the signalling connection between two eNBs or between eNB and en-gNB in a controlled manner.

-	Inter-eNB UE Context Retrieval. This function allows retrieval of a UE context in case of resumption or re-establishment of an RRC connection.

-	Secondary RAT Data Usage Report. This function allows eNB to get the uplink and downlink data volumes for the Secondary RAT on a per E-RAB basis.

-	E-UTRA - NR Spectrum Sharing. This function allows uplink and downlink spectrum sharing between a number of E - UTRA and a number of NR cells with overlapping coverage.

-	EN-DC Configuration Transfer. This function supports en-gNB X2 TNL address discovery.

-	EN-DC Load Management. This function is used by MeNB/en-gNB to indicate resource status, overload and traffic load to each other.

-	UE Radio Capability ID Mapping.

The mapping between the above functions and X2 EPs is shown in the table below.

Table 7-1: Mapping between X2AP functions and X2AP EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| Mobility Management | a) Handover Preparation b) SN Status Transfer c) UE Context Release d) Handover Cancel e) Handover Success f) Conditional Handover Cancel |
| Dual Connectivity | a) SeNB Addition Preparation b) SeNB Reconfiguration Completion c) MeNB initiated SeNB Modification Preparation d) SeNB initiated SeNB Modification e) MeNB initiated SeNB Release f) SeNB initiated SeNB Release g) SeNB Counter Check |
| E-UTRA-NR Dual Connectivity | a) SgNB Addition Preparation b) SgNB Reconfiguration Completion c) MeNB initiated SgNB Modification Preparation d) SgNB initiated SgNB Modification e) SgNB change f) MeNB initiated SgNB Release g) SgNB initiated SgNB Release h) SgNB Counter Check i) RRC transfer j) EN-DC X2 Setup k) EN-DC Configuration Update l) EN-DC Cell Activation m) SgNB Activity Notification n) EN-DC X2 Removal o) gNB Status Indication p) EN-DC Resource Status Reporting Initiation q) EN-DC Resource Status Reporting r) F1-C Traffic Transfer |
| Load Management | a) Load Indication b) Resource Status Reporting Initiation c) Resource Status Reporting |
| Reporting of General Error Situations | Error Indication |
| Resetting the X2 | Reset |
| Setting up the X2 | X2 Setup |
| eNB Configuration Update | a) eNB Configuration Update b) Cell Activation |
| Mobility Parameters Management | Mobility Settings Change |
| Mobility Robustness Optimisation | a) Radio Link Failure Indication b) Handover Report c) SCG Failure Information Report d) SCG Failure Transfer |
| Energy Saving | a) eNB Configuration Update b) Cell Activation |
| X2 Release | X2 Release |
| Message Transfer Registration | X2AP Message Transfer |
| Removing the X2 | X2 Removal |
| Inter-eNB UE Context Retrieval | a) Retrieve UE Context b) Data Forwarding Address Indication |
| Secondary RAT Data Usage Report | Secondary RAT Data Usage Report |
| E-UTRA - NR Spectrum Sharing | E-UTRA - NR Cell Resource Coordination |
| EN-DC Configuration Transfer | EN-DC Configuration Transfer |
| UE Radio Capability ID Mapping | UE Radio Capability ID Mapping |

8	X2AP procedures

8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Load Indication | LOAD INFORMATION |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| UE Context Release | UE CONTEXT RELEASE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Error Indication | ERROR INDICATION |
| Radio Link Failure Indication | RLF INDICATION |
| Handover Report | HANDOVER REPORT |
| X2 Release | X2 RELEASE |
| X2AP Message Transfer | X2AP MESSAGE TRANSFER |
| SeNB Reconfiguration Completion | SENB RECONFIGURATION COMPLETE |
| MeNB initiated SeNB Release | SENB RELEASE REQUEST |
| SeNB Counter Check | SENB COUNTER CHECK REQUEST |
| SgNB Reconfiguration Completion | SGNB RECONFIGURATION COMPLETE |
| SgNB Counter Check | SGNB COUNTER CHECK REQUEST |
| RRC Transfer | RRC TRANSFER |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| SgNB Activity Notification | SGNB ACTIVITY NOTIFICATION |
| Data Forwarding Address Indication | DATA FORWARDING ADDRESS INDICATION |
| gNB Status Indication | GNB STATUS INDICATION |
| EN-DC Configuration Transfer | EN-DC CONFIGURATION TRANSFER |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| EN-DC Resource Status Reporting | EN-DC RESOURCE STATUS UPDATE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Access and Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| RACH Indication | RACH INDICATION |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

8.7.x	SCG Failure Information Report

8.7.x.1	General

The purpose of the SCG Failure Information Report procedure is to provide SCG mobility related information to an en-gNB.

The procedure uses UE-associated signalling.

8.7.x.2	Successful Operation

Figure 8.7.x.2-1: SCG Failure Information Report, successful operation

The MeNB initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the en-gNB. Upon receiving the message, the en-gNB shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the en-gNB.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the en-gNB shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the en-gNB shall, if supported, store the information and act as specified in TS37.340 [32].

If the SCG FAILURE INFORMATION REPORT message includes the Time SCG Failure IE, the en-gNB shall, if supported, store the information and act as specified in TS37.340 [32].

If received, the en-gNB uses the above information for SCG failure reason detection and optimisation.

8.7.x.3	Unsuccessful Operation

Not applicable.

8.7.x.4	Abnormal Conditions

Void.

8.7.y	SCG Failure Transfer

8.7.y.1	General

The purpose of the SCG Failure Transfer procedure is to indicate to the MeNB that the root cause of the SCG failure may have occurred in the other nodes.

The procedure uses UE-associated signalling.

8.7.y.2	Successful Operation

Figure 8.7.y.2-1: SCG Failure Transfer, successful operation

en-gNB initiates the procedure by sending the SCG FAILURE TRANSFER message to MeNB.

If received, MeNB uses the information according to TS 36.300 [15].

8.7.y.3	Unsuccessful Operation

Not applicable.

8.7.y.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.4.x	SCG FAILURE INFORMATION REPORT

This message is sent by the MeNB to the en-gNB to report a PSCell change failure event.

Direction: MeNB  the en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Source PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformationNR message as defined in TS 36.331 [9] | YES | ignore |
| Time SCG Failure | O |  | INTEGER (0..1023) | timeSCGFailure included in SCGFailureInformationNR message as defined in TS36.331 [9] | YES | ignore |

9.1.4.y	SCG FAILURE TRANSFER

This message is sent by the en-gNB to the MeNB to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |

9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

X2AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM X2AP-CommonDataTypes

CellActivationRequest,

CellActivationResponse,

CellActivationFailure,

ENBConfigurationUpdate,

ENBConfigurationUpdateAcknowledge,

ENBConfigurationUpdateFailure,

ErrorIndication,

HandoverCancel,

HandoverReport,

HandoverPreparationFailure,

HandoverRequest,

HandoverRequestAcknowledge,

LoadInformation,

PrivateMessage,

ResetRequest,

ResetResponse,

ResourceStatusFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusUpdate,

RLFIndication,

SNStatusTransfer,

UEContextRelease,

X2SetupFailure,

X2SetupRequest,

X2SetupResponse,

MobilityChangeRequest,

MobilityChangeAcknowledge,

MobilityChangeFailure,

X2Release,

X2APMessageTransfer,

SeNBAdditionRequest,

SeNBAdditionRequestAcknowledge,

SeNBAdditionRequestReject,

SeNBReconfigurationComplete,

SeNBModificationRequest,

SeNBModificationRequestAcknowledge,

SeNBModificationRequestReject,

SeNBModificationRequired,

SeNBModificationConfirm,

SeNBModificationRefuse,

SeNBReleaseRequest,

SeNBReleaseRequired,

SeNBReleaseConfirm,

SeNBCounterCheckRequest,

X2RemovalFailure,

X2RemovalRequest,

X2RemovalResponse,

RetrieveUEContextRequest,

RetrieveUEContextResponse,

RetrieveUEContextFailure,

SgNBAdditionRequest,

SgNBAdditionRequestAcknowledge,

SgNBAdditionRequestReject,

SgNBReconfigurationComplete,

SgNBModificationRequest,

SgNBModificationRequestAcknowledge,

SgNBModificationRequestReject,

SgNBModificationRequired,

SgNBModificationConfirm,

SgNBModificationRefuse,

SgNBReleaseRequest,

SgNBReleaseRequestAcknowledge,

SgNBReleaseRequestReject,

SgNBReleaseRequired,

SgNBReleaseConfirm,

SgNBCounterCheckRequest,

SgNBChangeRequired,

SgNBChangeConfirm,

SgNBChangeRefuse,

RRCTransfer,

ENDCX2SetupRequest,

ENDCX2SetupResponse,

ENDCX2SetupFailure,

ENDCConfigurationUpdate,

ENDCConfigurationUpdateAcknowledge,

ENDCConfigurationUpdateFailure,

SecondaryRATDataUsageReport,

ENDCCellActivationRequest,

ENDCCellActivationResponse,

ENDCCellActivationFailure,

ENDCPartialResetRequired,

ENDCPartialResetConfirm,

EUTRANRCellResourceCoordinationRequest,

EUTRANRCellResourceCoordinationResponse,

SgNBActivityNotification,

ENDCX2RemovalRequest,

ENDCX2RemovalResponse,

ENDCX2RemovalFailure,

DataForwardingAddressIndication,

GNBStatusIndication,

ENDCConfigurationTransfer,

DeactivateTrace,

TraceStart,

HandoverSuccess,

EarlyStatusTransfer,

ConditionalHandoverCancel,

ENDCResourceStatusRequest,

ENDCResourceStatusResponse,

ENDCResourceStatusFailure,

ENDCResourceStatusUpdate,

CellTrafficTrace,

F1CTrafficTransfer,

UERadioCapabilityIDMappingRequest,

UERadioCapabilityIDMappingResponse,

AccessAndMobilityIndication,

CPC-cancel,

RachIndication,

SCGFailureInformationReport,

SCGFailureTransfer

FROM X2AP-PDU-Contents

id-cellActivation,

id-eNBConfigurationUpdate,

id-errorIndication,

id-handoverCancel,

id-handoverReport,

id-handoverPreparation,

id-loadIndication,

id-privateMessage,

id-reset,

id-resourceStatusReporting,

id-resourceStatusReportingInitiation,

id-rLFIndication,

id-snStatusTransfer,

id-uEContextRelease,

id-x2Setup,

id-mobilitySettingsChange,

id-x2Release,

id-x2APMessageTransfer,

id-seNBAdditionPreparation,

id-seNBReconfigurationCompletion,

id-meNBinitiatedSeNBModificationPreparation,

id-seNBinitiatedSeNBModification,

id-meNBinitiatedSeNBRelease,

id-seNBinitiatedSeNBRelease,

id-seNBCounterCheck,

id-x2Removal,

id-retrieveUEContext,

id-sgNBAdditionPreparation,

id-sgNBReconfigurationCompletion,

id-meNBinitiatedSgNBModificationPreparation,

id-sgNBinitiatedSgNBModification,

id-meNBinitiatedSgNBRelease,

id-sgNBinitiatedSgNBRelease,

id-sgNBChange,

id-sgNBCounterCheck,

id-rRCTransfer,

id-endcX2Setup,

id-endcConfigurationUpdate,

id-secondaryRATDataUsageReport,

id-endcCellActivation,

id-endcPartialReset,

id-eUTRANRCellResourceCoordination,

id-SgNBActivityNotification,

id-endcX2Removal,

id-dataForwardingAddressIndication,

id-gNBStatusIndication,

id-endcConfigurationTransfer,

id-deactivateTrace,

id-traceStart,

id-handoverSuccess,

id-earlyStatusTransfer,

id-conditionalHandoverCancel,

id-endcresourceStatusReporting,

id-endcresourceStatusReportingInitiation,

id-cellTrafficTrace,

id-f1CTrafficTransfer,

id-UERadioCapabilityIDMapping,

id-accessAndMobilityIndication,

id-CPC-cancel,

id-rachIndication

id-SCGFailureInformationReport,

id-SCGFailureTransfer

FROM X2AP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

X2AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode 	UNIQUE,

&criticality			Criticality 	DEFAULT ignore

}

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

f1CTrafficTransfer						|

accessAndMobilityIndication				|

cPC-cancel								|

rachIndication							|

scgFailureInformationReport				|

scgFailureTransfer						,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

rachIndication X2AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RachIndication

PROCEDURE CODE			id-rachIndication

CRITICALITY				ignore

}

scgFailureInformationReport XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE 		ScgFailureInformationReport

PROCEDURE CODE			id-scgFailureInformationReport

CRITICALITY				ignore

}

scgFailureTransfer XNAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		ScgFailureTransfer

PROCEDURE CODE			id-scgFailureTransfer

CRITICALITY				ignore

}

END

-- ASN1STOP

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ABSInformation,

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

IABAuthorized,

SCG-FailureReportContainer,

TimeSCG-Failure

FROM X2AP-IEs

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

id-IABAuthorized,

id-SourcePSCellCGI,

id-FailedPSCellCGI,

id-SCG-FailureReportContainer,

id-TimeSCG-Failure,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

maxnoofCellIDforMDT,

maxnoofTAforMDT,

maxCellinengNB,

maxnoofCellIDforQMC,

maxnoofTAforQMC,

maxnoofPLMNforQMC,

maxnoofProtectedResourcePatterns,

maxnoNRcellsSpectrumSharingWithE-UTRA,

maxnoofNrCellBands,

maxnoofSSBAreas

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

FROM X2AP-Constants;

-- **************************************************************

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

#### -- RACH INDICATION

--

-- **************************************************************

RachIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RachIndication-IEs}},

...

}

RachIndication-IEs X2AP-PROTOCOL-IES ::= {

{ ID id-RaReportIndicationList				CRITICALITY reject		TYPE RaReportIndicationList							PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- SCG FAILURE INFORMATION REPORT

--

-- **************************************************************

ScgFailureInformationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureInformationReport-IEs}},

...

}

ScgFailureInformationReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID							CRITICALITY ignore		TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID							CRITICALITY ignore		TYPE SgNB-UE-X2AP-ID						PRESENCE mandatory}|

{ ID id-SourcePSCellCGI							CRITICALITY ignore		TYPE NRCGI									PRESENCE optional }|

{ ID id-FailedPSCellCGI							CRITICALITY ignore		TYPE NRCGI									PRESENCE optional }|

{ ID id-SCG-FailureReportContainer				CRITICALITY ignore		TYPE SCG-FailureReportContainer		PRESENCE mandatory}|

{ ID id-TimeSCG-Failure						CRITICALITY ignore		TYPE TimeSCG-Failure						PRESENCE optional },

...

}

-- **************************************************************

--

#### -- SCG FAILURE TRANSFER

--

-- **************************************************************

ScgFailureTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureTransfer-IEs}},

...

}

ScgFailureTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY ignore		TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID					CRITICALITY ignore		TYPE SgNB-UE-X2AP-ID						PRESENCE mandatory},

...

}

END

-- ASN1STOP

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

9.3.5	Information Element definitions

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

#### -- S

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

SCG-FailureReportContainer ::= OCTET STRING

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

#### -- T

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

TimeSCG-Failure ::= INTEGER (0..1023)

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

X2AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) x2ap (2) version1 (1) x2ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM X2AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-handoverCancel												ProcedureCode ::= 1

id-loadIndication												ProcedureCode ::= 2

id-errorIndication												ProcedureCode ::= 3

id-snStatusTransfer												ProcedureCode ::= 4

id-uEContextRelease												ProcedureCode ::= 5

id-x2Setup														ProcedureCode ::= 6

id-reset														ProcedureCode ::= 7

id-eNBConfigurationUpdate										ProcedureCode ::= 8

id-resourceStatusReportingInitiation							ProcedureCode ::= 9

id-resourceStatusReporting										ProcedureCode ::= 10

id-privateMessage												ProcedureCode ::= 11

id-mobilitySettingsChange										ProcedureCode ::= 12

id-rLFIndication												ProcedureCode ::= 13

id-handoverReport												ProcedureCode ::= 14

id-cellActivation												ProcedureCode ::= 15

id-x2Release													ProcedureCode ::= 16

id-x2APMessageTransfer											ProcedureCode ::= 17

id-x2Removal													ProcedureCode ::= 18

id-seNBAdditionPreparation										ProcedureCode ::= 19

id-seNBReconfigurationCompletion								ProcedureCode ::= 20

id-meNBinitiatedSeNBModificationPreparation						ProcedureCode ::= 21

id-seNBinitiatedSeNBModification								ProcedureCode ::= 22

id-meNBinitiatedSeNBRelease										ProcedureCode ::= 23

id-seNBinitiatedSeNBRelease										ProcedureCode ::= 24

id-seNBCounterCheck												ProcedureCode ::= 25

id-retrieveUEContext											ProcedureCode ::= 26

id-sgNBAdditionPreparation										ProcedureCode ::= 27

id-sgNBReconfigurationCompletion								ProcedureCode ::= 28

id-meNBinitiatedSgNBModificationPreparation						ProcedureCode ::= 29

id-sgNBinitiatedSgNBModification								ProcedureCode ::= 30

id-meNBinitiatedSgNBRelease										ProcedureCode ::= 31

id-sgNBinitiatedSgNBRelease										ProcedureCode ::= 32

id-sgNBCounterCheck												ProcedureCode ::= 33

id-sgNBChange													ProcedureCode ::= 34

id-rRCTransfer													ProcedureCode ::= 35

id-endcX2Setup													ProcedureCode ::= 36

id-endcConfigurationUpdate										ProcedureCode ::= 37

id-secondaryRATDataUsageReport									ProcedureCode ::= 38

id-endcCellActivation											ProcedureCode ::= 39

id-endcPartialReset												ProcedureCode ::= 40

id-eUTRANRCellResourceCoordination								ProcedureCode ::= 41

id-SgNBActivityNotification										ProcedureCode ::= 42

id-endcX2Removal												ProcedureCode ::= 43

id-dataForwardingAddressIndication								ProcedureCode ::= 44

id-gNBStatusIndication											ProcedureCode ::= 45

id-deactivateTrace												ProcedureCode ::= 46

id-traceStart													ProcedureCode ::= 47

id-endcConfigurationTransfer									ProcedureCode ::= 48

id-handoverSuccess												ProcedureCode ::= 49

id-conditionalHandoverCancel									ProcedureCode ::= 50

id-earlyStatusTransfer											ProcedureCode ::= 51

id-cellTrafficTrace												ProcedureCode ::= 52

id-endcresourceStatusReporting									ProcedureCode ::= 53

id-endcresourceStatusReportingInitiation						ProcedureCode ::= 54

id-f1CTrafficTransfer											ProcedureCode ::= 55

id-UERadioCapabilityIDMapping									ProcedureCode ::= 56

id-accessAndMobilityIndication									ProcedureCode ::= 57

id-procedure-code-58-not-to-be-used								ProcedureCode ::= 58

id-CPC-cancel													ProcedureCode ::= 59

id-rachIndication												ProcedureCode ::= 60

id-scgFailureInformationReport									ProcedureCode ::= xx

id-scgFailureTransfer											ProcedureCode ::= xy

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

#### -- IEs

--

-- **************************************************************

id-E-RABs-Admitted-Item														ProtocolIE-ID ::= 0

///////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////

id-IABAuthorized															ProtocolIE-ID ::= 449

id-SourcePSCellCGI															ProtocolIE-ID ::= xx1

id-FailedPSCellCGI															ProtocolIE-ID ::= xx2

id-SCG-FailureReportContainer												ProtocolIE-ID ::= xx3

id-TimeSCG-Failure															ProtocolIE-ID ::= xx4

///////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////