3GPP TSG-RAN WG3 Meeting #128	R3-253852

Malta, MT, 19th – 23th May 2025

Title: 	(TP for BLCR to TS 38.401) OAM Requirements for Energy Saving for split architecture

Source: 	CMCC, Samsung, Huawei, NEC, CATT, ZTE, Nokia, Ericsson, Deutsche Telekom

Agenda item: 	11.4

Document Type:	Other

# Introduction

This contribution provides TP to reflect the agreement on OAM requirement for Energy Cost in split architecture.

# 2. TP for the BLCR to TS 38.401

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 7.11	Support of AI/ML for NG-RAN

### 7.11.1	General

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

For a split gNB, the Energy Cost (EC) of the gNB is the sum of the Energy Cost of its gNB-DUs.

### 7.11.2	OAM requirements

OAM configures the following on gNB-DU:

-	the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area.

-	the recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node's consumed energy.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>