3GPP TSG-RAN WG3 Meeting #128	R3-253887

Malta,MT, 19th – 23rd May 2025

Agenda item:	11.4

Source: 	Samsung, Ericsson, Huawei, ZTE, Deutsche Telekom

Title: 	(TP to BL CR for 38.425) UE performance enhancement for split architecture

Document for:	Text Proposal

# 1. Introduction

This paper provides text proposals on the BLCR of 38.425 for the UE performance enhancement in split architecture.

# 2. Text Proposals to BLCR of 38.425

<<<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>>>

## 5.2	NR user plane protocol layer services

NOTE 1:	In this section, NR user plane protocol layer services are also applicable to E-UTRA PDCP. With this understanding, each instance of NR PDCP can be replaced by E-UTRA PDCP.

The following functions are provided by the NR user plane protocol:

-	Provision of NR user plane specific sequence number information for user data transferred from the node hosting NR PDCP to the corresponding node for a specific data radio bearer.

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for user data associated with a specific data radio bearer.

-	Information of NR PDCP PDUs that were not delivered to the UE or not transmitted to the lower layers.

-	Information of NR PDCP PDUs transmitted to the lower layers for user data associated with a specific data radio bearer.

-	Information of downlink NR PDCP PDUs to be discarded for user data associated with a specific data radio bearer;

-	Information of the currently desired buffer size at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer.

-	Information of the currently desired data rate in bytes at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer;

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for retransmission user data associate with a specific data radio bearer;

-	Information of NR PDCP PDUs transmitted to the lower layers for retransmission user data associated with a specific data radio bearer.

-	Information of the specific events at the corresponding node.

-	Information on Radio Link Quality from the corresponding node for user data associated with a specific data radio bearer.

-	Information for QoS monitoring from the corresponding node for user data associated with a specific data radio bearer.

-	Information for UE performance feedback from the corresponding node for user data associated with a specific data radio bearer.

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 5.4.3.1	Successful operation

NOTE 1:	In this section, PDCP duplication and delay measurement related information are not applicable to E-UTRA PDCP.

The purpose of the Transfer of Assistance Information procedure is to provide assistance information to the node hosting the NR PDCP entity. Such information may be taken into consideration by the node hosting the NR PDCP entity for UP management and optimisation procedures.

An NR user plane protocol instance making use of the Transfer of Assistance Information procedure is associated to a single data radio bearer only.

The Transfer of Assistance Information procedure may be invoked if

-	the corresponding node decides to send the Radio Quality Assistance Information and/or the PDCP duplication activation suggestion to the node hosting the NR PDCP entity for the concerned data radio bearer or,

-	the corresponding node decides to send the Radio Quality Assistance Information to the node hosting the NR PDCP entity for the concerned RLC entity.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the QoS monitoring results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The Transfer of Assistance Information procedure may be invoked if the corresponding node is configured to perform collection of UE performance and to send the UE performance results to the node hosting the NR PDCP entity for the concerned data radio bearer.

The ASSISTANCE INFORMATION DATA frame may include one or more Radio Quality Assistance Information. The information shall consist of the information indicated in the Assistance Information Type.

The ASSISTANCE INFORMATION DATA shall be sent, if supported, when the corresponding node receives a DL USER DATA PDU including the Assistance Information Report Polling Flag set to 1.

The ASSISTANCE INFORMATION DATA frame may include the PDCP Duplication Activation Suggestion, which informs the node hosting the NR PDCP entity of the suggestion from the corresponding node on whether to activate or not activate DL PDCP duplication. The node hosting the NR PDCP entity may take this information into account to take a decision on whether to activate or not activate PDCP duplication.

The ASSISTANCE INFORMATION DATA frame may include the UL Delay or/and DL Delay measured by the corresponding node. The node hosting the NR PDCP entity may take this information into account to calculate the whole UL or/and DL delay of RAN.

The ASSISTANCE INFORMATION DATA frame may include UL Congestion Information and/or DL Congestion information measured by the corresponding node. The node hosting the NR PDCP entity shall, if supported, take this information into account to perform ECN marking in the NG-RAN node, or to further send it to the UPF for ECN marking or information exposure as specified in TS 23.501 [8].

Figure 5.4.3.1-1: Successful Transfer of Assistance Information Data

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 5.5.3.48	UL Delay DU Result

Description: This field indicates UL delay measured at the corresponding node in milliseconds for the concerned DRB over Uu interface. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the total UL delay for the concerned DRB and report to the UPF for the purpose of QoS monitoring as specified in [8], or for the purpose of deriving UE performance feedback. This information element is not applicable to E-UTRA PDCP.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.49	DL Delay DU Result

Description: This field indicates DL delay measured at the corresponding node in milliseconds for the concerned DRB over Uu interface. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the total DL delay for the concerned DRB and report to the UPF for the purpose of QoS monitoring as specified in [8], or for the purpose of deriving UE performance feedback. This information element is not applicable to E-UTRA PDCP.

Value range: {0..232-1}.

Field length: 4 octets.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>