3GPP TSG-RAN WG3 Meeting #128	R3-253894

Malta, MT, 19 – 23 May 2025

Agenda item:	20.2

Source:	Ericsson, Xiaomi, CATT, Nokia, ZTE, Samsung

Title:	(TP for TS 38.455) Support of Sample-based measurement (case 3b)

Document for:	other

# Annex A TP to 38.455 (support of case 3b)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP, sample-based UL-RTOA (FFS) ) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Channel Response Information | O |  | 9.2.Y | Applicable to sample-based UL-RTOA only |  |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: UL SRS-RSRPP in Additional Path Fourth Bit: Multiple UL AoA in Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Tenth bit: Mobile TRP location information Eleventh bit: SRS bandwidth aggregation used for joint UL positioning measurement. Twelfth bit: Aggregated Positioning SRS resources IDs used for joint UL positioning measurement. Thirteen bit: UL SRS-RSRPP in sample-based UL-RTOA. Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >>Sample-based UL-RTOA |  |  | 9.2.X |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used Positioning SRS resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| >>>Point A | M |  | INTEGER (0..3279165) |  | YES | ignore |
| >>>SCS Specific Carrier |  | 1 |  |  | YES | ignore |
| >>>>Offset To Carrier | M |  | INTEGER (0..2199, …) |  |  |  |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …, kHz480, kHz960) |  |  |  |
| >>>>Carrier Bandwidth | M |  | INTERGER (1..275, …) |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoaggregatedPosSRS-Resources | Maximum no of aggregated Positioning SRS resources per UL BWP. Value is 3. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.2.X	Sample-based UL-RTOA (FFS)

This information element contains the Channel Response of the sample-based UL-RTOA.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Channel Response List |  | 1 |  |  |
| >Channel Response Item |  | 1..<maxnoofChannelRes> |  |  |
| >>CHOICE Timing Information | M |  |  |  |
| >>>k0 |  |  | INTEGER (0.. 1970049) | TS 38.133 [16] |
| >>>k1 |  |  | INTEGER (0.. 985025) | TS 38.133 [16] |
| >>>k2 |  |  | INTEGER (0.. 492513) | TS 38.133 [16] |
| >>>k3 |  |  | INTEGER (0.. 246257) | TS 38.133 [16] |
| >>>k4 |  |  | INTEGER (0.. 123129) | TS 38.133 [16] |
| >>>k5 |  |  | INTEGER (0.. 61565) | TS 38.133 [16] |
| >>Power Information | O |  | UL SRS-RSRPP 9.2.72 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofChannelRes | Maximum no of channel response. Value is 24 |

### 9.2.Y	Channel Response Information

This IE contains information about the requested sample-based UL-RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Channel Response Window Size | M |  | ENUMERATED (32, 64, 128, …) | Represents the window size over which channel response values are selected. |
| Channel Response Number | M |  | ENUMERATED (8, 16, 24, …) | Represents the number of channel responses selected over the channel response window size. |

Next change

ASN.1 to be added later

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>