3GPP TSG-RAN WG3 Meeting #128	R3-253942

Malta, MT, 19th May– 23rd May 2025

Agenda Item:       10.2

Source:	ZTE Corporation, Nokia, Lenovo, Samsung

Title: 		(TP for SON BLCR for 38.401) MRO for LTM

Document for:	Discussion and Approval

# 1	Introduction

The TP is capturing the following agreements in RAN3#128.

BFR shortly after successful LTM cell switch to the wrong beam(Case 1)

RACHless LTM cell switch failure due to wrong beam (Case 2)

Network-based solution for the LTM cell switch failure due to wrong beam using the same signaling flow as for Case1.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization.

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization.

Target DU needs to send the reconnect/re-established/recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

LTM cell switch failure due to outdated TA

RAN3 supports network-based solution for out dated TA.

Reusing the same signalling flow as case1 and case2.

# 2	TP for BL CR for TS38.401

<< Start of Change >>

## 7.X	MRO support for LTM

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt).The target gNB-DU performs initial analysis and may send the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while reconnecting/re-establishing/recovering to the same target cell but to a beam different from the beam used during the failed LTM cell switch execution. The target gNB-DU may send the reconnected/re-established/recovery beam information to the source gNB-DU via the gNB-CU. The source gNB-DU performs root cause analysis to detect RACH-less LTM failure caused by wrong beam selection.

The target gNB-DU identifies that the UE successfully performed a RACH-based access while reconnecting/re-establishing/recovering to the same target cell and same target beam but with TA value different from the one which was included in CU-DU CELL SWITCH NOTIFICATION message received from gNB-CU at LTM Cell Switch execution. The target gNB-DU sends the TA value used at successful RACH-based access to the source gNB-DU via the gNB-CU.

<< End of Change >>