3GPP TSG-RAN WG3 Meeting #128	R3-253968

# Malta, MT, 19th – 23th May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.300) Introduction of Ambient IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-06-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce the Ambient IoT features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3#127: - Add the description part of A-IoT architecture aspects. - Add the description part of inventory and command procedure. RAN3#127bis: - Update the description part of A-IoT architecture aspects. - Update the description part of inventory and command procedure. - Add the description part of A-IoT protocol stack. - Add the description part of locating A-IoT devices. RAN3#128: - Update the description part of A-IoT architecture aspects. - Update the description part of inventory and command procedure. - Add the description part of A-IoT CN initiated A-IoT Session Release procedure and gNB initiated A-IoT Session Release procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Ambient IoT features will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 16.xx.x1, 16.xx.x2, 16.xx.x3, 16.xx.x4, 16.xx.x5, 16.xx.x6, 16.xx.x7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.410 CR 0053 TS 38.401 CR 0469 TS 38.412 CR 0025 TS 38.413 CR 1260 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Capture the agreed TP R3-250906 and R3-250908 in RAN3#127 meeting. Resubmission to RAN3#127bis meeting. Capture the agreed TP R3-252328, R3-252330, R3-252459 and R3-252477 in RAN3#127bis meeting. Resubmission to RAN3#128 meeting. Capture the agreed TP R3-253792 and R3-253935 in RAN3#128 meeting. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[6]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[9]	3GPP TS 37.324: " E-UTRA and NR; Service Data Protocol (SDAP) specification".

[10]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[14]	3GPP TS 22.168: "Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1".

[15]	3GPP TS 22.268: "Public Warning System (PWS) Requirements".

[16]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[17]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[18]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[19]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[20]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer"

[21]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[22]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[23]	IETF RFC 4960 (2007-09): "Stream Control Transmission Protocol".

[24]	3GPP TS 26.114: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[25]	Void.

[26]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[27]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

[28]	3GPP TS 24.501: "NR; Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[29]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[30]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[31]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[32]	3GPP TS 38.470: "NG-RAN; F1 application protocol (F1AP) ".

[33]	3GPP TS 38.425: "NG-RAN; NR user plane protocol".

[34]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[35]	3GPP TS 38.101-2: "User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[36]	3GPP TS 38.101-3: "User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[37]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

[38]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[39]	3GPP TS 22.104 "Service requirements for cyber-physical control applications in vertical domains".

[40]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[41]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[42]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[43]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[44]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[45]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[46]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[47]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[48]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[49]	3GPP TS 28.541: "5G Network Resource Model (NRM)".

[50]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[51]	NIMA TR 8350.2, Third Edition, Amendment 1, 3 January 2000: "DEPARTMENT OF DEFENSE WORLD GEODETIC SYSTEM 1984".

[52]	3GPP TS 38.211: "NR; Physical channels and modulation".

[53]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS)".

[54]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[55]	3GPP TS 24.554: "Technical Specification Group Core Network and Terminals; Proximity-services (ProSe) in 5G System (5GS) protocol".

[56]	3GPP TS 38.214: "Technical Specification Group Radio Access Network; NR; Physical layer procedures for data".

[57]	3GPP TR 38.835: "NR; Study on XR enhancements for NR".

[58]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[59]	3GPP TS 38.215: "NR; Physical layer measurements".

[60]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[61]	IETF RFC 9330: "Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[62]	IETF RFC 9331: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)".

[63]	IETF RFC 9332: "Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[64]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[65]	3GPP TS 38.351: "NR; Sidelink Relay Adaptation Protocol (SRAP) Specification".

[xx]	3GPP TS 23.369: " Architecture support for Ambient power-enabled Internet of Things; Stage 2".

3	Abbreviations and Definitions

3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A2X	Aircraft-to-Everything

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

AR	Augmented Reality

ARP	Allocation and Retention Priority

/******* Unchanged part skipped *******/

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

IoT	Internet of Things

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LBT	Listen Before Talk

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

2Rx XR UE: two antenna port XR UE as specified in TS 38.101-1 [18].

A2X communication: A communication to support A2X services leveraging PC5 reference points. A2X services are realized by various types of A2X applications, i.e. BRID or DAA.

Aerial UE communication: functionality enabling Aerial UE function, as defined in 16.18.

A-IoT CN node: The core network node which the gNB connects to 5GC for A-IoT operation. More specifically, it is the AIOTF in case the gNB directly connects with the AIOTF, and it includes both the AMF and AIOTF in case the gNB connects with the AIOTF via AMF.

Air to Ground network: An NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000m and take-off/landing altitudes down to 3000m.

BH RLC channel: an RLC channel between two nodes, which is used to transport backhaul packets.

Boundary IAB-node: as defined in TS 38.401 [4].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

16.xx	Support of Ambient IoT

16.xx.x1	Introduction

16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or multiple readers.

-	An A-IoT Area may include readers belonging to the same or different gNB.

-	One reader only belongs to one gNB.

-	One reader can be mapped to one or multiple A-IoT Areas.

-	As specified in TS 23.369 [xx], the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct connectivity with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF).

NOTE 1:	A deployment with both direct connectivity and indirect connectivity is not expected.

-	In this version of the specification, A-IoT specific information is transported between the gNB and the A-IoT  CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

The AIOTF may request the gNB to perform an A-IoT operation within a specific area (requested service area). The requested service area may be indicated by means of a list of A-IoT Areas and/or a list of readers. The A-IoT Area is represented by an A-IoT Area ID.

The AIOTF is aware of the area within which the gNB supports provision of A-IoT services by means of OAM configuration. This area may be represented as A-IoT Areas and/or the readers supported by the gNB.

The AIOTF may also be aware of the reader’s location by means of OAM configuration.

The AIOTF is aware of the mapping relationship among gNBs, readers and A-IoT areas by means of OAM configuration, as needed.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the Expected D2R message size and may include Inventory Assistance Information e.g. Approximate number of Target A-IoT devices.

Editor’s Note 1:	Whether the Expected D2R message size is mandatory is FFS.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 2:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

7.	Upon completion of the inventory procedure the gNB may send an Inventory Complete Indication.

Editor’s Note 3:	It is FFS whether this inventory complete indication is done via a new procedure or introducing indication IE in the current Inventory Report message.

16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices or in parallel to other command procedures targeting other A-IoT devices within the same session.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB in order to send a command to a single A-IoT device. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU corresponding to the targeted A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size, command type (FFS).

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response messageincluding the AIOTF Identifier  the Correlation ID and the RAN A-IoT device NGAP ID received in the Command Request, and the A-IoT NAS PDU received from the device.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

### 16.xx.x5	A-IoT CN initiated A-IoT Session Release procedure

Figure 16.xx.x5-1: A-IoT CN initiated A-IoT Session Release procedure

1.	The A-IoT CN node initiates the A-IoT Session Release procedure over NG-C by sending the A-IoT Session Release Command message to the gNB. The A-IoT Session Release Command message includes the Correlation ID and the AIOTF Identifier to identify the session to be released.

2.	The gNB releases the session context, the associated NG resources and the A-IoT radio resources.

3.	The gNB confirms the release to the A-IoT CN node by replying with the A-IoT Session Release Complete message. The A-IoT Session Release Complete message includes the AIOTF Identifier and the Correlation ID received in the A-IoT Session Release Command message.

### 16.xx.x6	gNB initiated A-IoT Session Release procedure

Figure 16.xx.x6-1: gNB initiated A-IoT Session Release procedure

1.	The gNB initiates the A-IoT Session Release Request procedure over NG-C by sending the A-IoT Session Release Request message to the A-IoT CN node in order to request the release of an A-IoT session. The A-IoT Session Release Request message includes the Correlation ID and the AIOTF Identifier to identify the session requested to be released.

2.	The A-IoT CN node accepts the request and triggers the CN initiated A-IoT Session Release procedure.

3.	The CN initiated A-IoT Session Release procedure is performed as described in section 16.xx.x5.

Editor’s Note 4:	The exact name of the AIoT Session Release Request message at step 1 is FFS.

Editor’s Note 5:	Whether to keep step 2 and step 3 in the figure is FFS.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

16.xx.x7 Locating A-IoT devices

In this version of specification, the A-IoT device location is reported by the gNB at Reader ID granularity to the AIOTF via Inventory Report procedure. The AIOTF may use the A-IoT device location information in performing A-IoT operations.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>