# 3GPP TSG-RAN WG3 Meeting #128	R3-253781

# Malta, MT, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0471 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce eRedcap indication for eRedCap UE access control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Telecom, China Unicom, Samsung, Huawei, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2025-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-17, during Redcap UE initial access, a NR RedCap UE Indication is introduced. Also in Rel-18, during eRedcap UE initial access, a NR eRedCap UE Indication is introduced. However, the NR eRedcap UE indication is missing in stage 2 spec in case of split gNB deployment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a NR eRedCap UE Indication in the INITIAL UL RRC MESSAGE TRANSFER message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact because the change affects only the eRedcap function. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of split gNB deployment, the NR eRedcap UE indication is missing, then stage 2 spec is not aligned with stage 3 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-253252, add consigned companies: Samsung, Huawei, Nokia, Ericsson |  |  |  |  |  |  |  |  |

--------------------------------------Start of the change---------------------------------------------

8.1	UE Initial Access

The signalling flow for UE Initial access is shown in Figure 8.1-1.

Figure 8.1-1: UE Initial Access procedure

1.	The UE sends an RRCSetupRequest message to the gNB-DU.

2.	The gNB-DU includes the RRC message and, if the UE is admitted, the corresponding low layer configuration for the UE in the INITIAL UL RRC MESSAGE TRANSFER message and transfers to the gNB-CU. The INITIAL UL RRC MESSAGE TRANSFER message includes the C-RNTI allocated by the gNB-DU. If the gNB-DU identifies the UE as a Reduced Capability UE during the random access procedure, a NR RedCap UE Indication or a NR eRedCap UE Indication is provided in the INITIAL UL RRC MESSAGE TRANSFER message.

3.	The gNB-CU allocates a gNB-CU UE F1AP ID for the UE and generates a RRCSetup message towards UE. The RRC message is encapsulated in -the DL RRC MESSAGE TRANSFER message.

4.	The gNB-DU sends the RRCSetup message to the UE.

5.	The UE sends the RRC CONNECTION SETUP COMPLETE message to the gNB-DU.

6.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

7.	The gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

8.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

9.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the UE context in the gNB-DU. In this message, it may also encapsulate the SecurityModeCommand message. In case of NG-RAN sharing, the gNB-CU includes the serving PLMN ID (for SNPNs the serving SNPN ID).

10.	The gNB-DU sends the SecurityModeCommand message to the UE.

11.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message to the gNB-CU.

12.	The UE responds with the SecurityModeComplete message.

13.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

14.	The gNB-CU generates the RRCReconfiguration message and encapsulates it in the DL RRC MESSAGE TRANSFER message.

15.	The gNB-DU sends RRCReconfiguration message to the UE.

16.	The UE sends RRCReconfigurationComplete message to the gNB-DU.

17.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

18.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

--------------------------------------Next of the change---------------------------------------------

8.7 	RRC connection reestablishment

This procedure is used for the case that UE tries to reestablish the RRC connection, as shown in Figure 8.7-1.

Figure 8.7-1: RRC connection reestablishment procedure

1.	The UE sends a preamble to the gNB-DU.

2.	The gNB-DU allocates new C-RNTI and responds with RAR.

3.	The UE sends an RRCReestablishmentRequest message to the gNB-DU, which contains old C-RNTI and old PCI.

4.	The gNB-DU includes the RRC message and, if the UE is admitted, the corresponding low layer configuration for the UE in the INITIAL UL RRC MESSAGE TRANSFER message and transfers to the gNB-CU. The INITIAL UL RRC MESSAGE TRANSFER message includes the new C-RNTI. If the gNB-DU identifies the UE as a Reduced Capability UE during the random access procedure, a NR RedCap UE Indication or a NR eRedCap UE Indication is provided in the INITIAL UL RRC MESSAGE TRANSFER message.

5.	The gNB-CU includes an RRCReestablishment message and transfers to the gNB-DU. If the UE requests to re-establish RRC connection in the last serving gNB-DU, the DL RRC MESSAGE TRANSFER message shall include old gNB-DU UE F1AP ID.

6.	The gNB-DU retrieves the UE context based on the old gNB-DU UE F1AP ID, and replaces old C-RNTI/PCI with new C-RNTI/PCI. It sends the RRCReestablishment message to UE.

7-8.	The UE sends an RRCReestablishmentComplete message to the gNB-DU. The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends to the gNB-CU.

9-10.	The gNB-CU triggers an UE Context Modification procedure by sending UE CONTEXT MODIFICATION REQUEST message, which may include DRBs to be modified and released list. The gNB-DU responses with the UE CONTEXT MODIFICATION RESPONSE message.

9'-10'.	The gNB-DU triggers an UE Context Modification procedure by sending UE CONTEXT MODIFICATION REQUIRED message, which may include DRBs to be modified and released list. The gNB-CU responses with UE CONTEXT MODIFICATION CONFIRM message.

NOTE 1:	Here it is assumed that the UE accessed the original gNB-DU where the UE context is available for that UE, and either steps 9-10 or steps 9’-10’ may be executed or both could be skipped.

NOTE 2:	If the UE accessed from a gNB-DU other than the original one, the gNB-CU should trigger the UE Context Setup procedure toward this new gNB-DU.

11-12.	The gNB-CU includes an RRCReconfiguration message into the DL RRC MESSAGE TRANSFER message and transfers to the gNB-DU. The gNB-DU forwards it to the UE.

13-14.	The UE sends an RRCReconfigurationComplete message to the gNB-DU, and the gNB-DU forwards it to the gNB-CU.

--------------------------------------End of the change---------------------------------------------