3GPP TSG-RAN WG3 #128	R3-253802

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	Reply LS on Ambient IoT progress of RAN3 on A-IoT Area

Response to:	R3-253015/S2-2504288

Release:	Release 19

Work Item:	Ambient_IoT_solutions-Core

Source:	RAN3

To:	SA2

Cc:	SA5

Contact Person:

Name:	Wuyang Zheng

E-mail Address:	zheng_wuyang@nec.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

RAN3 discussed the SA2's LS regarding the supported area and would like to provide the following clarifications:

- SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area.

Answer: The “supported Area” in the A-IoT RAN information is a new type of area referred as “A-IoT Area”, and each A-IoT Area is identified by an A-IoT Area ID. And the A-IoT Area ID is encoded as:

A-IoT Area ID = PLMN ID + NID (optional) + A-IoT Area Code (OCTET STRING (SIZE(3)))

- SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

Answer: RAN3 agreed that one reader can map to one or multiple A-IoT Area ID(s). And the mapping relationships among gNBs, readers and A-IoT areas are configured by OAM.

Agreements in RAN3#128 related to A-IoT Area is copied below:

The new A-IoT Area is encoded as an A-IoT Area ID. A-IoT Area ID = PLMN ID +NID(optional) + A-IoT Area Code (OCTET STRING (SIZE(3))) One reader only belongs to one gNB. One reader can map to one or multiple A-IoT Area ID(s). One A-IoT Area may include readers belong to the same or different gNBs. One gNB may serve multiple readers which belong to the same or different A-IoT Areas. OAM configures in the AIOTF the mapping relationships among gNBs, readers and A-IoT areas, as needed.

2. Actions:

To SA2:

ACTION: RAN3 kindly asks SA2 to take the above feedback into account.

3. Date of Next TSG-RAN WG3 Meetings:

TSG RAN WG3 Meeting RAN3#129		2025-08-25 - 2025-08-29		Bangalore, India

TSG RAN WG3 Meeting RAN3#129bis		2025-10-13 - 2025-10-17		Prague, Czech