3GPP TSG-RAN WG3 #128	          R3-253810

Malta, MT, 19th – 23th May  2025

Agenda item:	20.2

Source:	Ericsson

Title:	Summary of offline discussion on CB: # AIPHY

Document for:	Discussion and Decision

# 1	Introduction

This contribution provides a summary of offline discussion on CB: # AIPHY.

CB: # AIPHY

- Check LSout in R3-253806 and R3-253807

- Work on the details of solution for Case3a

- Check Case3b progress

- Stage2 TPs if agreeable

(moderator - E///)

The offline discussion will have two phases:

- Phase 1: for the discussion on the questions. Please share your view on the questions by 10:00 Thursday May 22nd (Malta time)

- Phase 2: Based on the result of Phase 1, TPs will be prepared. Phase 2 will discuss the TPs.

# 2	For the Chair’s Notes

Issue#1 LSout in R3-253806 and R3-253807

- Agree R3-253806 and R3-253807

Issue#2 Case 3a data collection

- Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator.

- FFS if UE location can be requested.

- The Positioning Data Collection Needed IE is introduced in the MEASUREMENT REPORT message.

- Introduce POSITIONING DATA COLLECTION REPORT message as class 2 procedure including Positioning Data Information IE and RAN/LMF Measurement IDs

- Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason.

- RAN3 confirms that Performance monitoring is done in the RAN in case 3a.

- RAN3 confirms the WA: For case 3a data collection, Part A is known internally by the gNB and is not signalled over any network interface.

- Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion

- FFS: if LMF should specifically request for AIML inferred measurement

Issue#4 TPs for checking

- Agree R3-253888 NRPPa TP capturing above proposals (E///)

- Agree R3-253889 (TP to TS 38.300) Stage 2 for case 3a (Nokia)

- Agree R3-253890 (TP to TS 38.305) Stage 2 for cases 3a/3b (Qualcomm)

- Agree R3-253891 (NGAP TP) adding new procedure name. (Huawei)

- Agree R3-253892 (TP to TS 38.401) (ZTE)

Issue#5 Case 3b

- Agree R3-253893: (TP for TS 38.473) Support of Sample-based measurement (case 3b) adding FFS on all IEs and remove Nt from the TRP report (Xiaomi)

- Agree R3-253894: (TP for TS 38.455) Support of Sample-based measurement (case 3b) adding FFS on all IEs and remove Nt from the TRP report (E///)

# 3	LSout in R3-253806 and R3-253807

Online session agreed the following:

| [Draft LS] Discussion on Relpy LSs from RAN2 (ZTE Corporation) | Other Move to 20.2 For both RAN2 agreements, they can be supported based on current management based immediate MDT mechanism without impact on RAN3. Rev in R3-253807 |
| --- | --- |
| (Draft LS out) Reply LS on signalling feasibility of dataset and parameter sharing (Xiaomi, MediaTek, Qualcomm Incorporated, Huawei, Apple, China Mobile) | LS out To: RAN2, RAN1 CC: SA2, SA5, SA3 It is feasible from RAN3 point of view for gNB to transfer the model related data to OAM. The exchange of AI/ML-related data between the CN and gNB is not supported in R19, and no evaluation on the feasibility in Rel-19 has been done. Rev in R3-253806 |

Moderator asks ZTE and Xiaomi to upload drafts to R3-253806 and R3-253807 to draft folder for final checking ftp://10.10.10.10/RAN/RAN3/Inbox/Drafts/CB%20%23%20AIPHY

P-0: It is proposed to agree R3-253806 and R3-253807 with text as agreed online.

- Case 3a discussion

## 4-1) Stage 3 details for opportunistic signalling (IE, and new procedures)

In RAN3#127bis meeting, the following call flow was agreed for the opportunistic method as part of the AI/ML model training.

Figure 1: Positioning Information collection for UEs under positioning

It was agreed that in step 4 the gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

It is proposed to discuss how to encode the Positioning Data Collection Needed indication.

- Option 1: ENUMERATED(true, …)

- Option 2: ENUMERATED (UL RTOA, LOS/NLOS, ..., UL RTOA and LOS/NLOS)

- Option 3: a SEQUENCE containing a CHOICE between “UE location” encoded as ENUMERATED(true,…) and a BITSTRING where each position in the bitmap represents a requested measurement type: UL-RTOA, gNB Rx-tx, other attributes, etc.

### 9.2.Z	Positioning Data Collection Needed

This information element contains the requested positioning information for data collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Positioning data Information Type | M |  |  |  |
| >UE location | M |  | ENUMERATED(true,…) |  |
| >Measurement type | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents a requested measurement type: first bit: UL-RTOA Second bit: gNB Rx-Tx Time Difference Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement type’, Value ‘0’ indicates ‘not requested’. |

Q1-1: which option is preferred by companies?

Q1-2: Should the IE also be introduced in the MEASUREMENT REPORT message?

| Company | Option | Comment |
| --- | --- | --- |
| Ericsson | Q1-1: Option 3 Q1-2: Yes | We prefer option 3 as it is more future-proof and allows gNB to request exact information it requires for training, Bitstring can be extended for future usage. In our view the new IE should be encoded in dedicated tabular. We realize the RAN should include the bitstring in an area of interest, i.e., in a list of TRPs. |
| CATT | See Comments | For Q1-1, generally we are fine to introduce a requested type to say exact information requested by the gNB for training. For the Encoding, no need to use the choice type, just a bitmap. Whether “UE location” info could be requested or provided is not confirmed, if it’s added in the future, it could also be added via the bitmap. For Q1-2, maybe we should be careful to use two messages to request for training data. For example, in Measurement Response, gNB indicate UL-RTOA is needed, in following-up measurement report, it include the other bitmap, how to understand and how to proceed? Wholly replace the previous bitmap or combine the bitmaps??? So, it’s prefer not to extend the measurement report. |
| Xiaomi | Q1-1: option 2 (encoding as bitstring is also ok) Q1-2: yes | Ok to use bitstring, which can be used to request the data of measurement type, and user location as well, but for user location, we need further check whether this is allowed to be provided to the gNB for the PRU and UE with user consent. We can put FFS on user location. We think the data collection needed is for a specific measurement, so it’s ok to extend to measurement report. |
| ZTE | See comments | Regarding the Q1, the use of a Choice structure is problematic. If a Choice structure is used, it implies that only one item can be requested—either the UE location or the Measurement type, but not both. Moreover, whether UE location can be requested in this context is still under discussion and has not been confirmed. A better approach would be to use a bitmap to indicate which measurement types are being requested, similar to the Report Characteristics IE used in AI/ML for NG-RAN. This allows for greater flexibility and extensibility without limiting the request to a single measurement. Regarding the Q2, this approach deviates from the procedure flow agreed in the previous meeting. We do not recommend introducing a new request indication in a separate message, as it would complicate the established signaling procedure. |
| NEC | Q1-1: Option 2 Q1-2: No | Regarding Q1, Option 2 is preferred. For Option 1, We partly agree with ZTE that “only one item can be requested”. For Option 3, the UE location should not be provided from LMF to gNB, as the gNB is not allowed to obtain the UE location according to existing specification. Regarding Q2, agree with CATT/ZTE that additional IE is unnecessary as we have the similar IE in previous procedure. |
| CMCC | Q1-1: option 1, expendable for future new part b Q1-2 prefer a new class 2 positioning data collection report |  |
| Qualcomm | Q1-1: Option 3 Q1-2: Yes | For Q1-2, if periodic measurements are requested for an ongoing positioning session anyhow, a gNB may use these UL transmissions for data collection as well and may then want the labels periodically. Feasibility/success of providing the labels depends on the reporting interval for the positioning measurement reporting. |
| CEWiT | Q1-1: Option 1 or Option 3 Q1-2: Yes | For Question 1-1 we see the option 1 and option 3 works. Solution 1 is for ongoing positioning which means positioning measurement type is already selected and LMF can provide same one that is configured to be measured. SO mere indication to provide same back works. To support the periodic measurements, it is necessary to add in MEASUREMENT REPORT too. We are curious why UL-AOA measurement is skipped from measurement type?? |
| Nokia | Q1-1: Option 3 Q1-2: Yes | For Q1-1, we support option 3 but the specific IEs should be FFS. There is no RAN1 agreement that UE location information is sent from LMF to gNB. The only related agreement from RAN1 is: For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. |
| Moderator’s conclusion: - Majority of companies prefer introducing a bitmap in the new IE indicating the requested positioning data type by gNB. FFS if UE location can also be requested - For periodic UL SRS measurements, the gNB may want to receive the labels periodically: this can be indicated in MEASUREMENT REPORT, or indicated by the gNB in the first request with a periodicity, or new class 2 message. Simplest way is to add the IE in the MEASUREMENT REPORT message, assuming the UL SRS periodicity may be re-used. |  |  |

P1-1: Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with LoS/NLoS indicator.

FFS on UE location.

P1-2: the Positioning Data Collection Needed IE is introduced in the MEASUREMENT REPORT message.

Several companies propose to define the new class 2 POSITIONING DATA COLLECTION REPORT message as below, containing:

- the correlation information: NRPPa Transaction ID, LMF Measurement ID, RAN Measurement ID

- Positioning Data Information, which according to RAN1 agreements contains measurements and LoS/NLoS information. RAN3 assumed for several meetings that  it should also contain UE location information.

#### 9.1.x.1	POSITIONING DATA COLLECTION REPORT

This message is sent by the LMF to report information related to UE location.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Data Information | O |  | 9.2.X |  | YES | ignore |

### 9.2.x	Positioning Data Information

This IE contains the Positioning Data Information.

| IE/Group Name | Presence |  | Range |  | IE Type and Reference |  | Semantics Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHOICE Data Collection Information Type | M |  |  |  |  |  |  |  |
| >UE location | M |  |  |  |  |  |  |  |
| >>Location Information | M |  |  |  | OCTET STRING |  | Location of the mobile TRP, Includes the locationEstimate IE as defined in TS 37.355 [14]. |  |
| >>Time Stamp | O |  |  |  | 9.2.42 |  | Location time stamp. |  |
| >Measurement | M |  |  |  |  |  |  |  |
| >>TRP Measurement List |  |  | 0..1 |  |  |  | Indicates the area of interest. |  |
| >>>TRP Measurement Item |  |  | 1..<maxnoofMeasTRPs> |  |  |  |  |  |
| >>>>TRP ID | M |  |  |  | 9.2.24 |  |  |  |
| >>>>UL RTOA | O |  |  |  | 9.2.39 |  |  |  |
| >>>>gNB Rx-Tx Time Difference | O |  | 9.2.40 |  |
| >>>>Time Stamp | O |  |  |  | 9.2.42 |  |  |  |
| >>>>Measurement Quality | O |  |  |  | 9.2.43 |  |  |  |
| >>>>LoS/NLoS Information | O |  |  |  | 9.2.77 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

Q2:  Companies provide comments on whether they are fine with the encoding of the message and inclusion of the above described IEs: correlation information and Positioning Data Information

| Company | Comment |
| --- | --- |
| Ericsson | Yes. Fine to keep 9.2.x details as FFS |
| CATT | Fine with 9.1.x.1, 9.2.x could be FFS. |
| Xiaomi | Prefer to put “FFS” on user location |
| ZTE | Put FFS on user location. |
| NEC | >>Location Information; M; OCTET STRING; Location of the mobile TRP, Includes the locationEstimate IE as defined in TS 37.355 [14].. 1. For above message in 9.2.x, we don't agree and have serious concerns. As analysed in our tdoc R3-253212, according to RAN1 agreement, if label incudes timing information, then the label is provided by LMF to gNB in the format of timing information but not UE location, because UE location belongs to UE privacy information, which cannot be provided to gNB explicitly. Agreement (RAN1#120bis) For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Moreover, for case 3a, the label is never be UE location itself. It should be "timing information and/or LOS/NLOS indicator" We think in 9.2.x, the Positioning Data Information contains timing information (M), LOS/NLOS indicator (O), time stamp (O). UE location and location information should not be there. 2. For 9.1.x.1, we also have concerns because we think time stamp of the part A and time stamp of part B are enough to correlate part A and part B, we don't think these IDs are necessary (at least for FFS). 3. One question, if this IE is used to provide the UE location to gNB, why “Semantics Description” for Location Information is the “the Location of the mobile TRP” rather than “the Location of the mobile UE”? |
| CMCC | Put FFS on user location. |
| Qualcomm | Looks O.K. |
| CEWiT | We are fine with 9.1.x.1 and 9.2.x. For Location Information semantics description, it is cross-referred to TS 37.355. We would prefer to introduce new IE in TS 38.455. |
| Nokia | 9.1.x.1 is OK. For 9.2.x, it is generally OK to have this structure but related to UE location there is no RAN1 agreement. Therefore, we propose to remove UE location. |
| Moderator’s conclusion: - OK to define new procedure POSITIONING DATA COLLECTION REPORT message and new Positioning Data Information IE with FFS on UE location |  |

P2: Agree TP for NRPPa BL CR introducing POSITIONING DATA COLLECTION REPORT message as class 2 procedure and new Positioning Data Information IE, with FFS on UE location

Introduce RAN/LMF Measurement IDs in the message

Details of the TP to be checked offline

Furthermore, several companies have proposed to define a new Positioning Data Collection Failure in case the LMF cannot provide any UE location related information to the NG-RAN node.

One company mentions to define the new POSITIONING DATA COLLECTION REPORT with OPTIONAL IEs, and a new IE defining that “Positioning Data Information Unavailable” IE, referring that LMF cannot provide any of the requested ground truths.

It is proposed to discuss the following options:

- Option 1: introduce new class 2 Positioning Data Collection Failure procedure

- Option 2: define new IE in the POSITIONING DATA COLLECTION REPORT, indicating the unavailability of any Positioning Data that could be provided by LMF.

- Option 3: keep FFS.

Q3: which option is preferred by companies?

| Company | Option | Comment |
| --- | --- | --- |
| Ericsson | Option 1 | Option 1 is cleaner. But we are fine to consider alternative 2 and discuss if cause values are needed. |
| CATT | Slightly prefer Option 2 | As Positioning Data Information is optional, not include it in the report could implicitly indicate the requested data could not be provided. Whether need to add additional indication is FFS. |
| Xiaomi | Option 1 with rewording | Rewording the name to “Positioning Data Collection Failure Indication”, meaning it is an indication procedure, instead of procedure failure, to solve the comments received online. |
| ZTE | Option 1 | Option 1 is more clearer. |
| NEC |  | Both Options are OK. Option 2 is preferred. |
| CMCC | Option 3 |  |
| Qualcomm | Option 2 | In particular when labels can be requested/provided periodically (see Q1-2). |
| CEWiT | Option 1 | Option 1 is cleaner and does not intertwin the functionality. Option 2 is fine but it should have cause code included. |
| Nokia | Option 2 | In Option 2 we can include additional IEs if needed for cases when no data can be provided by the LMF. We don’t consider data unavailability as a failure case and therefore we don’t think that we should use cause values. |
| Moderator’s conclusion: - There is a split of preference. Considering the case of periodic reporting it is preferred to introduce a new IE in the POSITIONING DATA COLLECTION REPORT message indicating unavailability of the requested GT. |  |  |

P3: Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason.

## 4-2) Model inference

It is proposed by several companies (E///, SS, CMCC, Huawei, Xiaomi, CT, QC) to add a new flag for model inference in the TRP Measurement Report:

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| //skipped rows// |  |  |  |  |  |  |
| >>>PCI | O |  | INTEGER (0..1007) |  | YES | ignore |
| >Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >Inferred measurement | O |  | ENUMERATED(true, …) | This IE is only valid for UL-RTOA, gNB Rx-Tx Time Difference and LoS/NLoS Information. | YES | ignore |

NOTE, RAN1 has agreed that LMF needs to distinguish Rel-19 FROM LEGACY:

Conclusion For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, - From RAN1 perspective, LMF shall be able to distinguish whether the timing information is pre Rel-19 legacy timing measurement or Rel-19 Case 3a timing measurement. - It is up to RAN3 to decide how to ensure that LMF can distinguish between the two types of timing measurement.

The motivation is to allow LMF to know that gNB has generated inferred measurements and stores information for analytics purposes.

Q4: are companies OK to introduce this new flag?

| Company | Y/N | Comment |
| --- | --- | --- |
| Ericsson | Yes | As compromise, we can capture that as WA that LMF’s awareness of the measurement being AI/ML inferred by gNB does not mean that monitoring is performed by LMF or that LMF changes behaviour. |
| CATT | No | Such indicator seems not necessary, as we do not see what’s the expected behaviours when LMF receives this indicator. |
| Xiaomi | Yes | Maybe only the timing information needs to be applied. It’s possible that timing information is generated by AI/ML (AI/ML-based timing info is based on LOS), while LOS/NLOS is generated via legacy (e.g., could be NLOS), to avoid the misalignment, we need one flag for AI/ML-based timing info. |
| ZTE | No | We do not see a clear benefit in indicating whether the measurement is AI-generated or non-AI-generated. From the LMF’s perspective, it only needs to use the measurement report received from the gNB to calculate the UE’s location. There is no need for the LMF to be aware of how the measurement was obtained. |
| NEC | No | Is the intention of this flag to indicate that the measurement is generated from AI/ML model or from legacy way? We support to let the LMF know whether the reported measurement is generated from AI/ML model or from legacy way. However, we still think it is premature to discuss this issue before there absent the agreement for LCM operation of Case 3a. RAN2 agreed “LMF is responsible for functionality management” for Case 1 in last meeting. Although we are discussing Case 3a here, the same logical can be reused for Case 3a very likely. If the LMF is responsible for functionality management for Case 3a, the LMF is aware of whether a AI/ML model is activated at gNB side, and also is aware of whether the reported measurement is generated from AI/ML or not (unless both AI/ML measurement and non AI/ML measurement can be reported simultaneous, but we do not think this scenario is reasonable). Therefore, we suggest this issue can be postponed until more discussion on LCM for Case 3a. |
| CMCC | Yes | To track model accuracy in the model monitoring function, in our view, the LMF could provide some assistance information to the NG-RAN Node, such as the precision level or accuracy level of the final positioning based on the intermediate measurement statistics inferred from the AI/ML assisted positioning model hosted at the NG-RAN Node. Thus, it is important for the LMF be aware of that those intermediate measurement statistics actually inferred from the AI/ML assisted positioning model hosted at the NG-RAN Node. In other words, AI/ML inferred measurements are differentiated from the measurements generated in a traditional manner. |
| Qualcomm | O.K. | Positioning typically requires a multitude of TRPs, and not all TRPs may be able to report e.g., AI/ML enhanced measurements. A LMF may e.g., perform different weighting of the measurement types when doing the position calculation. |
| CEWiT | Yes | It will help LMF for classifying the measurements which LMF may explore. |
| Nokia | No | We should not create interoperability issues regarding which node is responsible for performance monitoring. |
| Moderator’s view: - Majority prefer the introduction of the new flag. There is also requirement coming from the RAN1 conclusion. - Moderator propose to capture the concern from opposing companies as agreement that the flag does not mean LMF is not responsible for performance monitoring |  |  |

RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

P4-0: Performance monitoring is done in the RAN in case 3a.

P4-1: introduce the new flag “inferred measurement” to align with RAN1 conclusion

FFS: if LMF should request for AIML measurement

FFS on terminology: inferred vs predicted

## 4-3) Stage 2 TPs

Stage 2 TPs were provided by some companies for checking:

- R3-253592 (TP to TS 38.300) Stage 2 for case 3a (Nokia, CATT, Xiaomi, Ericsson, CMCC)

- R3-253524: (TP to TS 38.401) Support of AI/ML assisted positioning (ZTE Corporation, Xiaomi, CATT, Ericsson)

- R3-253145: (TP to BL CR for TS 38.455 and TS 38.305) Support of "Channel Measurement" for Case 3b and Stage 2 Support for Case 3a/3b (Qualcomm Incorporated) (Annex B)

- R3-253505 TP to NGAP BL CR for new procedure

Q5:  Companies provide comments on the three TPs

| Company | Comment |
| --- | --- |
| Ericsson | TP 1 OK TP 2 can rediscuss if F1AP needs to be impacted for case 3a. Alternative to leave F1 unspecified in case 3a case. TP 3: OK some wording revisions needed: “predicted” means predicted in future rather than AI/ML generated, prefer to replace the word with “inferred”, i.e. replace all instances of “predict”, “predicted” by “infer”, “inferred” |
| CATT | TP1 ok TP2 ok TP3, same texts may need to be refined, e.g. -As suggested by E/// -machine learning model AI/ML model -may have been trained by the is trained by the |
| Xiaomi | Agree with Ericsson and CATT |
| ZTE | Both TP1 and TP2 should be agreed together. 2 TPs all mention the deployment of AI/ML model. Explicit description should be captured to support AI/ML assisted positioning in case of split architecture and non-split architecture. |
| CMCC | TP 1 ok TP 2 OK TP3: similar comments as Ericsson and CATT |
| Qualcomm | TP 1: “The other principles for support of AI/ML for NG-RAN described in clause 16.20.2 apply for AI/ML assisted positioning with gNB-side AI/ML model”. Prefer to explicitly list these “other principles”, because some seem rather specific to AI/ML for NG-RAN and seem not applicable. TP2: Don’t see the benefit of specifying the location of the training and inference function if F1AP remains unspecified. Otherwise, training/inference should be at the gNB-DU. TP3 (proponent): The terminology proposed is based on the “positioning terminology” in 23.273. AI and ML are related but distinct concepts. Anyhow, the terminology will have to be aligned within 38.305 later when all pieces got implemented/merged. |
| CEWiT | TP1: Agreeing clause 16.20.2 complete for AIML for NR Air interface may not be accurate. We can remove that like for now. TP2: Okay |
| Nokia | TP1: OK TP2: Not OK. We don’t see the benefit to impact TS 38.401 without clear F1 impacts. TP3: It is OK in general but some wording updates are needed. Measurements in our view are not predictions but generated by AI/ML. A proposed update below: A gNB may host machine learning models to infer measurements from radio signals transmitted by a target UE. The machine learning model that is used for measurement inference by the gNB is trained by the gNB. Further alignment of the terminology may be needed e.g., machine learning models versus AI/ML Models so we can also add a Note that “Text can be further refined”. |
| Moderator’s conclusion: - TP1 is agreeable, no consensus on TP2 - TP3 is ok with some wording revisions as suggested by companies |  |

P5-1: Agree R3-253592 (TP to TS 38.300) Stage 2 for case 3a (Nokia, CATT, Xiaomi, Ericsson, CMCC)

P5-2: Agree revised version of R3-253145: Annex B

P5-3: agree revision of R3-253505 (NGAP TP) adding new procedure name.

P5-4: split architecture is supported in Case 3a

# 5	Case 3b discussion

No time was allocated to case 3b discussion during the online session, but a long offline discussion happened before the meeting. It is proposed to agree the following TPs capturing majority’s views on the encoding and continue on this basis.

- R3-253382: (TP for TS 38.473) Support of Sample-based measurement (case 3b) (Xiaomi, Ericsson, ZTE, CATT, Samsung, Nokia)

- R3-253461: (TP for TS 38.455) Support of Sample-based measurement (case 3b) (Ericsson, Xiaomi, CATT, Nokia)

Q6:  Companies provide comments on the three TPs

| Company | Comment |
| --- | --- |
| Ericsson | Yes, ok to capture majority’s view and take WA that measurement type B are new measurement codepoints and IEs with FFS where needed. |
| CATT | Agree with Moderator. |
| Xiaomi | Agree to use new measurement type, it’s cleaner, the term of the “additional path list” is not suitable for sample-based measurements. |
| ZTE | Agree 2 TPs. |
| NEC | Agree. |
| Qualcomm | The 3rd TP seems missing, which is R3-253145 and preferred (proponent). Do not agree that a new “Measurement Type” for “Enhanced RTOA” is needed. This can all be handled with the existing “RTOA” and possibly “gNB RxTx Time Difference”. |
| Nokia | Agree. |
| Moderator’s conclusion: - All companies except one support definition of new measurement type B as proposed in R3-253382/R3-253461 |  |

P6: agree revision R3-253382/R3-253461 with WA that measurement type B is new type of measurement in NRPPa and F1AP with FFS on name and encoding