3GPP TSG-RAN3 Meeting #128	R3-253814

# St. Julian’s, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1495 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE performance feedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Lenovo, Deutsche Telekom, BT, Ericsson, ZTE, CATT, FiberCop, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the context of the Rel-18 “AI/ML for NG-RAN” RAN3 introduced in TS 38.423 a set of metrics representing the UE performance that could be requested by the source gNB to be reported by the target gNB via the Data Collection Reporting (Initiation) procedures after an AI/ML-driven mobility decision. UE performance metrics, i.e., Average UE Throughput DL/UL, Average Packet Delay DL/UL and Average Packet Loss DL, are intended to be collected by the target gNB on a per-UE basis. However, there is no clear understanding on how a gNB could derive such per-UE metrics. In RAN3#126 it was agreed to send a LS to SA5 (R3-247775) to check whether the Average UE Throughput DL/UL measurements in TS 28.558 are already defined with per-UE granularity, and SA5 confirmed in R3-253023 (S5-252056) that Average UE Throughput DL/UL in TS 28.558 are per-UE measurements, originating from corresponding per-DRB measurements being aggregated to reflect a per-UE throughput. This CR clarifies in the semantics description of Average UE Throughput DL and Average UE Throughput UL IEs that the corresponding measurements refer to TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2, respectively. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous release of the specification because the changes only affect the AI/ML for NG-RAN function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The semantics description of Average UE Throughput DL and Average UE Throughput UL IEs within the UE Performance IE is updated so that the corresponding measurements refer to TS 28.558 clauses 6.3.1.4.1 and 6.3.1.4.2, respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It remains unclear how Average UE Throughput DL and Average UE Throughput UL are derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.2.3.179 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: Updates after discussion at RAN3#128 |  |  |  |  |  |  |  |  |

*** Start of Changes ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

*** skip unmodified parts ***

[57]	3GPP TS 23.527: "5G System; Restoration procedures".

[XX]	3GPP TS 28.558: “Management and orchestration; UE level measurements for 5G system”.

*** Next Change ***

#### 9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per-UE as specified in TS 28.558 [XX] clause 6.3.1.4.1. |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per-UE as specified in TS 28.558 [XX] clause 6.3.1.4.2. |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |

*** End of Changes ***