3GPP TSG-RAN WG3 Meeting #128					    					R3-253891

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	20.2

Source:	Huawei, ZTE, Lenovo, Nokia, Ericsson

Title:	(TP to TS38.413) Support of Case 3a

Document for:	Discussion and decision

# TP for the AI/ML BLCR to TS 38.413

<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>

## 8.10	NRPPa Transport Procedures

### 8.10.1	General

The purpose of the NRPPa Transport procedures is to carry NRPPa signalling (defined in TS 38.455 [19]) between the NG-RAN node and the LMF over the NG interface.

The Downlink UE Associated NRPPa Transport procedure and the Uplink UE Associated NRPPa Transport procedure use UE-associated signalling. The UE-associated signalling is used to support E-CID Location Information Transfer, Positioning Information Transfer, Measurement Preconfiguration Information Transfer, and Reporting of General Error Situations due to reception of an NRPPa message that utilized UE-associated signalling.

The Downlink Non UE Associated NRPPa Transport procedure and the Uplink Non UE Associated NRPPa Transport procedure use non-UE associated signalling. The non-UE associated signalling is used to support OTDOA Information Transfer, Assistance Information Transfer, TRP Information Transfer, Measurement Information Transfer, PRS Information Transfer, Area-specific Information Transfer, Positioning Data Collection Information Transfer and Reporting of General Error Situations due to reception of an NRPPa message that utilized non-UE associated signalling.

<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>