3GPP TSG-RAN WG3 Meeting #128	R3-253906

Malta, MT, 19th May– 23rd May 2025

Agenda Item:       10.2

Source:	ZTE Corporation, Nokia, Lenovo

Title: 		(TP for SON BLCR for 38.401) MRO for LTM

Document for:	Discussion and Approval

# 1	Introduction

The TP is capturing the following agreements in RAN3#128.

BFR shortly after successful LTM cell switch to the wrong beam(Case 1)

RACHless LTM cell switch failure due to wrong beam (Case 2)

Network-based solution for the LTM cell switch failure due to wrong beam using the same signaling flow as for Case1.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization.

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization.

Target DU needs to send the reconnect/re-established/recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

LTM cell switch failure due to outdated TA

RAN3 supports network-based solution for out dated TA.

Reusing the same signalling flow as case1 and case2.

# 2	TP for BL CR for TS38.401

<< Start of Change >>

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch. The target gNB-DU performs initial analysis and in case of the BFR caused by wrong beam selection at the source gNB-DU, it may send the recovery beam information to the source gNB-DU via the gNB-CU.

The target gNB-DU identifies that the UE successfully performs a RACH-based access with reconnecting/re-establishing/recovering to the same target cell but a beam different from the target beam for theLTM cell switch execution. The target gNB-DU may send the reconnected/re-established/recovery beam information to the source gNB-DU via the gNB-CU. The source gNB-DU performs analysis to detect RACH-less LTM failure caused by wrong beam selection.

The target gNB-DU identifies that the UE successfully performs a RACH-based access with reconnecting/re-establishing/recovering to the same target cell and same target beam with different TA value from the one which contained in CU-DU CELL SWITCH NOTIFICATION. The target gNB-DU sends the TA value used for successful RACH-based accessing to the source gNB-DU via the gNB-CU.

<< End of Change >>