3GPP TSG RAN WG3 Meeting #128	R3-253956

Malta, MT, 19 - 23 May, 2025

Agenda Item:	x.y

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion on Continuous MDT

Document for:	Approval

# Introduction

The following CB is discussed in this document:

CB: # AIRAN4_MDT

- Reply LS to SA5 with the single solution with minimum standard impact in R19 and clarify to SA5 on the questions raised if needed

(moderator - E///)

Summary of offline disc R3-253829

# For the Chairman’s Notes

Information from OAM to participating nodes in the NG-RAN allowing for the identification of a continuous MDT process are provided in the form of specific MDT TR(s)

RAN3 agrees that solutions for continuous MDT shall not have an impact on the 5GC.

Agree to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem

Agree to send an LS to SA5 capturing

- the agreements above

- the assumption from RAN3 that security aspects will be checked by SA5

- provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”

- elaborate on the details of how to avoid overlapping of the identifiers used for continuous MDT reports correlation

- Action to SA5 is to check feasibility of the principles agreed by RAN3, specify a solution that reflects such principles (if feasible) and provide feedback to RAN3

# Discussion

The group has been tasked to down select Continuous MDT solutions presented so far and to converge ideally to a single solution to be described in an LS to SA5. The LS shall ask SA5 to check on feasibility of the solution and, if feasible, to specify the solution.

To achieve this, we take the LS in from SA5 in R3-252034 as a starting point to derive filtering criteria.

The LS quotes the following:

- For Measurement Continuity Problem, regarding RAN3 agreements: - RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.” Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements. - RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.” Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”. - RAN3: “- A solution should minimise impacts on the current Management Based MDT framework.” Answer from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework. - For Trace Correlation Problem, - RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.” Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

At the same time we provide a summary list of solutions presented so far in RAN3 for continuous MDT:

- Solution 1: Based on RAN-OAM interaction.

  - A TraceJob is sent to all participating RAN nodes at the time of Continuous management-based MDT initiation. The TraceJob includes a specific Trace Reference identifier (i.e. TR) to indicate this is a Continuous management-based MDT procedure. The same TR is used in all TraceJob signalled to the participating RAN nodes. A dedicated TRSR range is assigned to each participating RAN node, which is used by RAN node during Trace Recording Session Reference (TRSR) assignment.

  - When a UE is selected for the continuous management-based MDT, and a Trace Recording session is initiated, the gNB allocates a TRSR from the received range. The same TRSR will be used for the whole duration of the Continuous MDT trace of a UE for correlation purpose. Namely, the TRSR is kept for the UE even if the UE moves to a different participating node.

  - If the selected UE is handed over to a participating RAN node, the target RAN node recognize the UE as a Continuous management-based MDT UE based on its TR. Therefore, the UE will be reselected in the target RAN node for the Continuous management-based MDT.

  - If the UE has been configured with Immediate Continuous management-based MDT and the UE moves to a different RAN node, the source RAN node may signal to the target RAN node, e.g. as part of the Handover Request message, the TR and TRSR assigned to the UE. If the target RAN node is a participating RAN node, it understands that the UE was selected for Continuous management-based MDT (from the received TR). Hence the target RAN would select the UE again for further Continuous management-based MDT measurements. The target RAN node assigns to the UE the TRSR received from the source RAN node.

  - If the UE has been configured with Logged Continuous management-based MDT, and the UE moves to RRC_Idle mode or RRC_Inactive state, the UE will collect measurements corresponding to the measurement configuration the UE has received for the logged continuous management-based MDT process. Such measurements will be stored by the UE together with the assigned TR, and TRSR.

  - Once the UE moves to RRC_Connected mode, the UE will report to the serving RAN node the logs of measurements collected while in RRC_Idle or RRC_Inactive state. If the serving RAN node is a participating RAN node, it will therefore identify that the UE was selected for Continuous management-based MDT by looking at the reported TR. The serving RAN would therefore further select the UE for Continuous management-based MDT, i.e., the UE will be selected for further continuous management-based MDT measurements. The serving RAN node will assign to the UE the same TRSR contained in the reported MDT logs.

  - Trace Correlation is achieved by the assigned TR/TRSR which can identify that the trace records belong to the same selected UE. The same TRSR will be used for the whole duration of the Continuous management-based MDT trace of a UE.

- Solution 1B: Based on List of TRs/TRSR

  - Similar as solution 1 with the following differences

  - each TraceJob signalled to a participating RAN node will have a dedicated TR, together with a TR list. The TR list contains all dedicated TRs being used in the same continuous MDT Job in each participated gNB. This TR list is used by the RAN node to deduce that a UE establishing connection or handing over from a neighbour node needs to be configured with continuous MDT.

  - Each selected UE has a TRSR assigned. The TR/TRSR combination is unchanged for a selected UE during the lifetime of this continues MDT.

  - At handover, the target gNB recognizes the UE is a continues MDT UE from its TR. The target gNB receives from the source gNB the TR+TRSR for the UE and it reassigns to the UE the same TR+TRSR.

  - When the UE moves from RRC_Idle/Inactive to RRC_Connected and it reports Logged MDT logs, the new serving gNB detects the TR+TRSR for the UE and re-assigns the same TR+TRSR for the immediate continuous MDT process for the same UE.

Involved domains: RAN and OAM

- Solution 2: Based on OAM-RAN-AMF interaction.

  - OAM configures Management Based MDT at a gNB;

  - At connected-to-idle transition the MDT configuration is stored by the serving gNB in 5GC/AMF;

  - Upon idle-to-connected transition the 5GC/AMF indicates to the (new) serving gNB the MDT configuration of the UE along with an identifier that the UE needs to be selected again for management-based immediate MDT;

  - During handover, the source gNB sends the Immediate MDT configuration to the target gNB  as part of the handover preparation.

Involved domains: RAN, OAM and 5GC/AMF

- -	Solution 3: Based on OAM-RAN-AMF interaction.

  - OAM configures Management Based MDT with a new Job Type “Immediate MDT and Logged MDT” and assigns a unique TR to this MDT session (valid for all UEs in the area of gNBs where this configuration is provided);

  - The gNB selects a UE and assigns a TRSR which is UE-specific;

  - At connected-to-idle transition the MDT configuration (at least Job Type and TRSR) is sent by the serving gNB to the 5GC/AMF which stores this information; when the UE resumes the (new) serving gNB receives from 5GC/AMF the stored MDT configuration; the presence of the MDT configuration is an indication for the (new) serving gNB that the UE needs to be reselected for Continuous MDT

  - At handover, the source gNB sends the MDT configuration (at least Job Type and TRSR) to the target gNB; the presence of the MDT configuration is an indication for the target gNB that the UE needs to be reselected for Continuous MDT.

Involved domains: RAN, OAM and 5GC/AMF

- Solution 4: Based on RAN-OAM interaction (Trace Correlation without RAN3 Impact).

  - OAM configures gNBs in an area with same immediate MDT configuration (TR1) and logged MDT configuration (TR2);

  - A gNB chooses UE and configures the UE for Immediate MDT and Logged MDT separately (with separate TRSRs, i.e. TRSR1 and TRSR2);

  - At connected-to-idle transition the serving gNB sends the Immediate MDT report (TR1/TRSR1) to TCE by also including the TR2/TRSR2 assigned for the Logged MDT;

  - At idle-to-connected transition the (new) serving gNB sends the Logged MDT report (TR2/TRSR2) to TCE by also including the TR1/TRSR3 assigned for the Immediate MDT.

Involved domains: RAN and OAM

Discussion on filtering criteria:

Excerpt from the SA5 LS:

- RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.”

Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

The criteria in the reply to question 1) is fulfilled by all solutions so far presented.

Information from OAM to participating nodes in the NG-RAN allowing for the identification of a continuous MDT process are provided in the form of specific MDT TR(s)

Excerpt from the SA5 LS:

- RAN3: “- A solution should minimise impacts on the current Management Based MDT framework.”

Answer from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework.

In order to fulfil the criteria from the answer above, it is suggested that solutions with an impact on the CN are filtered out.

Additionally, solutions with large impacts on the MDT framework should be filtered out, e.g. solutions based o the definition of a new Trace Job type

RAN3 agrees that solutions for continuous MDT shall not have an impact on the 5GC.

Excerpt from the SA5 LS:

- For Trace Correlation Problem,

  - RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.”

Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

To fulfil the criteria above, it is suggested to filter out solutions that impact the structure of MDT identifiers, e.g. TR/TRSR

Agree to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem

# Conclusion, Recommendations [if needed]

If needed

# References