3GPP TSG-RAN WG3 Meeting #128	R3-253999

# Malta, MT, 19th - 23th May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1413 | rev | 9 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.423 for SON) Addition of SON enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Samsung, Nokia, Lenovo, Jio |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-06-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of SON enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Time SCG Failure IE in SCG FAILURE INFORMATION REPORT message. - Correct the reference of Time SCG Failure IE in SCG FAILURE INFORMATION REPORT message. - Add a new handover type “Too Late CHO with candidate SCG” in HANDOVER REPORT message. - Add a new procedure “SCG Failure Indication”. - Add UE History Information IE in SN RELEASE REQUEST message and SN RELEASE CONFIRM message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SON features for Rel-19 are missing in XnAP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.6.2, 8.3.7.2, 8.3.17.2, 8.4.8.2, 8.4.x (new), 9.1.2.14, 9.1.2.18, 9.1.2.29, 9.1.3.17, 9.1.3.xx (new), 9.3.3, 9.3.4, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.0: TP agreed at RAN3 #125bis: R3-245821 Rev 1: Re-submission in RAN3#126 Rev 2: Re-submission in RAN3#127 Rev 3: TPs agreed at RAN3#127: R3-250850, R3-250853 Rev 4: Re-submission in RAN3#127bis Rev 5: Addition of the agreed correction in R3-250850 Rev 6: TP agreed at RAN3#127bis: R3-252513; Addition of ASN.1 on “SCG Failure Indication” procedure Rev 7: Re-submission in RAN3#128 Rev 8: Addition of missing part in 8.4.8 and correction of the meeting information. Rev 9: TP agreed at RAN3#128: R3-253885 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

8.3.6.2	Successful Operation

Figure 8.3.6.2-1: M-NG-RAN node initiated S-NG-RAN node Release, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE RELEASE REQUEST message. Upon reception of the S-NODE RELEASE REQUEST message the S-NG-RAN node shall stop providing user data to the UE.

>>>skip unchanged part<<<

If the S-NODE RELEASE REQUEST message contains a PDU session resource to be released which is configured with the SCG bearer option within the PDU Session Resources To Be Released List IE, the S-NG-RAN node shall include the RLC Mode IE within the DRBs To Be Released List IE in the S-NODE RELEASE REQUEST ACKNOWLEDGE message. The RLC Mode IE indicates the RLC mode used in the S-NG-RAN node for the DRB.

If the S-NODE RELEASE REQUEST message contains the UE History Information IE, the S-NG-RAN node may use it to optimise the S-CPAC configuration.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3.7.2	Successful Operation

Figure 8.3.7.2-1: S-NG-RAN node initiated S-NG-RAN node Release, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE RELEASE REQUIRED message to the M-NG-RAN node.

>>>skip unchanged part<<<

If the S-NODE RELEASE REQUIRED message includes the SCG UE History Information IE, the M-NG-RAN node shall, if supported, use the information to update UE History Information with PSCell history.

If the S-NODE RELEASE REQUIRED message contains the User Plane Error Indicator IE within the PDU Sessions List To be Released - UPError IE set to "GTP-U Error Indication Received", the M-NG-RAN node shall, if supported, consider that the PDU session is released due to GTP-U Error Indication received over the NG-U tunnel, and inform the SMF as specified in 3GPP TS 23.527 [57].

If the S-NODE RELEASE CONFIRM message contains the UE History Information IE, the S-NG-RAN node may use it to optimise the S-CPAC configuration.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3.17.2	Successful Operation

Figure 8.3.17.2-1: SCG Failure Information Report, successful operation

The M-NG-RAN node initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the S-NG-RAN node. Upon receiving the message, the S-NG-RAN node shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the SN Mobility Information IE, if the SN Mobility Information IE was sent for the PSCell change procedure from the S-NG-RAN node;

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the S-NG-RAN node.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 37.340 [8].

If the SCG FAILURE INFORMATION REPORT message includes the CPAC Configuration IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 37.340 [8].

If the SCG FAILURE INFORMATION REPORT message includes the Time SCG Failure IE, the S-NG-RAN node shall, if supported, store the information and act as specified in TS 37.340 [8].

If received, the S-NG-RAN node uses the above information for SCG failure reason detection and optimisation.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.4.8	Handover Report

8.4.8.1	General

The purpose of the Handover Report procedure is to transfer mobility related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

8.4.8.2	Successful Operation

Figure 8.4.8.2-1: Handover Report, successful operation

NG-RAN node1 initiates the procedure by sending the HANDOVER REPORT message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a mobility-related problem was detected.

If the Handover Report Type IE is set to "HO too early" or "HO to wrong cell", then NG-RAN node1 indicates to NG-RAN node2 that, following a successful handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, a radio link failure occurred and the UE attempted RRC Re-establishment or re-connected either at the original cell of NG-RAN node2 (Handover Too Early), or at another cell (Handover to Wrong Cell). The detection of Handover Too Early and Handover to Wrong Cell events is made according to TS 38.300 [9].

If the Handover Report Type IE is set to "Too Late CHO with candidate SCG", then NG-RAN node2 shall deduce that a too late CHO with candidate SCG from a cell of NG-RAN node1 to a suitable cell of NG-RAN node2 occurred. The detection of Too Late CHO with candidate SCG event is made according to TS 38.300 [9].

The HANDOVER REPORT message may include:

-	the Mobility Information IE, if the Mobility Information IE was sent for this handover from NG-RAN node2 (in case the NG-RAN node2 provided it more than once, the most recent Mobility Information IE is included in the HANDOVER REPORT message);

-	the Source cell C-RNTI IE.

-	the CHO Configuration IE, if the CHO Configuration IE was sent for this handover from NG-RAN node2.

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

If the Handover Report Type IE is set to "Inter-system ping-pong", then NG-RAN node2 shall deduce that a completed handover from a cell of NG-RAN node2 to a cell in another system might have resulted in an inter-system ping-pong and the UE was successfully handed over to a cell of NG-RAN node1 (indicated with Target cell CGI IE).

If the Target cell C-RNTI IE and the Time Since Failure IE are received, NG-RAN node2 uses the information as specified in TS 38.300 [9].

Interaction with the Failure Indication procedure:

If NG-RAN node1 receives a UE RLF Report from an NG-RAN node via the FAILURE INDICATION message, as described in TS 38.300 [9], NG-RAN node1 may also include it in the UE RLF Report Container IE included in the HANDOVER REPORT message.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.4.x	SCG Failure Indication

8.4.x.1	General

The purpose of the SCG Failure Indication procedure is to transfer SCG failure related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SCG Failure Indication, successful operation

NG-RAN node1 initiates the procedure by sending the SCG FAILURE INDICATION message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a SCG failure related problem was detected.

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

8.4.x.3	Unsuccessful Operation

Not applicable.

8.4.x.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.14	S-NODE RELEASE REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the release of resources.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session Resources To Be Released List | O |  | PDU Session List with Cause 9.2.1.26 |  | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | Indicates that the target M-NG-RAN node reconfigured the listed DRBs as MN-terminated bearers. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.18	S-NODE RELEASE CONFIRM

This message is sent by the M-NG-RAN node to confirm the release of all resources for a specific UE at the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session Resources Released |  | 0..1 |  |  | YES | ignore |
| >PDU sessions released List – SN terminated | O |  | PDU Session List with data forwarding info from the target node 9.2.1.25 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.103 |  | YES | ignore |
| Time SCG Failure | O |  | INTEGER (0..1023) | Corresponds to information provided in the timeSCGFailure contained in the SCGFailureInformationNR message as defined in TS36.331 [14] | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …, Too Late CHO with candidate SCG) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Target cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to “Inter-system ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cell. If the Handover Report Type is set to “Too Late CHO with candidate SCG”, it is the suitable cell in the NG-RAN node 2. | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | Global Cell Identity 9.2.2.73 | CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the failure | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message or in the SN STATUS TRANSFER message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | UE RLF Report 9.2.2.59 | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| CHO Configuration | O |  | 9.2.2.76 |  | YES | ignore |
| Target cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the target NG-RAN node. | YES | ignore |
| Time Since Failure | O |  | INTEGER (0.. 172800, ...) | Corresponds to the TimeSinceFailure IE received from the UE in RLF Report as defined in TS 36.331 [14]. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.3.xx	SCG Failure Indication

This message is sent by NG-RAN node1 to NG-RAN node2 to report a SCG failure event that occurred while CHO with candidate SCG was configured.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the candidate target NG-RAN node |  |  |
| SCG Failure Report Container | M |  | OCTET STRING | Includes the SCGFailureInformation message as defined in TS 38.331 [10] | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

XnAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM XnAP-CommonDataTypes

HandoverRequest,

HandoverRequestAcknowledge,

HandoverPreparationFailure,

>>>skip unchanged part<<<

FailureIndication,

HandoverReport,

SCGFailureIndication,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

FROM XnAP-PDU-Contents

id-handoverPreparation,

>>>skip unchanged part<<<

id-earlyStatusTransfer,

id-failureIndication,

id-handoverReport,

id-scgFailureIndication,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

FROM XnAP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

XNAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

>>>skip unchanged part<<<

XNAP-ELEMENTARY-PROCEDURES-CLASS-2 XNAP-ELEMENTARY-PROCEDURE ::= {

sNStatusTransfer						|

handoverCancel							|

rANPaging								|

xnUAddressIndication					|

uEContextRelease						|

sNGRANnodeReconfigurationCompletion		|

sNGRANnodeCounterCheck					|

rRCTransfer								|

errorIndication							|

privateMessage							|

notificationControl						|

activityNotification					|

secondaryRATDataUsageReport				|

deactivateTrace							|

traceStart								|

handoverSuccess							|

conditionalHandoverCancel				|

earlyStatusTransfer						|

failureIndication						|

handoverReport							|

scgFailureIndication					|

resourceStatusReporting					|

accessAndMobilityIndication				|

cellTrafficTrace						|

rANMulticastGroupPaging					|

scgFailureInformationReport				|

scgFailureTransfer						|

f1CTrafficTransfer						|

retrieveUEContextConfirm				|

cPCCancel								|

rachIndication							|

dataCollectionReporting,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

handoverPreparation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-handoverPreparation

CRITICALITY				reject

}

>>>skip unchanged part<<<

failureIndication XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		FailureIndication

PROCEDURE CODE			id-failureIndication

CRITICALITY				ignore

}

handoverReport XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverReport

PROCEDURE CODE			id-handoverReport

CRITICALITY				ignore

}

scgFailureIndication XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SCGFailureIndication

PROCEDURE CODE			id-scgFailureIndication

CRITICALITY				ignore

}

resourceStatusReportingInitiation	XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE				ResourceStatusRequest

SUCCESSFUL OUTCOME				ResourceStatusResponse

UNSUCCESSFUL OUTCOME			ResourceStatusFailure

PROCEDURE CODE					id-resourceStatusReportingInitiation

CRITICALITY						reject

}

resourceStatusReporting XNAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

>>>skip unchanged part<<<

UserPlaneFailureIndication,

SRSPositioningConfigOrActivationRequest,

NRPPaPositioningInformation,

TimeSCG-Failure

>>>skip unchanged part<<<

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-TimeSCG-Failure,

id-TargetNG-RANnodeUEXnAPID,

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- HANDOVER REPORT

--

-- **************************************************************

HandoverReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ HandoverReport-IEs}},

...

}

HandoverReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-HandoverReportType			CRITICALITY ignore		TYPE HandoverReportType				PRESENCE mandatory}|

{ ID id-HandoverCause				CRITICALITY ignore		TYPE Cause							PRESENCE mandatory}|

{ ID id-SourceCellCGI				CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE mandatory }|

{ ID id-TargetCellCGI				CRITICALITY ignore		TYPE GlobalNG-RANCell-ID			PRESENCE mandatory }|

{ ID id-ReEstablishmentCellCGI		CRITICALITY ignore		TYPE GlobalCell-ID					PRESENCE conditional }|

-- This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell"

{ ID id-TargetCellinEUTRAN			CRITICALITY ignore		TYPE TargetCellinEUTRAN				PRESENCE conditional }|

-- This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong"

{ ID id-SourceCellCRNTI				CRITICALITY ignore		TYPE C-RNTI							PRESENCE optional }|

{ ID id-MobilityInformation			CRITICALITY ignore		TYPE MobilityInformation			PRESENCE optional }|

{ ID id-UERLFReportContainer		CRITICALITY ignore		TYPE UERLFReportContainer			PRESENCE optional }|

{ ID id-CHOConfiguration			CRITICALITY ignore		TYPE CHOConfiguration				PRESENCE optional}|

{ ID id-TargetCellCRNTI				CRITICALITY ignore		TYPE C-RNTI							PRESENCE optional}|

{ ID id-TimeSinceFailure			CRITICALITY ignore		TYPE TimeSinceFailure				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- SCG FAILURE INDICATION

--

-- **************************************************************

SCGFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SCGFailureIndication-IEs}},

...

}

SCGFailureIndication-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-TargetNG-RANnodeUEXnAPID	CRITICALITY				TYPE NG-RANnodeUEXnAPID				PRESENCE mandatory}|

{ ID id-SCGFailureReportContainer   CRITICALITY ignore		TYPE SCGFailureReportContainer		PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- SCG FAILURE INFORMATION REPORT

--

-- **************************************************************

ScgFailureInformationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureInformationReport-IEs}},

...

}

ScgFailureInformationReport-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-SourcePSCellCGI							CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-FailedPSCellCGI							CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-SCGFailureReportContainer				CRITICALITY ignore		TYPE SCGFailureReportContainer					PRESENCE mandatory}|

{ ID id-SNMobilityInformation				CRITICALITY ignore		TYPE SNMobilityInformation							PRESENCE optional }|

{ ID id-CPACConfiguration					CRITICALITY ignore		TYPE CPACConfiguration									PRESENCE optional }|

{ ID id-TimeSCG-Failure						CRITICALITY ignore		TYPE TimeSCG-Failure									PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- S-NODE RELEASE REQUEST

--

-- **************************************************************

SNodeReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseRequest-IEs}},

...

}

SNodeReleaseRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE optional }|

{ ID id-Cause									CRITICALITY ignore		TYPE Cause											PRESENCE mandatory}|

{ ID id-PDUSessionToBeReleased-RelReq			CRITICALITY ignore		TYPE PDUSession-List-withCause					PRESENCE optional }|

{ ID id-UEContextKeptIndicator					CRITICALITY ignore		TYPE UEContextKeptIndicator						PRESENCE optional }|

{ ID id-MN-to-SN-Container						CRITICALITY ignore		TYPE OCTET STRING									PRESENCE optional }|

{ ID id-DRBs-transferred-to-MN					CRITICALITY ignore		TYPE DRB-List										PRESENCE optional }|

{ ID id-UEHistoryInformation					CRITICALITY ignore		TYPE UEHistoryInformation						PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- S-NODE RELEASE CONFIRM

--

-- **************************************************************

SNodeReleaseConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeReleaseConfirm-IEs}},

...

}

SNodeReleaseConfirm-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-PDUSessionReleasedList-RelConf			CRITICALITY ignore		TYPE PDUSessionReleasedList-RelConf			PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-UEHistoryInformation					CRITICALITY ignore		TYPE UEHistoryInformation						PRESENCE optional },

...

}

PDUSessionReleasedList-RelConf ::= SEQUENCE {

pduSessionsReleasedList-SNterminated		PDUSession-List-withDataForwardingFromTarget					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionReleasedList-RelConf-ExtIEs} }	OPTIONAL,

...

}

PDUSessionReleasedList-RelConf-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.5	Information Element definitions

>>>skip unchanged part<<<

HandoverReportType ::= ENUMERATED {

hoTooEarly,

hoToWrongCell,

intersystempingpong,

...,

tooLateCHOwithcandidateSCG

}

>>>skip unchanged part<<<

Threshold-RSRQ ::= INTEGER(0..127)

Threshold-RSRP ::= INTEGER(0..127)

Threshold-SINR ::= INTEGER(0..127)

TimeSCG-Failure ::= INTEGER (0..1023)

TimeSinceFailure ::= INTEGER (0..172800, ...)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication											ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

id-cellTrafficTrace												ProcedureCode ::= 38

id-RANMulticastGroupPaging										ProcedureCode ::= 39

id-scgFailureInformationReport									ProcedureCode ::= 40

id-ProcedureCode41-NotToBeUsed									ProcedureCode ::= 41

id-scgFailureTransfer											ProcedureCode ::= 42

id-f1CTrafficTransfer											ProcedureCode ::= 43

id-iABTransportMigrationManagement								ProcedureCode ::= 44

id-iABTransportMigrationModification							ProcedureCode ::= 45

id-iABResourceCoordination										ProcedureCode ::= 46

id-retrieveUEContextConfirm										ProcedureCode ::= 47

id-cPCCancel													ProcedureCode ::= 48

id-partialUEContextTransfer										ProcedureCode ::= 49

id-rachIndication												ProcedureCode ::= 50

id-dataCollectionReportingInitiation							ProcedureCode ::= 51

id-dataCollectionReporting										ProcedureCode ::= 52

id-scgFailureIndication											ProcedureCode ::= xx

>>>skip unchanged part<<<

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-TimeSCG-Failure																					ProtocolIE-ID ::= aaa

id-TargetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= bbb

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>