3GPP TSG-RAN WG3 Meeting #129bis			R3-256832

Prague, Czech Republic, October 13-17, 2025

Source:	NTT DOCOMO, INC.

Title:	Views on 6G RAN internal functionality split and interfaces

Agenda item:	10.4

Document for:	Discussion

- Introduction

Revised SID Study on 6G Radio has been approved in RAN#109 [1]. This contribution provides views on 6G RAN internal functionality split and interfaces.

- Discussion

  - Whether to support CU-DU split in 6G?

In today’s 5G RAN architecture, the network is split between a Centralized Unit (CU), which handles higher-layer functions such as PDCP, SDAP, RRC, and NG, and a Distributed Unit (DU), which manages lower-layer functions including RLC, MAC, and PHY, as illustrated below.

One of the key advantages of the CU-DU split architecture is its flexible deployment. The CU can be located in centralized or cloud environments, while the DU can be positioned closer to the network edge. A single CU can control multiple DUs, significantly reducing hardware costs—for example, by minimizing the number of required baseband units. Additionally, the CU can be virtualized on cloud or NFV platforms, and the DU can be made programmable at edge sites.

Another major benefit is latency optimization. Since the DU is located near the UE, it can support low-latency operations such as HARQ and MAC scheduling. With this architecture, DU can be easily introduced for small enterprise business use case or Femto cell use case within MNO’s network. Meanwhile, the CU, with its broader network visibility, can manage mobility and QoS decisions without being constrained by latency.

Furthermore, the CU-DU split allows for independent upgrades of the CU—such as protocol enhancements or AI/ML optimizations—without requiring changes to the DU. Centralized OAM via the CU also reduces the need for on-site maintenance at cell towers.

Lastly, and perhaps most importantly, the standardized F1 interface between the CU and DU enables multi-vendor interoperability. Operators can deploy DU hardware from one vendor and CU software from another, fostering innovation and reducing vendor lock-in.

With all the benefits stated above, we think 6G RAN should continue to support CU-DU split architecture.

Figure 6.1-1: Overall architecture [3]

Obsevation1: CU-DU split architecture has following benefits.

- CU-DU split architecture allows flexible deployment. CU can be virtualized in cloud/NFV platform. DU can also be programable at edge sites, which reduce the cost of hardware equipment.

- CU-DU split architecture allows that DU can provide low latency HARQ/MAC scheduling. CU, with broader visibility, it can handle mobility/QoS decision without being latency-constrained.

- CU-DU split architecture allows that DU can be easily introduced for small enterprise business use case or Femto cell use case within MNO’s network.

- CU-DU split architecture allows for independent upgrades of the CU—such as protocol enhancements or AI/ML optimizations—without requiring changes to the DU.

- CU-DU split architecture enables multi-vendor interoperability. Operators can deploy DU hardware from one vendor and CU software from another, fostering innovation and reducing vendor lock-in.

Proposal1: CU-DU split architecture should be supported in 6G.

# 3	Conclusion

In this contribution, the follows are proposed.

Obsevation1: CU-DU split architecture has following benefits.

- CU-DU split architecture allows flexible deployment. CU can be virtualized in cloud/NFV platform. DU can also be programable at edge sites, which reduce the cost of hardware equipment.

- CU-DU split architecture allows that DU can provide low latency HARQ/MAC scheduling. CU, with broader visibility, it can handle mobility/QoS decision without being latency-constrained.

- CU-DU split architecture allows that DU can be easily introduced for small enterprise business use case or Femto cell use case within MNO’s network.

- CU-DU split architecture allows for independent upgrades of the CU—such as protocol enhancements or AI/ML optimizations—without requiring changes to the DU.

- CU-DU split architecture enables multi-vendor interoperability. Operators can deploy DU hardware from one vendor and CU software from another, fostering innovation and reducing vendor lock-in.

Proposal1: CU-DU split architecture should be supported in 6G.

# 4	References

[1]	RP-252912  “Revised SID: Study on 6G Radio”  NTT DOCOMO, CMCC, AT&T, Vodafone

[2]	RP-252870  “pCR for 38.914 on Requirements for architecture and migration for 6G”  NTT DOCOMO

[3] 	TS 38.401