3GPP TSG-RAN WG3 Meeting #129-bis	R3-257235

Prague, Czech Republic, 13th ~17th Oct, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1641 | Rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for LTM failure without RLF-report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Huawei, Ericsson, Nokia, Google, CATT, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There was an agreement to enable failure indication without RLF report, and a basic solution in stage3 was introduced, but this solution needs to be refined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revised the information carried for the failure reporting without RLF report including replacing the source C-RNTI with gNB-DU UE F1AP ID, and updating LTM failure type from optional to mandatory. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No working solution for failure reporting without RLF report. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.10.1, 9.4.4, 9.4.5, and 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU. This IE is included in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| Failure Reporting without RLF report |  | 0..1 |  |  |  |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| >Recovery or Re-establishment cell ID | M |  | NR CGI 9.3.1.12 | NR CGI for the re-establishment or recovery cell. | - |  |
| >Failure Type | M |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| MRO for LTM Information |  | 0..1 |  |  | YES | ignore |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| >BFR SSB Index | O |  | INTEGER (0..63) | SSB Index of the recovery beam used at successful Beam Failure Recovery. | - |  |
| >Target SSB Index after Cell Switch Failure | O |  | INTEGER (0..63) | SSB Index of the re-established or recovery beam after LTM Cell Switch Failure. | - |  |
| >TA Information | O |  | INTEGER (0..4095) | Indicates the TA value, as defined in TS 38.213 [31], used at successful Random Access during LTM recovery or re-establishment after a Cell Switch failure in same cell. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

<< The Next Changes >>

9.4.4	PDU Definitions

<SKIP>

LPWUSPSAssistanceInfo,

FurtherExtendedUEIdentityIndexValue,

CLI-MeasurementResult-List,

SRS-Resource-Indication,

FailureReportingWithoutRLFReport,

MROForLTM-Information,

LastVisitedLTMCells,

OnDemand-SIB1-Cell,

LTMSecurityInformation,

LTMInformationSCGAdd,

<SKIP>

id-LPWUSPSAssistanceInfo,

id-FurtherExtendedUEIdentityIndexValue,

id-CLI-MeasurementResult-List,

id-SRS-Resource-Indication,

id-FailureReportingWithoutRLFReport,

id-MROForLTM-Information,

id-LastVisitedLTMCells,

id-OnDemand-SIB1-Cell,

id-LTMSecurityInformation,

<SKIP>

-- **************************************************************

--

#### --  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RAReportList				CRITICALITY ignore	TYPE RAReportList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional }|

{ ID id-SuccessfulPSCellChangeReportInformationList	CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportInformationList		PRESENCE optional }|

{ ID id-FailureReportingWithoutRLFReport				CRITICALITY ignore 	TYPE FailureReportingWithoutRLFReport		PRESENCE optional }|

{ ID id-MROForLTM-Information					CRITICALITY ignore 	TYPE MROForLTM-Information			PRESENCE optional },

...

}

<< The Next Changes >>

9.4.5	Information Element Definitions

<SKIP>

FailureReportingWithoutRLFReport ::= SEQUENCE {

gNB-DU-UE-F1AP-ID					GNB-DU-UE-F1AP-ID,

reestRecoveryCGI			NRCGI,

failureType		RLFReportFailureType		,

iE-Extensions			ProtocolExtensionContainer { { FailureReportingWithoutRLFReport-ExtIEs } }		OPTIONAL,

...

}

FailureReportingWithoutRLFReport-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

<< The Next Changes >>

### 9.4.7	Constant Definitions

<SKIP>

id-NZP-CSI-RS-Resources-Config						ProtocolIE-ID ::= 884

id-SRS-Resource-Indication							ProtocolIE-ID ::= 885

id-SRS-Resource-Configuration						ProtocolIE-ID ::= 886

id-rLFReportFailureType								ProtocolIE-ID ::= 887

id-FailureReportingWithoutRLFReport							ProtocolIE-ID ::= 888

id-MROForLTM-Information								ProtocolIE-ID ::= 889

id-LastVisitedLTMCells								ProtocolIE-ID ::= 890

id-OnDemandSIB1										ProtocolIE-ID ::= 891

id-PagingAdaptationIndication						ProtocolIE-ID ::= 892

id-OnDemand-SIB1-Cell								ProtocolIE-ID ::= 893

<< End of Changes >>