3GPP TSG-RAN WG3 Meeting #129-bis	R3-257236

Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	12.2.1

Source:	ZTE Corporation (moderator)

Title:	Summary of Offline Discussion for CB: # 21_AIMLMultiHop

Document for:	Discussion

# 1	Introduction

CB: # 21_AIMLMultiHop

-  TP to capture the agreement in appropriate way

-  transfer the multi-hop UE trajectory prediction in UE associated message?

-  discuss the above open issues

(ZTE - moderator)

Summary of offline discussion in R3-257236

# 2	For the Chair Notes

Editor’s Note: For Rel-20 study/work items, please consider that when agreements/FFSes are captured in a TP, additional inclusion in the Chair Notes may be unnecessary (particularly for stage 3 details).

Propose the following:

The delivery of the measured&predicted cell-based multi-hop UE trajectory should not impact the core network.

The request and reporting to support measured multi-hop UE trajectory between gNBs should utilize the Data Collection Reporting Initiation procedure and Data Collection Reporting procedure.

To be discussed online:

WA: The cell-based multiple-hop UE trajectory prediction is transferred to the target NG-RAN node via the Handover Preparation procedure.

Common understanding:

- The cell-based multiple-hop UE trajectory can be updated to remove the past visited cell(s).

- When UE moves to unpredicted target cell, the serving gNB can discard the cell-based multiple-hop UE trajectory prediction

# 3	Discussion

Following the online discussion, the discussion how to transfer the predicted multiple-hop UE trajectory, via Handover Request message or Data Collection Request message.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Based on what we captured in the stage2 spec for single hop UE trajectory:

From the moderator’s perspective, the Rel-19 study phase should focus on extending the definition of cell-based UE trajectory prediction, while maintaining consistency with the principles agreed in Rel-18. Moreover, according to the protocol design, the predicted multi-hop UE trajectory is a per-UE measurement. Therefore, if this measurement needs to be transferred over the Xn interface, it must be carried in a UE-associated message, rather than a non-UE-associated message.

Proposal 1: The delivery of predicted multi-hop UE trajectory can continue to be carried in the Handover Request message to the next target gNB.

WA: The cell-based multiple-hop UE trajectory prediction is transferred to the target NG-RAN node via the Handover Preparation procedure.

The cell-based multiple-hop UE trajectory can be updated to remove the past vistied cell(s).

When UE moves to unpredicted target cell, the serving gNB can discard the cell-based multiple-hop UE trajectory prediction

Nokia: What’s the benefits of multiple-hop UE trajectory?

Huawei, ZTE: For subsequent mobility decisions.

For the measured multi-hop UE trajectory across gNBs, the collected measurement results should be forwarded to the initial gNB. This allows the initial gNB to evaluate and analyze the end-to-end performance of the trajectory prediction and mobility optimization.

Following is the solutions captured in the online meeting:

- Option 1: Parallel transmission from each hop gNB to the initial source gNB (i.e., Node2 to Node1, Node3 to Node1)

- Option 2: Hop-by-hop transmission (i.e., Node3 to Node2, Node2 to Node1)

- Option 3: Final-hop to initial-hop transmission (i.e., Node2 to Node3, Node3 to Node1)

- Option 4: Measured UE trajectory transfer via AMF (i.e. Node3 to AMF, AMF to Node1).

Before discussing the options above, the following principle should be agreed first:

- The delivery of the measured multi-hop UE trajectory should not involve impact of the core network.

- The delivery of the measured multi-hop UE trajectory should minimize the specification impacts.

- The request and report to support measured multiple-hop UE trajectory should utilize the Data Collection Initiation procedure and Data Collection Update procedure.

Proposal 2: Agree the above proposal of principles.

Capture the following agreements in the TP:

-	Correct the scope in TR38.745.

-	Capture the definition of multip-hop UE trajectory prediction “Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.”

-	Capture the deployment of mulit-hop UE trajectory.

# 4	Annex – TP to 38.745

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, the cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Multi-hop predicted UE trajectory across gNBs consists of a list of cells belonging to one or more gNBs where the UE is expected to connect and these cells are listed in chronological order.

For the measured multi-hop UE trajectory across gNBs, the collected measurement results should be forwarded to the initial gNB. This allows the initial gNB to evaluate and analyze the end-to-end performance of the trajectory prediction and mobility