3GPP TSG-RAN WG3 Meeting #129	R3-255109

# Bengaluru, India, 25-29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1262 | rev | 3 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.413) Support of Multi-hop relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, Huawei, LG Electronics, NEC, CATT, InterDigital, Samsung, ZTE, FirstNet, NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_multihop-Core |  |  |  |  |  | Date: |  |  | 2025-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of NR SL multi-hop relay |  |  |  |  |  |  |  |  |
| Summary of change: |  | introduce new authorization IE for 5G ProSe Layer-2 multi-hop UE-to-Network Relay, 5G ProSe Layer-2 multi-hop Intermediate UE-to-Network Relay, 5G ProSe Layer-2 multi-hop Remote, and 5G ProSe Layer-3 multi-hop UE-to-Network Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR SL multi-hop relay is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.233, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR 0463 TS 38.423 CR 1461 TS 38.470 CR 0165 TS 38.473 CR 1548 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: Capture agreed TP R3-250226 Rev1: upadated based on TS 38.413 v18.5.0 Rev2: resubmit to RAN3#128 Rev3: resubmit to RAN3#129 |  |  |  |  |  |  |  |  |

----------------Start of the First Change---------------

#### 9.3.1.233	5G ProSe Authorized

This IE provides information on the authorization status of the UE to use the 5G ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5G ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Discovery | - |  |
| 5G ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Communication | - |  |
| 5G ProSe Layer-2 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay | - |  |
| 5G ProSe Layer-3 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay | - |  |
| 5G ProSe Layer-2 Remote UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE | - |  |
| 5G ProSe Layer-2 Multipath | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the 5G ProSe Layer-2 Remote UE is authorized for 5G ProSe multipath transmission | YES | ignore |
| 5G ProSe Layer-2 UE-to-UE Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Relay UE | YES | ignore |
| 5G ProSe Layer-2 UE-to-UE Remote | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-UE Remote UE. | YES | ignore |
| 5G ProSe Layer-2 Multi-Hop UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Multi-Hop UE-to-Network Relay. | YES | ignore |
| 5G ProSe Layer-2 Multi-Hop Intermediate UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Multi-Hop Intermediate UE-to-Network Relay. | YES | ignore |
| 5G ProSe Layer-2 Multi-Hop Remote | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Multi-Hop Remote. | YES | ignore |
| 5G ProSe Layer-3 Multi-Hop UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 Multi-Hop UE-to-Network Relay. | YES | ignore |

-----------------Next Change-------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-ExtendedBackupAMFName,

id-FiveGProSeLayer2MHUEtoNetworkRelay,

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay,

id-FiveGProSeLayer2MHRemote,

id-FiveGProSeLayer3MHUEtoNetworkRelay,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

#### -- F

FailureIndication ::= SEQUENCE {

uERLFReportContainer 	UERLFReportContainer,

iE-Extensions		ProtocolExtensionContainer { { FailureIndication-ExtIEs} }	OPTIONAL,

...

}

FailureIndication-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

FiveGCAction ::= CHOICE {

hLComActivate			HLComActivate,

hLComDeactivate			HLComDeactivate,

choice-Extensions		ProtocolIE-SingleContainer { {FiveGCAction-ExtIEs} }

}

FiveGCAction-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

FiveG-ProSeAuthorized ::= SEQUENCE {

fiveGProSeDirectDiscovery				FiveGProSeDirectDiscovery						OPTIONAL,

fiveGProSeDirectCommunication			FiveGProSeDirectCommunication					OPTIONAL,

fiveGProSeLayer2UEtoNetworkRelay		FiveGProSeLayer2UEtoNetworkRelay				OPTIONAL,

fiveGProSeLayer3UEtoNetworkRelay		FiveGProSeLayer3UEtoNetworkRelay				OPTIONAL,

fiveGProSeLayer2RemoteUE				FiveGProSeLayer2RemoteUE						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {FiveG-ProSeAuthorized-ExtIEs} }		OPTIONAL,

...

}

FiveG-ProSeAuthorized-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-FiveGProSeLayer2Multipath		CRITICALITY ignore	EXTENSION FiveGProSeLayer2Multipath			PRESENCE optional		}|

{ ID id-FiveGProSeLayer2UEtoUERelay		CRITICALITY ignore	EXTENSION FiveGProSeLayer2UEtoUERelay		PRESENCE optional		}|

{ ID id-FiveGProSeLayer2UEtoUERemote	CRITICALITY ignore	EXTENSION FiveGProSeLayer2UEtoUERemote		PRESENCE optional		}|

{ ID id-FiveGProSeLayer2MHUEtoNetworkRelay				CRITICALITY ignore	EXTENSION FiveGProSeLayer2MHUEtoNetworkRelay		PRESENCE optional}|

{ ID id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay	CRITICALITY ignore	EXTENSION FiveGProSeLayer2MHIntermediateUEtoNetworkRelay		PRESENCE optional}|

{ ID id-FiveGProSeLayer2MHRemote						CRITICALITY ignore	EXTENSION FiveGProSeLayer2MHRemote					PRESENCE optional}|

{ ID id-FiveGProSeLayer3MHUEtoNetworkRelay				CRITICALITY ignore	EXTENSION FiveGProSeLayer3MHUEtoNetworkRelay		PRESENCE optional},

...

}

FiveGProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer3UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2RemoteUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2Multipath ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2UEtoUERelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2UEtoUERemote ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2MHUEtoNetworkRelay	::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2MHIntermediateUEtoNetworkRelay	::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2MHRemote	::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer3MHUEtoNetworkRelay	::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSePC5QoSParameters ::= SEQUENCE {

fiveGProSepc5QoSFlowList					FiveGProSePC5QoSFlowList,

fiveGProSepc5LinkAggregateBitRates		BitRate												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { FiveG-ProSePC5QoSParameters-ExtIEs} }	OPTIONAL,

...

}

FiveG-ProSePC5QoSParameters-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-----------------Next Change-------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unaffected part is skipped >>>>>>>>>>>>>>>>>>>>

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-TAIMBSSupportList									ProtocolIE-ID ::= 441

id-ExtendedBackupAMFName    							ProtocolIE-ID ::= 442

id-ExtendedOldAMF			    						ProtocolIE-ID ::= 443

id-FiveGProSeLayer2MHUEtoNetworkRelay					ProtocolIE-ID ::= a1

id-FiveGProSeLayer2MHIntermediateUEtoNetworkRelay		ProtocolIE-ID ::= a2

id-FiveGProSeLayer2MHRemote								ProtocolIE-ID ::= a3

id-FiveGProSeLayer3MHUEtoNetworkRelay					ProtocolIE-ID ::= a4

END

-- ASN1STOP

-----------------End of Change-------------------