3GPP TSG-RAN WG3#129 	R3-255138

Bengaluru, India, 25-29 August 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	TP (BL CR TS 38.300, TS 38.473, TS 38.423) Remaining issues on Inter-CU LTM procedure

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-250339 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This paper discusses the remaining issues for the basic required mobility procedures for inter-CU L1/L2 triggered mobility (LTM). These issues are grouped based on the Inter-CU LTM phases (preparation phase, early synchronization phase, execution phase, and completion phase).

# 2	Preparation Phase

With respect to the preparation phase, the following aspects require further consideration.

## 2.1	Retrieval of CSI-RS Resource

To support CSI-RS based LTM, RAN3 has previously reached agreement for retrieving CSI-RS resource over Xn and F1. However, it was left open whether the resource type is needed. The open item should indicate “resource type”, which in our view needs to be indicated.

FFS on whether the gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP.

Proposal 1: The gNB-DU/gNB indicates the resource type (periodic or semi-persistent) of the CSI-RS resources in its reply to Source gNB both in F1AP and XnAP.

## 2.2	Semi-persistent CSI-RS Support for LTM

Support for semi-persistent CSI-RS has been agreed to be introduced over F1 and Xn via newly introduced procedures. However, it was left inconclusive whether to include the TCI State/QCL-info List in the new procedures.

FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure.

The TCI State/QCL-info List is necessary to indicate the associated TCI State for each CSI-RS being activated/deactivated. The TCI state/QCL-info is initially retrieved and informed to source cell during the LTM preparation phase utilizing existing LTM framework. For an SP CSI-RSs, the associated TCI state is selected and provided dynamically. This can be selected by the source cell based on latest SSB or CSI-RS measurements. Hence, the TCI State/QCL-info List should be included and the FFS removed.

Proposal 2: The F1AP DU-CU CSI-RS COORDINATION REQUEST and CU-DU CSI-RS COORDINATION REQUEST messages include a TCI State/QCL-info List which includes the TCI State Index.

Proposal 3: The XnAP CSI-RS COORDINATION REQUEST message includes a TCI State/QCL-info List which includes the TCI State Index.

Additionally, the following working assumption with respect of having activation and deactivation of CSI-RS transmission at level of CSI Resource ID can be turned into agreement.

WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs.

Proposal 4: Turn WA into agreement regarding that “for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource ID”.

## 2.3	Aspects for CSI Acquisition

Likewise, RAN1 agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition, which although discussed in RAN3 on how to support it was left inconclusive.

<< excerpt from RAN1 LS >> Similarly to the semi-persistent CSI-RS resource supported for candidate cell L1-RSRP measurement for gNB scheduled reporting, RAN1 has also agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition. Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 As captured in the agreement, RAN1 sees the spec impact in RAN2 to define the MAC CE. Also, RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case.

To support CSI acquisition, several steps must be carried out, resulting in three “rounds” of interactions between source and candidate. Firstly, to retrieve the CSI-RS resource (Steps a-b). Secondly, to signal the common CSI-RS Resource configuration and retrieve the CSI-RS Report configuration from candidates. Thirdly, to signal the consolidated CSI-RS Reports to candidates (Steps g-e). These steps are described in more detail below:

(a) Source gNB Source requests CSI-RS and CSI-IM resources that are needed for CSI acquisition to the Candidate gNB/gNB-DU via XnAP HANDOVER REQUEST and F1AP UE CONTEXT SETUP messages.

(b) Candidate gNB/gNB-DU provides the CSI-RS/CSI-IM resources requested by source gNB via XnAP HANDOVER REQUEST ACKNOWLEDGEMENT and F1AP UE CONTEXT SETUP RESPONSE messages.

(c) The Source gNB based on the received information prepares the CSI-RS Resource configurations for LTM.

(d) Source gNB provides the CSI-RS Resource configuration and provides it to each Candidate to obtain the CSI Report Configuration for early CSI via XnAP LTM CONFIGURATION UPDATE and F1AP UE CONTEXT MODIFICATION REQUEST messages.

(e) Candidate gNB/gNB-DU provide the CSI-RS Report Configuration to the Source gNB via XnAP LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT and F1AP UE CONTEXT MODIFICATION RESPONSE.

(f) Source gNB consolidates the CSI-RS Reports and updates Candidate gNB/gNB-DU via LTM CONFIGURATION UPDATE and F1AP UE CONTEXT MODIFICATION REQUEST messages.

(g) Candidate gNB/gNB-DU respond to the updated information via XnAP LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT and F1AP UE CONTEXT MODIFICATION RESPONSE messages.

With respect to Steps (a) and (b) above, at last RAN3 meeting it was left inconclusive whether CSI-RS resources for CSI acquisition need to be separately requested from other CSI-RS resources and captured in chair notes within the following FFS.

<< excerpt from RAN3#128 chair notes >> FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP.

In our view, the CSI-RS resources for CSI Acquisition need to be differentiated since it is possible to use different CSI-RS configurations depending on whether the CSI-RS is for L1-RSRP measurements or for CSI acquisition. E.g., the number of ports can differ. Therefore, both the request and the response require explicit indication on whether the CSI-RS resources requested, and the CSI-RS resources provided back correspond specifically to CSI acquisition.

Proposal 5: The source gNB requests the candidate gNB/gNB-DU to provide the CSI-RS resource for CSI acquisition separately via a new IE in Xn HANDOVER REQUEST and F1 UE CONTEXT SETUP REQUEST messages.

In similar way, with respect to Steps (d) and (e), we think separate indication is needed to request the CSI-RS Report Configuration for CSI acquisition.

Proposal 6: When requested by Source gNB, the candidate gNB/gNB-DU provides the CSI-RS Report configuration for CSI Acquisition separately via a new IE via Xn HANDOVER REQUEST ACKNOWLEDGEMENT and F1 UE CONTEXT SETUP RESPONSE messages.

## 2.3	Security Considerations

A pending open item in RAN3 is with respect to how to handle the Rel-19 Set ID assignment among gNB-CUs, which was left open between two options as follows:

FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2: Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message. Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different. FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB.

In our view, it is straightforward to have the Source-CU indicate the Rel-19 Set ID(s) or the range to be assigned to the Candidate gNB. Hence, Option 1 can be taken as the approach to handle the assignment.

Proposal 7: For Rel-19 Set ID assignment, take Option 1, where Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Further, it was also discussed how to provide this information for subsequent LTM. In our view, and aligned with other updates required for Subsequent LTM, the LTM CONFIGURATION UPDATE procedure is appropriate.

Proposal 8: For Subsequent LTM, the Source gNB can provide the Rel-19 Set ID assignment via the LTM CONFIGURATION UPDATE procedure.

## 2.4	Subsequent LTM Update Information Mandatory vs. Optional:

At last RAN3 meeting, it was agreed that Steps 6-7 in Stage 2 signaling procedure for Inter-CU LTM are optional. However, this is pending to be updated in the signaling figure.

Remove Note in TS 38.300 BL CR “Editor’s Note: step 6 and 7 are optional.”

Proposal 9: Update Stage 2 (TS 38.300) signaling diagram to reflect agreement that Steps 6 and 7 are optional.

Further, for several scenarios, these additional steps incur unnecessary delay in several scenarios, as the Source gNB-CU could be able to generate the RRC Reconfiguration message toward the UE without waiting for an additional updated response from all the Candidate gNB-CUs. Thus, it should be possible carrying out any further updates toward the Candidate gNB-CUs at a later time (e.g., to update TNL related information). For example:

Single configuration: if the Source gNB-CU determines that a common LTM CSI-RS resource configuration is used, it could signal this decision to the Candidate gNB-CU(s) in the HANDOVER REQUEST message. Hence, removing the need to consolidate individual reports from each Candidate gNB-CUs prior to generating the RRC Reconfiguration message to the UE.

L3 inter-CU LTM: L3 based LTM is already agreed to be applicable to Inter-CU scenarios. Thus, another related case is when any given Candidate gNB-CU only supports L3 based LTM configuration. Such nodes need not generate a lower layer LTM configuration to be consolidated at the Source gNB-CU and could indicate it accordingly to the Source gNB-CU. The Source gNB-CU would in place proceed to generate the RRC Reconfiguration message to the UE without a second update procedure for these Candidates.

Subsequent LTM not required: Moreover, if subsequent LTM is determined not to be required after Inter-CU LTM operation either by the source or Candidate gNB-CU, these nodes could coordinate whether to restrict Subsequent Inter-CU LTM mobility via Xn signaling. In this case, Steps 6-7 would not be required entirely.

Proposal 10: RAN3 to study impact and required handling in Inter-CU LTM preparation phase when only L3 based Inter-CU LTM is supported at the Candidate gNB-CU(s).

Proposal 11: RAN3 to study means to simplify Inter-CU LTM preparation by support of using a common CSI-RS configuration, or by limiting execution of subsequent LTM.

# 3	Early Synchronization Phase

With respect to the early synchronization phase, the following aspects require further consideration.

## 3.1	TA Information Transfer

RAN3 has agreed to introduce a new non-UE associated procedure over Xn to transfer TA information.

Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. Include the following information in the TA information transfer message: Candidate Cell ID TA value Preamble index RA-RNTI FFS on the XnAP IDs.

The format of this message simply needs to match to existing F1AP messages introduced in Rel-18 for the same purpose, and adding an XnAP ID on top is not needed. Further, such addition would introduce a conflict with existing messages over F1.

Proposal 12: XnAP IDs are NOT needed in the XnAP TA INFORMATION TRANSFER message.

# 4	LTM Execution Phase

With respect to the LTM execution phase, the following aspects require further consideration.

## 4.1	Timing Advance Command (TAC)

Upon cell switch decision, the gNB-DU sends a MAC CE command to the UE which includes the TCI state indicating the UE which beams it should use to access the target cell (applicable for both RACH-less and RACH-based mobility), as well as a Timing Advance Command (TAC). Likewise, the value in the TAC indicates to the UE whether it should perform a RACH-less switch.

However, over F1AP and XnAP interfaces, the Cell Switch Notification messages although it includes the TCI State ID, lacks support for indicating whether RACH-less mobility was triggered to the UE. Therefore, the Candidate gNB-DU/Candidate gNB is unaware whether the upcoming UE access is RACH-less or RACH-based. Consequently, it can also not determine whether it should issue a Dynamic Grant (DG) for DG-based RACH-less cell switch, or whether it should activate a Configured Grant (CG) previously provided in the RRC Configuration for CG-based RACH-less switch. Thus, due to this uncertainty, the target cell is forced to issue a DG, even when the triggered switch by the source was RACH-based, leading to PDCCH and PUSCH resources being wasted. Similarly, to support CG-based RACH-less cell switch, the target cell is also forced to activate and monitor the CG, even for the RACH-based cell switch, leading to PUSCH resource waste. For these reasons, an IE from Source gNB-DU/gNB is required over F1 and Xn interfaces to indicate whether the cell switch triggered was RACH-less based correspondingly.

Proposal 13: Add a “Cell Switch Type (ENUMERATED(RACH-less, RACH-based))” in F1AP and XnAP CELL SWITCH NOTIFICATION messages.

## 4.2	Data Forwarding and TNL Information Update

To support data forwarding in case of inter-CU LTM, the Source gNB needs to transfer information regarding PDU Session Resource Information List of all other Candidate gNBs to the Target gNB to execute data forwarding for subsequent LTM. Similarly, RAN3 has agreed this information to be included within LTM CONFIGURATION UPDATE message with the exact timing left to implementation.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation. … Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).

Given the RAN3 agreement that timing is left to implementation, we believe it is useful to also optionally include the data forwarding related information in the LTM CELL SWITCH NOTIFICATION message. In this way the Source-gNB can utilize this existing signaling step to inform the data forwarding details to the new Source-gNB as well without having to wait for an additional procedure (i.e. a LTM CONFIGURATION UPDATE message afterwards).

Proposal 14: The Source gNB may also provide PDU Session Resource Information List of all Candidate gNBs via the LTM CELL SWITCH NOTIFICATION message.

## 4.3	TA Forwarding

In the Rel-18 DU-CU and CU-DU CELL SWITCH NOTIFICATION messages, the TA information list IE may be included to transfer obtained TA values to the target (i.e., new serving) cell. Similarly, the TA validity was left up to implementation in case of intra-CU LTM.

If TA forwarding is supported for Rel-19 Inter-CU LTM, we believe the TA validity cannot be left up to implementation as the Xn link delay needs to be accounted for. A possible approach which can be linked to UE signal experienced, is to provide information such as the RSRP which the UE experienced when TA was acquired. This would allow the Target-gNB/DU to better understand the signal conditions associated with a TA value being received, rather than a “time” value tied to the TA acquisition itself. Based on the received RSRP value along with TA value(s) and the new RSRP value, the new source DU can determine the validity of the TA value. Note that such scheme is already possible for SDT and should be considered for LTM.

Proposal 15: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

Figure 1: Execution Phase Considerations for Subsequent Inter-gNB-CU LTM

# 4	Conclusions

Preparation Phase

Proposal 1: The gNB-DU/gNB indicates the resource type (periodic or semi-persistent) of the CSI-RS resources in its reply to Source gNB both in F1AP and XnAP.

Proposal 2: Turn WA into agreement regarding that “for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource ID”.

Proposal 3: The F1AP DU-CU CSI-RS COORDINATION REQUEST and CU-DU CSI-RS COORDINATION REQUEST messages include a TCI State/QCL-info List which includes the TCI State Index.

Proposal 4: The XnAP CSI-RS COORDINATION REQUEST message includes a TCI State/QCL-info List which includes the TCI State Index.

Proposal 5: The source gNB requests the candidate gNB/gNB-DU to provide the CSI-RS resource for CSI acquisition separately via a new IE in Xn HANDOVER REQUEST and F1 UE CONTEXT SETUP REQUEST messages.

Proposal 6: When requested by Source gNB, the candidate gNB/gNB-DU provides the CSI-RS Report configuration for CSI Acquisition separately via a new IE via Xn HANDOVER REQUEST ACKNOWLEDGEMENT and F1 UE CONTEXT SETUP RESPONSE messages.

Proposal 7: For Rel-19 Set ID assignment, take Option 1, where Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message.

Proposal 8: For Subsequent LTM, the Source gNB can provide the Rel-19 Set ID assignment via the LTM CONFIGURATION UPDATE procedure.

Proposal 9: Update Stage 2 (TS 38.300) signaling diagram to reflect agreement that Steps 6 and 7 are optional.

Proposal 10: RAN3 to study impact and required handling in Inter-CU LTM preparation phase when only L3 based Inter-CU LTM is supported at the Candidate gNB-CU(s).

Proposal 11: RAN3 to study means to simplify Inter-CU LTM preparation by support of using a common CSI-RS configuration, or by limiting execution of subsequent LTM.

Early Synchronization Phase

Proposal 12: XnAP IDs are NOT needed in the XnAP TA INFORMATION TRANSFER message.

LTM Execution Phase

Proposal 13: Add a “Cell Switch Type (ENUMERATED(RACH-less, RACH-based))” in F1AP and XnAP CELL SWITCH NOTIFICATION messages.

Proposal 14: The Source gNB may also provide PDU Session Resource Information List of all Candidate gNBs via the LTM CELL SWITCH NOTIFICATION message.

Proposal 15: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

# References

- 3GPP TS 38.401

- RP-250339 Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

# Annex: Text Proposal for TS 38.300

<< First Change >>

##### 9.2.3.5.2	C-Plane Handling

Cell switch command is conveyed in a MAC CE, which contains the necessary information to perform the LTM cell switch.

The overall procedure for LTM is shown in Figure 9.2.3.5.2-1 below. Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without the need to release, reconfigure or add other LTM candidate configurations after each LTM cell switch completion. The general procedure over the air interface is applicable to SCG LTM. Further details of SCG LTM can be found in TS 37.340 [21].

Figure 9.2.3.5.2-1. Signalling procedure for LTM

The procedure for LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure LTM.

3.	The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-gNB LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and may contain the CSI resource configuration for subsequent LTM. For both intra and inter-gNB LTM, the source gNB may request the candidate cell(s)/gNB(s) to provide the CSI resource configuration.

4.	Admission Control may be performed by the candidate cells(s)/gNB(s).

5.	The candidate prepares and provides the LTM configuration(s) to the source gNB. For inter-gNB LTM, the candidate gNB(s) respond(s) with HANDOVER REQUEST ACKNOWLEDGE message to the source gNB including the generated RRC configurations for the accepted candidate cell. For both intra and inter-gNB LTM, the candidate may also include additional information related to the CSI-RS resource configuration and early sync information upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI resource configuration and the LTM configuration ID mapping list.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI report configuration.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

NOTE :	The source gNB may initiate CSI-RS Coordination procedure to activate or deactivate CSI-RS resource(s) of some candidate cells(s).

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to the candidate gNB(s).

10/11. Early DL and UL synchronization to some LTM candidate cell(s) may be performed. The source gNB may activate or deactivate the TCI states of the candidate LTM cells.  Depending on NW configuration, the UE may perform early UL synchronization with LTM candidate cell(s), by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the source gNB. With a NW triggered UL synchronization, a PDCCH order is received from the source cell to trigger CFRA to a candidate cell, the UE performs early TA acquisition by sending preamble towards the indicated candidate cell. In order to minimize the data interruption on the source cell due to CFRA towards the indicated candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB instead.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB, if configured. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate LTM. L3 measurement can also be used to determine this step.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a TA command for the target cell. In case of a security context change, the LTM cell switch command MAC CE also contains the NCC value. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID. In case of security context change, the UE generates and applies the new security keys based on the received NCC value.

NOTE :	Up to implementation, data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 16 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data

NOTE :	Steps 17/18 and 19 do not have to occur one after the other. The target gNB may send the HANDOVER SUCCESS message to the source gNB after receiving the RRCReconfigurationComplete message.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs. This message includes the new security key(s) to be used with the UE.

21. The candidate gNB(s) responds to the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

The steps 10-22 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 8.

The procedure over the air interface described in Figure 9.2.3.5.2-1 is applicable to both intra-gNB-DU LTM , inter-gNB-DU LTM and inter-gNB-CU LTM. The overall LTM procedures over F1-C interface are captured in TS 38.401[4].

<< End of Changes >>

# Annex: Text Proposal for TS 38.473

<< Start of Changes >>

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Cell Switch Type | O |  | ENUMERATED(RACH-less, RACH-based, …) | Indicates whether the cell switch is RACH-less or RACH-based | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >>RSRP | O |  | INTEGER (0… 127) | Indicates the RSRP when the TA value was requested. Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

/*********************Next change***********************/

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Cell Switch Type | O |  | ENUMERATED(RACH-less, RACH-based, …) | Indicates whether the cell switch is RACH-less or RACH-based | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >>RSRP | O |  | INTEGER (0… 127) | Indicates the RSRP when the TA value was requested. Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<< Next change >>

#### 9.2.2.x1	DU-CU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-DU to request the gNB-CU e.g. to activate/deactivate the SP CSI-RS transmissions from specific cells. (Detail is FFS)

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| CSI-RS to be Activated List |  | 0..1 | Detailed IE structure is FFS |  |
| >CSI-RS to be Activatedtem IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>CSI-RS Resources ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  |
| >>>SP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >>>TCI State ID | M |  | OCTET STRING |  |
| CSI-RS to be Deactivated List |  | 0..1 | Detailed IE structure is FFS |  |
| >CSI-RS to be Deactivated Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>CSI-RS Resources ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  |
| >>>SP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >>>TCI State ID | M |  | OCTET STRING |  |

| Range bound | Explanation |
| --- | --- |
| MaxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is to be activated or deactivated, the maximum value is 8. |
| maxnoofCSIRSResourceIDs | Maximum number of CSI RS Resource IDs. Value is 192. |

<< Next change >>

#### 9.2.2.y1	CU-DU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-CU e.g. to request the gNB-DU to activate/deactivate the SP CSI-RS transmissions from specific cells. (Detail is FFS)

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |
| CSI-RS to be Activated List |  | 0..1 | Detailed IE structure is FFS |  |
| >CSI-RS to be Activated Item IEs |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>CSI-RS Resources ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  |
| >>>SP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >>>TCI State ID | M |  | OCTET STRING |  |
| CSI-RS to be Deactivated List |  | 0..1 | Detailed IE structure is FFS |  |
| >CSI-RS to be Deactivated Item Ies |  | 1 .. <maxnoofCellList> |  |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>CSI-RS Resources ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  |
| >>>SP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >>>TCI State ID | M |  | OCTET STRING |  |

| Range bound | Explanation |
| --- | --- |
| MaxnoofCellList | Maximum no. of Cells in which the SP CSI-RS is to be activated or deactivated, the maximum value is 8. |
| maxnoofCSIRSResourceIDs | Maximum number of CSI RS Resource IDs. Value is 192. |

# Annex: Text Proposal for TS 38.423

<< Start of Changes >>

#### 9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Data Forwarding Info | FFS |  |  |  | YES | ignore |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |
| >Cell Switch Type | O |  | ENUMERATED(RACH-less, RACH-based, …) | Indicates whether the cell switch is RACH-less or RACH-based | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >>RSRP | O |  | INTEGER (0… 127) | Indicates the RSRP when the TA value was requested. Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.x.x7 	CSI-RS COORDINATION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to coordinate the activation and deactivation of CSI-RS transmission for a UE at NG-RAN node2.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Based on the WA: RAN3 agrees that, for both F1AP and XnAP, the activation and deactivation of CSI-RS transmission in LTM candidate cells are performed at the level of individual CSI-RS Resource IDs. Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 UE XnAP ID | M |  | NG-RAN node1 UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| NG-RAN node2 UE XnAP ID | M |  | NG-RAN node2 UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| CSI-RS Resource Request List |  | 1 |  | Cell ID list to which the request applies. | YES | reject |
| >CSI-RS Resource Request Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>Semi Persistent CSI-RS Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>CSI-RS Resources ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  | – |  |
| >>>CSI-RS Resource ID | M |  | INTEGER (0..191) |  | – |  |
| >>>TCI State Index | M |  | OCTET STRING |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |
| maxnoofCSIRSResourceIDs | Maximum number of CSI RS Resource IDs. Value is 192. |

<< End of Changes >>