3GPP TSG-RAN WG3#129 	R3-255140

Bengaluru, India, 25-29 August 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on Conditional LTM in split architecture

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-250339 NR mobility enhancements Phase 4), where the objectives relevant to conditional LTM (CLTM) are described as:

• Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] o Specify UE evaluated conditions for triggering LTM o Aim to support conditional LTM including subsequent LTM o Limit specifying the conditional LTM to the scenario where the UE is in non-DC • Specify RRM requirements related to the above objectives as necessary [RAN4]

The primary motivation for CLTM is to introduce robustness to the LTM procedure where the UE may not be able to receive the cell-switch command at right radio condition which may lead to radio link failure. Compared to CHO, the conditional LTM is expected to bring the benefits of robustness along with the interruption time reduction benefits of LTM. In this contribution, we discuss base requirements from RAN3 perspective to support Intra-CU Conditional LTM in disaggregated gNB architecture.

# 2	Discussion

RAN3 has also agreed the following at last meeting, from which we build our proposals.

From RAN3#128: RAN3 confirms that the network can trigger a LTM Cell Switch Command MAC CE towards a candidate cell with C-LTM candidate configuration. The candidate (initial source) DU need to know the L1 C-LTM and the L3 C-LTM. The DU needs to know to which other candidate cell(s) to generate the L1 execution condition for L1 C-LTM. WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”. WA: To introduce a new IE with a list of candidate cells for L1 execution condition. It is acknowledged that the C-LTM indicator needs to be sent to the initial source DU for the very first C-LTM. The candidate DU generates the TAT for its own candidate cells.

## 2.1	Conditional cell switch condition configuration

Conditional LTM follows Rel-18 LTM principle which reserves cell and RACH resources when configuring LTM and can be based on L1 or L3 execution conditions. Likewise, RAN2 defined RRC structure is such that a Conditional LTM cell is always also an LTM cell. In other words, the network can always trigger legacy LTM for a Conditional LTM and both functions coexist. A restriction however exists that both L1 Conditional LTM and L3 Conditional LTM cannot be simultaneously configured. At last meeting RAN3 confirmed this via the following agreement.

RAN3 confirms that the network can trigger a LTM Cell Switch Command MAC CE towards a candidate cell with C-LTM candidate configuration.

For L1-CLTM, the main action by the CU is to indicate when L1 execution conditions are requested for a L1-CLTM cell. For a L3-CLTM cell configuration, the DU does not need to carry any other function other than the legacy LTM preparation. However, it needs to be aware of which cells are L3-CLTM for later phases, such as early synch where the mechanism for delivering an acquired TA to the UE will differ from that of a legacy cell (TA delivered via a newly defined MAC CE in Rel-19, and not via Cell Switch Command as in Rel-18). At last meeting, RAN3 also agreed that L1-CLTM and L3-CLTM are indicated separately to the Source-DU.

The candidate (initial source) DU need to know the L1 C-LTM and the L3 C-LTM.

From an IE structure perspective, RAN3 also reached a working assumption to introduce a single codepoint in legacy LTM Indicator IE applicable for both L1 and L3 Conditional LTM, and a new IE with a list of candidate cells where L1 execution conditions are requested to be generated, and which when present also implicitly differentiates from L3 Conditional LTM. These working assumptions should be turned into agreements.

WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”. WA: To introduce a new IE with a list of candidate cells for L1 execution condition.

Proposal 1: Turn following WAs into agreements (a) “WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.”, and (b) “WA: To introduce a new IE with a list of candidate cells for L1 execution condition.”.

Additionally, it was acknowledged that the scenario where the Source DU cell requires to generate L1 execution conditions despite not being an LTM cell is valid (e.g., mobility is taking place from a non-LTM cell in DU1 toward a C-LTM cell in DU2). It was captured in the following RAN3 agreement in the chair notes.

It is acknowledged that the C-LTM indicator needs to be sent to the initial source DU for the very first C-LTM.

Hence, impact to F1AP is foreseen. In our view this new indication is needed separate from early sync related information, since the L1 execution conditions are not tied or require early sync to operate.

Proposal 2: The indication to Source DU to generate L1 execution conditions can be requested separately from early sync serving cell information.

## 2.2	Early Synchronization

L3 based Conditional LTM considerations

Conditional LTM is also meant to support early synchronization. For L1-CLTM, the existing Rel-18 LTM framework is sufficient for triggering early sync related procedures and we foresee no additional F1AP specification impact for this case. Similarly, for the L3 measurement based conditional LTM, the Rel-18 F1AP CU-DU MOBILITY INITIATION REQUEST message can also be reused to trigger PDDCH-ordered early RACH procedure toward given cells.

Proposal 3: For L3 measurement based Conditional LTM, the Rel-18 CU-DU MOBILITY INITIATION REQUEST message from the gNB-CU to the gNB-DU is reused to trigger PDCCH-ordered early RACH procedure toward given cell(s).

# 3	Conclusions

Proposal 1: Turn following WAs into agreements (a) “WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.”, and (b) “WA: To introduce a new IE with a list of candidate cells for L1 execution condition.”.

Proposal 2: The indication to Source DU to generate L1 execution conditions can be requested separately from early sync serving cell information.

Proposal 3: For L3 measurement based Conditional LTM, the Rel-18 CU-DU MOBILITY INITIATION REQUEST message from the gNB-CU to the gNB-DU is reused to trigger PDCCH-ordered early RACH procedure toward given cell(s).

# References

- RP-250339 Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom