3GPP TSG-RAN WG3 Meeting #129	R3-255157

Bengaluru, Karnataka, India, August 25th – 29th 2025

Agenda Item:	12.2

Source:	Ericsson, Jio Platforms

Title:	(TP for WAB BL CR for TS 38.401): Functional Aspects of WAB-Nodes

Document for:	Agreement

# Introduction

The present TP captures the proposals from R3-255156.

Proposal: Agree the TP for WAB BL CR for TS 38.401, provided below.

# TP for WAB BL CR for TS 38.401

-------------------------------------------Start of changes-------------------------------------------

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

Figure X.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-gNB. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information needed to establish NG connections towards one or more AMF(s), and the WAB-gNB is service-authorized by the SeGW or by the OAM.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step follows legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB can establish Xn connection(s) towards the BH-gNB and/or other NG-RAN node(s). If the WAB-gNB includes a WAB-MT identifier in the signalling for Xn connection setup, the BH-gNB and/or other NG-RAN node(s) can understand that the peer node is the WAB-gNB of the WAB-node.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

## X.4 	WAB-node authorization

WAB-node authorization includes authorization of the WAB-MT’s backhaul support, and service authorization of the WAB-gNB, as defined in TS 23.501 [3].

Authorization of the WAB-MT is different from the service authorization of the WAB-gNB.

Authorization of the WAB-MT is defined in TS 23.501 [3].

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs.

When the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”:

- The WAB-gNB attempts to handover, or releases, the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- The PDU sessions of the WAB-MT used for backhauling may be released.

It is expected that WAB-MT’s PLMN/SNPN ensures that backhaul PDU sessions of the WAB-MT are maintained long enough for the WAB-gNB to perform UE handover/release and the removal of NG and Xn connections, as specified in TS 23.501 [3].

Once the WAB-node’s service authorization status changes from “not authorized” to “authorized”, the WAB-gNB can use the WAB-MT’s PDU sessions to establish connections to the SeGW, the OAM, the AMF(s) and, optionally, towards the BH-gNB and neighbouring NG-RAN nodes.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### X.7.2	WAB-gNB mobility

#### X.7.2.1 	WAB-gNB mobility with change of UE’s AMF(s)

Due to the WAB-gNB’s mobility, the UEs moving together with the WAB-gNB may have to change AMF.

For the AMF change, a new logical WAB-gNB is instantiated, which establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may obtain from the OAM the configuration parameters needed to establish the connection(s) to the UE’s new AMF(s), based on, e.g., WAB-node’s location.

The new logical WAB-gNB may activate one or more cell(s) with new TAC, cell ID, and PCI, which depend on the WAB-node’s current location.

The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over from a cell served by the old logical WAB-gNB to a cell served by the new logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502 [7], after which the UE’s AMF is changed to the new AMF.

- A UE in RRC_IDLE or RRC_INACTIVE state camping on a cell served by the old logical WAB-gNB reselects a cell served by the new logical WAB-gNB. The reselection may be triggered by the removal of the old logical WAB-gNB’s cells from service. After cell reselection, the UE performs a Mobility Registration Update as defined in TS 23.502 [7], which is triggered by the new TAC broadcasted by the new logical WAB-gNB’s cell. During this Mobility Registration Update, the UE’s AMF is changed to the new AMF.

After all the UEs in RRC_CONNECTED state are handed over, the NG connection(s) between the WAB-node and the old AMF(s) are removed via NG Removal procedure and the old logical WAB-gNB’s cell(s) are removed from service.

## X.8 	Coordination of WAB-gNB and WAB-MT radio resources

Coordination of access link and backhaul link radio resources can be done up to implementation. It may be needed when the access link and the backhaul link of a WAB-node are operated in the same band (i.e., referred to as the in-band operation).

NOTE: 	In-band WAB operation may only be considered in scenarios where a WAB-gNB and its co-located WAB-MT connect to the same PLMN. When a WAB-gNB and its co-located WAB-MT connect to different PLMNs, it is assumed that the access link and the backhaul link of a WAB-node are operated in different bands.

## X.9 	Xn interface of WAB-nodes

A WAB-gNB can establish an Xn connection with the BH-gNB serving the co-located WAB-MT, and with the neighbouring gNBs, based on the OAM configuration. During setup or update of its Xn connections, the WAB-gNB can indicate an ID of the WAB-MT, thus making the BH-gNB aware of the co-location of the WAB-MT and the WAB-gNB. The WAB-MT’s ID consists of the C-RNTI assigned to the WAB-MT by the BH-gNB and the cell ID of BH-gNB´s cell serving the WAB MT.

The BH-gNB may indicate to the WAB-gNB the information of its neighbouring gNBs, and the information of the target BH-gNB for WAB-MT handover.

Xn connections can also be established between WAB-gNBs.

-------------------------------------------End of changes-------------------------------------------