3GPP TSG-RAN WG3 Meeting #129	R3-255160

Bengaluru, India, 25 – 29 August 2025

Agenda item:	20.2

Source: 	Qualcomm Incorporated

Title: 	Views on the Open Issues for AI/ML Positioning Accuracy Enhancements

Document for:		Discussion and Decision

# 1.	Introduction

In this contribution, we provide our view on the Open Issues for the AI/ML positioning accuracy enhancements for network-based positioning Case 3a:

-	Inferred measurement results and request;

-	Requested Positioning Data label information ("Part B");

-	POSITIONING DATA COLLECTION REPORT: Indication of "Positioning Data Unavailable";

-	Proactive data collection way forward;

and for Case 3b:

-	TRP "Channel Measurement" information.

# 2.	Discussion

## 2.1	Inferred measurement results and request

The TRP Measurement Result IE (9.2.37) can currently indicate whether the provided UL-RTOA and gNB Rx-Tx Time Difference measurements (timing measurements) are inferred measurements [1]. Whether the "LoS/NLoS Indicator" can also be inferred or not is currently "FFS" [1]. However, the following general agreement was made at RAN1#116 [2]:

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

and RAN1#118bis [3]:

Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

and RAN1#119 [3]:

Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements

and RAN1#120-bis [4]:

Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

and RAN1#121 [4]

For Case 3a, the FFS in RAN1#118bis agreement is resolved by updating the agreement as follows. Agreement (RAN1#118bis) From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. - FFS: LOS/NLOS indicator. - (Optional) LOS/NLOS indicator with legacy format Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Note: It is up to RAN3 to decide whether the field of LOS/NLOS indicator should be removed or kept..

Therefore, from the above set of agreements the LoS/NLoS indicator could be an inferred measurement for AI/ML assisted positioning Case 3a and a LMF can provide the "ground-truth label" for this measurement to the gNB for training and performance monitoring.

Proposal 1:	The "LoS/NLoS indicator" can also be a gNB inferred measurement.

It has also been clarified by RAN1 above that the LoS/NLoS indicator can’t be reported independently from other measurements (same as in current Releases for non-inferred measurements). However, the current version of the NRPPa baseline CR [1] also provides the LoS/NLoS label information ("Part B") only together with timing measurements (IE Positioning Data Information). There seems no corresponding RAN1 and/or RAN3 agreement for this. I.e., NG-RAN should be able to request and an LMF should be able to provide any of the desired label information. A LoS/NLoS label does not need to be contingent on timing measurements.

Proposal 2:	In a POSITIONING DATA COLLECTION REPORT message, a LMF can provide the LoS/NLoS label information independently of any timing information label. The current semantics description in IE Positioning Data Information (9.2.Z2 [1]) is removed.

Although, a MEASUREMENT RESPONSE/REPORT message can currently indicate whether the provided TRP measurement results are inferred measurements or not, the corresponding MEASUREMENT REQUEST message can currently not explicitly request inferred measurement results from a TRP. From RAN3#128:

FFS: if LMF should specifically request for AIML inferred measurement.

A LMF may decide whether to request inferred measurements from the gNB in AI/ML-based positioning based on for example application requirements (e.g., QoS, emergency vs. commercial use, etc.) or TRP capabilities. If only a small sub-set of the selected TRPs host machine learning models and are capable of providing inferred measurements, a LMF may decide to use conventional (non-inferred) measurement results based on the majority TRP support and may not want to mix inferred and non-inferred measurements in the position calculation. The gNB processing latency may also be different between AI/ML generated measurements and conventional measurements, and a LMF may choose to use e.g., conventional measurements only for time critical applications (e.g., emergency use). A LMF may also not always trust the accuracy and reliability of the gNB-side AI/ML models for critical applications (e.g., emergency use) and may prefer to use conventional measurements instead (e.g., also considering the current limitations for gNB-side "opportunistic" data collection for model training (see also section 2.4 below)). A gNB can monitor/validate a model performance only in "measurement domain" and not in the final "positioning domain". Allowing an LMF to specifically request AI/ML generated or conventional measurements may also enable an LMF to compare and/or use both types of measurement results. It also provides the flexibility to request for example inferred RTOA measurements for UL-TDOA processing and conventional (non-inferred) gNB Rx-Tx Time Difference measurements for Multi-RTT processing (e.g., due to lack of corresponding UE support). Therefore, there are the following main benefits for an explicit request for inferred measurements:

-	Application requirements (e.g., QoS, emergency vs. commercial use) may influence whether inferred or conventional measurements are preferred.

-	TRP capabilities: If only a few TRPs support AI/ML inference, the LMF may opt for conventional measurements to maintain consistency.

-	Mixing inferred and non-inferred measurements may be avoided to ensure uniformity in positioning calculations.

-	Processing latency may differ between AI/ML and conventional methods and may lead the LMF to prefer conventional data for time-critical applications.

-	Positioning method differences: E.g., a LMF may opt for inferred RTOA for UL-TDOA and conventional (non-inferred) Rx-Tx time differences for Multi-RTT, or vice versa.

Proposal 3:	It should be possible for an LMF to specifically request for AI/ML inferred measurements from TRPs.

A potential implementation of Proposal 3 could be (in principle analogous to LPP) an indication in the measurement request like for example 'AI/ML required', 'Non-AI/ML required', 'AI/ML preferred, but non-AI/ML allowed', 'Non-AI/ML preferred, but AI/ML allowed'.

## 2.2	Requested Positioning Data label information ("Part B")

The POSITIONING DATA COLLECTION REPORT provides the requested "ground-truth labels" to the gNB, which currently include:

-	UL RTOA

-	gNB Rx-Tx Time Difference

-	LoS/NLoS Information

-	Time Stamp and Measurement Quality

The above list is in agreement with the latest RAN1 parameter list [5]. However, this implies that different entities perform the data collection, model training, performance monitoring (at gNB) and the label calculation (at LMF). From RAN3#128:

FFS if UE location can be requested.

For RTOA, the label calculation typically requires obtaining an expected Time of Arrival by using the known UE location and the gNB (TRP/ARP) location to compute a geometric distance. However, a geometric distance alone typically may not accurately reflect a "true" RTOA measurement, since an RTOA measurement may also be impacted by clock offsets between UE and gNB, reference signal timing (e.g., Timing Advance) and probably other gNB implementation dependent factors which may be known only internally at the gNB.

Similar, obtaining a gNB Rx-Tx Time Difference label involves computing the difference between the reception and transmission timestamps at the gNB for a signal exchanged with the UE. A round-trip delay could be obtained from a geometric distance between UE and gNB (TRP/ARP). However, gNB hardware and processing delays may also affect a gNB Rx-Tx Time Difference label which may be known only internally at the gNB.

A LMF may be able to compute the measurement labels primarily from geometric distance calculations but may not be able to consider gNB-specific contributions to the measurement labels. Therefore, it seems more natural that the entity which performs the data collection, model training and performance monitoring also calculates the required label data. I.e., the entity which performs the "measurements" should also calculate the "labels" (and not a 3rd party LMF). Assuming a gNB knows its own TRP/ARP coordinates, the gNB would require the UE location for determining the intermediate "ground-truth measurement" label.

Proposal 4:	It should be possible for an LMF to optionally provide the UE location in a POSITIONING DATA COLLECTION REPORT to calculate the intermediate "ground-truth measurement" label at the gNB.

The UE location could be obtained at an LMF by using any NW and UE supported positioning method, which in general does not require any additional LMF functionality (as opposed to calculating labels at the LMF). However, the UE location should only be provided to the gNB when the LCS privacy profile settings allow for such a transfer.

The Data Collection procedure for Case 3b as described in clause 6.22.3 of TS 23.273 [6] checks the "User Consent" for data collection at Step 4. However, there is currently no checking of the LCS Privacy Profile, neither for Case 3a nor for Case 3b, as also noticed in the response LS from SA2 [7]:

SA2 is currently discussing for data collection for training models to be used by the LMF for positioning (RAN case 3b) whether a user consent check and/or a LCS privacy profile check is required. SA2 is anticipating that similar checks could also be required for case 3a (both for solution 1 and solution 2).

The "opportunistic data collection" procedure in clause 7.x.2 of the Stage 2 running CR [8] does currently not include any privacy check.

Proposal 5:	Add an "LCS Privacy Check" step to the data collection procedure for Case 3a before providing the UE location to the NG-RAN. A TP is proposed in [9].

Note, "User Consent" indicates whether the user has given consent for collecting, distributing and analysing UE related data. User consent is provided per purpose (e.g. analytics, model training) (TS 23.502).  "UE LCS privacy" is a feature which allows a UE and/or AF to control which LCS clients and AFs are and are not allowed to access the UE location information (23.273).

## 2.3	POSITIONING DATA COLLECTION REPORT: Indication of "Positioning Data Unavailable"

Per RAN3#128:

Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason.

The current NRPPa baseline CR includes only an ENUMERATED {true} value for the "unavailability indicator" [1]:

#### 9.1.X.1 POSITIONING DATA COLLECTION REPORT This message is sent to report positioning information for data collection. Direction: LMF NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3; YES; reject. NRPPa Transaction ID; M; 9.2.4; -. LMF Measurement ID; M; INTEGER (1..65536, …); YES; reject. RAN Measurement ID; M; INTEGER (1..65536, …); YES; reject. Positioning Data Information; O; 9.2.Z2; YES; ignore. Positioning Data Unavailable; O; ENUMERATED(true, …) [FFS]; YES; ignore.

The Positioning Data Unavailable IE should at least indicate whether the requested data are temporarily unavailable (e.g., due to positioning method failure) or permanently unavailable for this UE (e.g., due to UE privacy restrictions).  This would for example avoid that a gNB continues requesting the same data for this UE (e.g., in MEASUREMENT REPORT messages). The error causes could be similar to the LPP location server error causes. For example:

-	Positioning Data Information Not Supported

-	Positioning Data Information Supported But Currently Not Available

-	Not Provided Positioning Data Information Not Supported

Proposal 6a:	The Positioning Data Unavailable IE should at least indicate whether the requested data are temporarily unavailable (e.g., due to positioning method failure) or permanently unavailable for this UE (e.g., due to UE privacy restriction).

Proposal 6b:	Add the following error causes to the Positioning Data Unavailable IE:

-	Positioning Data Information Not Supported

-	Positioning Data Information Supported But Currently Not Available

-	Not Provided Positioning Data Information Not Supported

## 2.4	Proactive data collection solution

At RAN3#127, it was agreed:

For case 3a, it should be possible to collect data in both an "opportunistic" (gNB collects available data from ongoing positioning sessions) and "proactive" (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

However, the current baseline CRs support only the "opportunistic data collection" [1], [8]. An LS was sent to SA2 at RAN3#127bis which discussed the issue at SA2#169 and provided the following response [7]:

SA2 Answer: For solution 1 it may be desirable to document related LMF procedures in TS 23.273. There are SA2 impacts to support the solution but SA2 could address those impacts as alignment with RAN in Rel-19. For solution 2, SA2 concludes that this solution is not feasible in current frozen Rel-19. SA2 is currently discussing for data collection for training models to be used by the LMF for positioning (RAN case 3b) whether a user consent check and/or a LCS privacy profile check is required. SA2 is anticipating that similar checks could also be required for case 3a (both for solution 1 and solution 2).

To complete the "proactive" data collection for Case 3a, there seems the following possibilities:

(1)	Extend the Work Item [10] for 1 Quarter and complete the work in Rel-19.

(2)	Add a related objective to the Rel-20 Work Item [11].

(3)	Complete the work as TEI20 item.

Option (1) seems not sensible, since even if RAN3 completes the work in Rel-19, SA2 (and CT4) may not align their specifications in Rel-19 given the response LS from SA2 above.

The new Rel-20 Work Item on "Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements" [11] includes already some objectives for RAN3 but related to "Inter-vendor training collaboration for two-sided AI/ML models". Nevertheless, an additional objective for Case 3a data collection could be proposed at next RAN plenary. However, although the current RAN3 TU allocation may not need to be changed, RAN3 would begin the work at RAN3#133bis with the Rel-20 freeze scheduled for March 27. This may then also be quite late for any SA2/CT4 alignment CR work. Therefore, it may be better if RAN3 could complete the work as a TEI20 item as soon as TEI20 is open for contributions in RAN3 and provide the solution to SA2 for aligning the SA2 specs with the RAN3 solution. In any case, there should be no SA2 work item/work item updates required, since SA2 could simply align their specifications with the RAN3 solution (i.e., via LS; similar to the "opportunistic" data collection solution).

Proposal 7:	RAN3 agree to complete the "proactive data collection" solution for Case 3a as a TEI20 item and, when completed, ask SA2 via LS to align the SA2 specifications with the RAN3 solution.

## 2.5	TRP "Channel Measurement" information

For Case 3b, the model input is the "time domain channel measurement" for which two types are defined:

Type A:	"path-based measurements", which is referring to the existing NRPPa Additional Path List (9.2.41) and Extended Additional Path List (9.2.74).

Type B:	"sample-based measurements", which is providing the Nt' strongest samples with granularity of T = 2k × Tc in a window of Nt channel response samples relative to the 1st detected path.

The timing in both cases can be relative to the TOA reference time which, however, is applicable to UL-TDOA only [4]:

Agreement The FFS in RAN1#116bis agreements is addressed by updating the agreement as follows: Agreement (RAN1#116bis) For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT.

Observation 1:	According to RAN1, the "Time Domain Channel Measurement" is not used to support Multi-RTT.

Regarding the Quality Indicator for the enhanced Channel Measurement "Type B", RAN1 clarified that a single Quality Indicator for the Channel Measurement is sufficient (i.e., a per-path Quality Indicator as used for "Type A" is not needed) [4]:

Agreement The FFS in the RAN1#118bis agreement is resolved by updating the agreement as follows. Agreement (RAN1#118bis) For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information - For type (A) path-based measurement, the existing IE for timing quality is provided as in legacy signaling (i.e., per-path). - For type (B) Rel-19 enhanced measurement, one existing IE for timing quality is used to indicate the quality of the reported timing information.. Agreement The FFS in the RAN1#118bis agreement is resolved by adopting the following: Agreement (RAN1#118bis) From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - The quality indicator of timing information reuses IE “Timing Measurement Quality” in 38.455. - For type (A) path-based measurement, the IE “Timing Measurement Quality” is provided as in legacy signaling (i.e., per-path). - For type (B) Rel-19 enhanced measurement, one IE “Timing Measurement Quality” is used to indicate the quality of the reported timing information. - No separate quality indicator for power information is introduced if the channel measurement includes power information in addition to timing information. - (Mandatory) Time stamp of the channel measurement..

Observation 2:	For both, data collection and model inference, a per-path Quality Indicator for the Channel Measurement "Type B" is not needed.

The current version of the NRPPa baseline CR [1] seems in agreement with Observations 1 and 2 above.

Proposal 8:	The "FFS" in the NRPPa baseline CR for the TRP Measurement Type 'sample-based UL-RTOA(FFS))' and IE Sample-based UL-RTOA (9.2.X) can be removed.

# 3.	Summary

In this contribution, we discussed the Open Issues for the AI/ML positioning accuracy enhancements for network-based positioning. The following Observations and Proposals were made.

Inferred measurement results and request

Proposal 1:	The "LoS/NLoS indicator" can also be a gNB inferred measurement.

Proposal 2:	In a POSITIONING DATA COLLECTION REPORT message, a LMF can provide the LoS/NLoS label information independently of any timing information label. The current semantics description in IE Positioning Data Information (9.2.Z2 [1]) is removed.

Proposal 3:	It should be possible for an LMF to specifically request for AI/ML inferred measurements from TRPs.

Requested Positioning Data label information ("Part B")

Proposal 4:	It should be possible for an LMF to optionally provide the UE location in a POSITIONING DATA COLLECTION REPORT to calculate the intermediate "ground-truth measurement" label at the gNB.

Proposal 5:	Add an "LCS Privacy Check" step to the data collection procedure for Case 3a before providing the UE location to the NG-RAN. A TP is proposed in [9].

POSITIONING DATA COLLECTION REPORT: Indication of "Positioning Data Unavailable"

Proposal 6a:	The Positioning Data Unavailable IE should at least indicate whether the requested data are temporarily unavailable (e.g., due to positioning method failure) or permanently unavailable for this UE (e.g., due to UE privacy restriction).

Proposal 6b:	Add the following error causes to the Positioning Data Unavailable IE:

-	Positioning Data Information Not Supported

-	Positioning Data Information Supported But Currently Not Available

-	Not Provided Positioning Data Information Not Supported

Proactive data collection solution

Proposal 7:	RAN3 agree to complete the "proactive data collection" solution for Case 3a as a TEI20 item and, when completed, ask SA2 via LS to align the SA2 specifications with the RAN3 solution.

TRP "Channel Measurement" information

Proposal 8:	The "FFS" in the NRPPa baseline CR for the TRP Measurement Type 'sample-based UL-RTOA(FFS))' and IE Sample-based UL-RTOA (9.2.X) can be removed.

# References

[1]			R3-253980, "(BL CR to 38.455) Introduction of AI/ML air", Ericsson, et al.

[2]			RP-240499, "Status Report to TSG", RAN#103.

[3]			RP-242559, "Status Report to TSG", RAN#106.

[4]			RP-251185, "Status Report to TSG", RAN#108.

[5]			R3-255009 (R1-2505073), "LS on Rel-19 AI/ML positioning higher layer parameters list Post RAN1#121".

[6]			3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[7]			R3-255023 (S2-2505943), "Reply LS on AI/ML Positioning Case 3a", SA2.

[8]			R3-253977, Baseline CR for TS 38.305: "Introduction of AIML air", CATT, et al.

[9]			R3-255161, "TP to BL CR for TS 38.305", Qualcomm Incorporated.

[10]		RP-251186, "Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface".

[11]		RP-251870, "New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements".