3GPP TSG-RAN WG3 Meeting #129	R3-255161

Bengaluru, India, 25 – 29 August 2025

Agenda item:	20.2

Source: 	Qualcomm Incorporated

Title: 	TP to BL CR for TS 38.305

Document for:		Discussion and Decision

# 1.	Introduction

In this contribution, we propose a TP to the baseline CR for TS 38.305 [1] to complete the Stage 2 description for NG-RAN node assisted AI/ML positioning accuracy enhancements.

# 2.	Discussion

The current version of the 38.305 baseline CR [1] includes in Clause 7.x.2 (Positioning Data Collection Procedure) two Editor's Notes:

Editor’s Note: The texts above could be further refined, if needed. Editor’s Note: FFS on details of the signallings and IEs.

The procedure description in Clause 7.x.2 is generally aligned with the current NRPPa baseline [2]. However, as discussed in our companion contribution [3], a "LCS Privacy Check" step is missing as also indicated in the response LS from SA2 [4]:

SA2 is currently discussing for data collection for training models to be used by the LMF for positioning (RAN case 3b) whether a user consent check and/or a LCS privacy profile check is required. SA2 is anticipating that similar checks could also be required for case 3a (both for solution 1 and solution 2).

Proposal 1:	Add an "LCS Privacy Check" step to the data collection procedure for Case 3a before providing the UE location information to the NG-RAN. The Editor's Notes in clause 7.x.2 can be removed.

The Tables 8.10.2.3-3 and 8.13.2.2-1 (Measurement results that may be transferred from gNBs to the LMF) currently have an "FFS" for the inferred LoS/NLoS information. However, the following general agreement was made at RAN1#116 [6]:

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

and RAN1#118bis [7]:

Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

and RAN1#119 [7]:

Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements

and RAN1#120-bis [8]:

Conclusion For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

and RAN1#121 [8]

For Case 3a, the FFS in RAN1#118bis agreement is resolved by updating the agreement as follows. Agreement (RAN1#118bis) From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. - FFS: LOS/NLOS indicator. - (Optional) LOS/NLOS indicator with legacy format Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Note: It is up to RAN3 to decide whether the field of LOS/NLOS indicator should be removed or kept..

Therefore, from the above set of agreements the LoS/NLoS indicator can be an inferred measurement for AI/ML assisted positioning Case 3a and a LMF can provide the "ground-truth label" for this measurement to the gNB for training and performance monitoring [5]. It has also been clarified that the LoS/NLoS indicator can’t be reported independently from other measurements (same as in current Releases for non-inferred measurements).

Proposal 2:	The FFS in Tables 8.10.2.3-3 and 8.13.2.2-1 regarding the inferred LoS/NLoS indicator can be removed.

Clauses 8.10 (Multi-RTT) and 8.13 (UL-TDOA) also include sub-clauses for "Information that may be transferred from the LMF to gNBs". These subclauses should also describe the information provided via the new POSITIONING DATA COLLECTION REPORT message (i.e., the "ground-truth label" information).

Proposal 3:	The sub-clauses 8.10.2.4 and 8.13.2.3 ("Information that may be transferred from the LMF to gNBs") should also include the information transferred via the new POSITIONING DATA COLLECTION REPORT message (i.e., the "ground-truth label" information).

Clause 8.10.3.2 (Multi-RTT) and 8.13.3.2 (UL-TDOA) describe all individual procedures between LMF and gNB for Multi-RTT and UL-TDOA positioning, respectively. These clauses should also describe the new Positioning Data Collection Information Transfer procedure.

Proposal 4: 	Add a description of the new Positioning Data Collection Information Transfer procedure under sub-clauses 8.10.3.2 and 8.13.3.2.

For UL-TDOA, the measurement results that may be transferred from gNBs to the LMF include the "Time domain channel measurements" with FFS in Table 8.13.2.2-1. However, there are various RAN1 agreements that the "Time domain channel measurements" for Case 3a include the (existing) "path-based measurement" and an additional "sample-based measurement". For example, RAN1#120 [9]:

Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only.

Therefore, the Table 8.13.2.2-1 (Measurement results that may be transferred from gNBs to the LMF) should include both variants of the "Time domain channel measurements".

Proposal 5:	Table 8.13.2.2-1 (Measurement results that may be transferred from gNBs to the LMF) should include both variants of the "Time domain channel measurements" (i.e., path-based and enhanced sample-based) and the FFS can be removed.

# 3.	Summary

In this contribution, we propose a TP to the baseline CR for TS 38.305 [1] to complete the Stage 2 description for NG-RAN node assisted AI/ML positioning accuracy enhancements. The following Proposals were made.

Proposal 1:	Add an "LCS Privacy Check" step to the data collection procedure for Case 3a before providing the UE location information to the NG-RAN. The Editor's Notes in clause 7.x.2 can be removed.

Proposal 2:	The FFS in Tables 8.10.2.3-3 and 8.13.2.2-1 regarding the inferred LoS/NLoS indicator can be removed.

Proposal 3:	The sub-clauses 8.10.2.4 and 8.13.2.3 ("Information that may be transferred from the LMF to gNBs") should also include the information transferred via the new POSITIONING DATA COLLECTION REPORT message (i.e., the "ground-truth label" information).

Proposal 4: 	Add a description of the new Positioning Data Collection Information Transfer procedure under sub-clauses 8.10.3.2 and 8.13.3.2.

Proposal 5:	Table 8.13.2.2-1 (Measurement results that may be transferred from gNBs to the LMF) should include both variants of the "Time domain channel measurements" (i.e., path-based and enhanced sample-based) and the FFS can be removed.

A TP according to Proposals 1-5 above is proposed in the Annex of this contribution,

# References

[1]		R3-253977, Baseline CR for TS 38.305: "Introduction of AIML air", CATT, et al.

[2]		R3-253980, "(BL CR to 38.455) Introduction of AI/ML air", Ericsson, et al.

[3]		R3-255160, "Views on the Open Issues for AI/ML Positioning Accuracy Enhancements", Qualcomm Incorporated.

[4]		R3-255023 (S2-2505943), "Reply LS on AI/ML Positioning Case 3a", SA2.

[5]		R3-255009 (R1-2505073), "LS on Rel-19 AI/ML positioning higher layer parameters list Post RAN1#121".

[6]		RP-240499, "Status Report to TSG", RAN#103.

[7]		RP-242559, "Status Report to TSG", RAN#106.

[8]		RP-251185, "Status Report to TSG", RAN#108.

[9]		RP-250310, "Status Report to TSG", RAN#107.

# Annex: TP to BL CR for TS 38.305

## 7.x	AI/ML assisted positioning with gNB side model

### 7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML model.

### 7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to UL related positioning procedures described in Clause 8.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clause 8.

4.	The LMF checks the LCS privacy profile for the target UE.

5.	If the UE LCS Privacy Check from Step 4 allows the delivery of the UE location information to the NG-RAN, the LMF sends a NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated in step 3 that positioning data collection is needed. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

## 8.10	Multi-RTT positioning

#### […]

#### 8.10.2.3	Information that may be transferred from the gNB to LMF

[…]

The measurement results that may be signalled from gNBs to the LMF is listed in Table 8.10.2.3-3.

Table 8.10.2.3-3: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

gNB Rx-Tx time difference measurement NOTE 2

UL-SRS-RSRP

UL-SRS-RSRPP

UL-RSCP measurement

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type

Time stamp of the measurement

Quality for each measurement

Beam Information of the measurement

LoS/NLoS information for each measurement NOTE 2

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be inferred by using a trained AI/ML model hosted by the NG-RAN node,

[…]

#### 8.10.2.4	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.10.2.4-1.

Table 8.10.2.4-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Number of requested SRS resource sets and SRS resources per set

Pathloss reference: - PCI, SSB Index - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

Periodicity of the SRS for each SRS resource set

SSB Information

Carrier frequency of SRS transmission bandwidth

Bandwidth aggregation request indication

Positioning validity area cell list

Validity area specific SRS information

The TRP measurement request information that may be signalled from the LMF to the gNBs is listed in Table 8.10.2.4-2.

Table 8.10.2.4-2: TRP Measurement request information that may be transferred from LMF to gNBs.

Information

TRP ID, and NCGI of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity and amount

Measurement beam information request

Search window information

Expected UL AoA/ZoA and uncertainty range

Number of TRP Rx TEGs

Number of TRP RxTx TEGs

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

Time window information for measurements

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.10.2.4-3.

Table 8.10.2.4-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS - Aperiodic SRS Resource Trigger List - Activation Time

UL-SRS: - Release all

The positioning information for data collection that may be signalled from the LMF to the gNB is listed in Table 8.10.2.4-4.

Table 8.10.2.4-4: Positioning information for data collection that may be transferred from LMF to gNB.

Information

TRP ID for which the ground-truth label data are provided

gNB Rx-Tx time difference ground-truth label

LoS/NLoS information ground-truth label

Time Stamp when the ground-truth label data are valid

Quality information of the ground-truth label data

[…]

#### 8.10.3.2	Procedures between LMF and gNB

[…]

##### 8.10.3.2.2	Location Information Transfer/Assistance Data Transfer Procedure

The purpose of this procedure is to enable the LMF to request position measurements from a gNB for position calculation of the UE and also provide necessary assistance data to the gNB.

Figure 8.10.3.2.2-1 shows the messaging between the LMF and the gNB to perform this procedure.

Figure 8.10.3.2.2-1: LMF-initiated Location Information Transfer Procedure

(1)	The LMF sends a NRPPa message to the selected gNB to request Multi-RTT measurement information. The message includes any information required for the gNB to perform the measurements as defined in Table 8.10.2.4-2.

(2)	If the report characteristics in step 1 is set to "on demand", the gNB obtains the requested Multi-RTT measurements and returns them in a Measurement Response message to the LMF. The Measurement Response message includes the obtained Multi-RTT measurements as defined in Table 8.10.2.3-3.

If the report characteristics in step 1 is set to "periodic", the gNB replies with a Measurement Response message without including any measurements in the message. The gNB then periodically initiates the Measurement Report procedure in step 3 for the Multi-RTT measurements, with the requested reporting periodicity.

If the gNB is not able to accept the Measurement Request message in step 1, the gNB returns a failure message indicating the cause of the failure.

NOTE:	The Measurement Response and Measurement Report message may also include an indication that positioning data collection is needed as described in clause 7.x.2.

(3)	The gNB periodically provides the Multi-RTT measurements as defined in Table 8.10.2.3-3. to the LMF if that was requested at step 1.

(4)	At any time after step 2, the LMF may send a Measurement Update message to the gNB providing updated information required for the gNB to perform the Multi-RTT measurements as defined in Table 8.10.2.4-2. Upon receiving the message, the gNB overwrites the previously received measurement configuration information.

(5)	If the previously requested Multi-RTT measurements can no longer be reported, the gNB notifies the LMF by sending a Measurement Failure Indication message.

(6)	When the LMF wants to abort an ongoing Multi-RTT measurement it sends a Measurement Abort message to the gNB.

[…]

##### 8.10.3.2.4	Positioning Data Collection Information Transfer Procedure

The purpose of this procedure is to enable the LMF to provide positioning information to the gNB to facilitate AI/ML model training and performance monitoring at the gNB.

Figure 8.10.3.2.4-1 shows the messaging between the LMF and the gNB to perform this procedure.

Figure 8.10.3.2.4-1: Positioning Data Collection Information Transfer Procedure.

- The LMF sends the NRPPa Positioning Data Collection Report message to the gNB which previously requested data collection in a NRPPa Measurement Response or Measurement Report message as described in clause 8.10.3.2.2. The message contains the ground-truth label information as listed in Table 8.10.2.4-4. If the LMF is not able to provide the data requested, the LMF provides an error reason in the NRPPa Positioning Data Collection Report message.

[…]

## 8.13	UL-TDOA positioning

[…]

#### 8.13.2.2	Location Information that may be transferred from the gNBs to LMF

The information that may be transferred from gNBs to the LMF include measurement results listed in Table 8.13.2.2-1. The individual measurements are defined in TS 38.215 [37].

Table 8.13.2.2-1: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

UL-RTOA NOTE 2

UL-SRS-RSRP

UL-SRS-RSRPP

UL-RSCP measurement

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type

Time stamp of the measurement

Quality for each measurement

Beam Information for each measurement

LoS/NLoS information for each measurement NOTE 2

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

Time domain channel measurements (additional-path list or UL SRS time domain channel timing/time domain channel power)NOTE 3

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be inferred by using a trained AI/ML model hosted by the NG-RAN node. NOTE 3: These measurements may also be used by an LMF to directly infer the UE location using a trained AI/ML model hosted by the LMF.

#### 8.13.2.3	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-1.

Table 8.13.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

SSB Information

Periodicity of the SRS for each SRS resource set

Carrier frequency of SRS transmission bandwidth

Bandwidth aggregation request indication

Positioning validity area cell list

Validity area specific SRS information

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.13.2.3-2.

Table 8.13.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID, cell ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity and amount

Measurement beam information request

Search window information

Expected UL AoA/ZoA and uncertainty range

Number of TRP Rx TEGs

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

Time window information for measurements

Information on time window size and number of channel response samples for UL SRS time domain channel timing/time domain channel power measurements

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-3.

Table 8.13.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger List - Activation time

UL-SRS: - Release all

The positioning information for data collection that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-4.

Table 8.13.2.3-4: Positioning information for data collection that may be transferred from LMF to gNB.

Information

TRP ID for which the ground-truth label data are provided

UL-RTOA ground-truth label

LoS/NLoS information ground-truth label

Time Stamp when the ground-truth label data are valid

Quality information of the ground-truth label data

[…]

#### 8.13.3.3	Location Information Transfer/Assistance Data Transfer Procedure

The purpose of this procedure is to enable the LMF to request position measurements from a gNB for position calculation of the UE and also provide necessary assistance data to the gNB.

Figure 8.13.3.3-1 shows the messaging between the LMF and the gNB to perform this procedure.

Figure 8.13.3.3-1: LMF-initiated Location Information Transfer Procedure

(1)	The LMF sends a NRPPa message to the selected gNB to request UL-TDOA measurement information. The message includes any information required for the gNB to perform the measurements as defined in Table 8.13.2.3-2.

(2)	If the report characteristics in step 1 is set to "on demand", the gNB obtains the requested UL-TDOA measurements and returns them in a Measurement Response message to the LMF. The Measurement Response message includes the obtained UL-TDOA measurements as defined in Table 8.13.2.2-1.

If the report characteristics in step 1 is set to "periodic", the gNB replies with a Measurement Response message without including any measurements in the message. The gNB then periodically initiates the Measurement Report procedure in step 3 for the UL-TDOA measurements, with the requested reporting periodicity.

If the gNB is not able to accept the Measurement Request message in step 1, the gNB returns a failure message indicating the cause of the failure.

NOTE:	The Measurement Response and Measurement Report message may also include an indication that positioning data collection is needed as described in clause 7.x.2.

(3)	The gNB periodically provides the UL-TDOA measurements as defined in Table 8.13.2.2-1 to the LMF if that was requested at step 1.

(4)	At any time after step 2, the LMF may send a Measurement Update message to the gNB providing updated information required for the gNB to perform the UL-TDOA measurements as defined in Table 8.13.2.3-2. Upon receiving the message, the gNB overwrites the previously received measurement configuration information.

(5)	If the previously requested UL-TDOA measurements can no longer be reported, the gNB notifies the LMF by sending a Measurement Failure Indication message.

(6)	When the LMF wants to abort an ongoing UL-TDOA measurement it sends a Measurement Abort message to the gNB.

[…]

##### 8.13.3.3b	Positioning Data Collection Information Transfer Procedure

The purpose of this procedure is to enable the LMF to provide positioning information to the gNB to facilitate AI/ML model training and performance monitoring at the gNB.

Figure 8.13.3.3b-1 shows the messaging between the LMF and the gNB to perform this procedure.

Figure 8.13.3.3b-1: Positioning Data Collection Information Transfer Procedure.

- The LMF sends the NRPPa Positioning Data Collection Report message to the gNB which previously requested data collection in a NRPPa Measurement Response or Measurement Report message as described in clause 8.13.3.3. The message contains the ground-truth label information as listed in Table 8.13.2.3-4. If the LMF is not able to provide the data requested, the LMF provides an error reason in the NRPPa Positioning Data Collection Report message.

[…]