3GPP TSG-RAN WG3 Meeting #129	R3-255163

Bengaluru, India, 25 – 29 August 2025

Agenda item:	12.3

Source:	Nokia

Title:	[TP for BL CR NR Femto TS 38.413] Paging issue with NR Femtos

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#125 the study item of NR Femto was successfully completed and the outcome is visible in TR 38.799 which was agreed in R3-244849.

At the following RAN Plenary#105, the corresponding work item was agreed in [1] including the following objective:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

As part of the remaining open points of the work item (see tdoc [2]), this paper investigates the specific paging issue due to limited Xn connectivity between NR Femtos.

# 2	Paging NR Femtos with limited Xn connectivity

NR Femtos will have very limited coverage. This creates special challenges with paging function because even if an end user is paged by an NR Femto near the last serving NR Femto where he made the last call these two new NR Femtos are likely to not be Xn connected unless they are immediate neighbors. Indeed, having Xn interface with neighbors of neighbors would lead to too many Xn connections.

As a result, a typical paging from AMF would first page in last serving NR Femto and because the end user is not there would escalade the paging at TA or RA level to a much bigger areas involving tens to hundreds of cells.

Observation 1: upon paging failure to last served NR Femto the paging escalation to TA or RA could lead to paging hundreds of NR Femtos in the area whereas the end user is likely near the last serving NR femto cell.

Proposal 1: RAN3 is encouraged to solve the paging issue above in the context of NR Femtos.

Possible solutions could be:

Option 1: introduce a second list of recommended gNbs for paging for second paging

This second list of recommended gNBs for paging can be sent by the last serving NR Femto in the NGAP UE Context Release Complete message together with current list of recommended cells and gNBs. This list contains the list of NR Femtos neighboring the last serving femto as configured in the last serving femto even if no Xn interface exists between them. The AMF may use this second list after failing the first paging to last serving NR femto in the second paging (first paging repetition).

Option 2: introduce a new NR Femto area ID

Each NR Femto can be configured in advance to belong to an NR Femto Area and associated with an NR Femto area ID. The NR Femto Area ID is sent by the NR Femto to the connected NR Femto GW or 	AMF at NG Setup Request.

This NR Femto Area ID is similarly included in the NGAP UE context release complete message by the last serving NR Femto. When AMF pages the last serving NR Femto and it fails two cases:

- If the NR femto is directly connected to AMF: AMF sends second paging message to all NR femtos which belong to the same NR Femto Area ID (which it received in NG setup request).

- If the NR Femto is connected to the GW: the AMF sends the second paging message including the NR Femto Area ID. The Femto GW relays the paging message to all NR Femtos which belong to this NR Femto area ID (which it received in NG setup request).

Proposal 2: RAN3 to discuss possible options above to solve the paging issue. TPs for possible solutions are proposed in annex A.

# 3	Conclusion

This paper has explored the remaining open points and made following proposals:

Proposal 1: RAN3 is encouraged to solve the paging issue above in the context of NR Femtos.

Proposal 2: RAN3 to discuss possible options above to solve the paging issue. TPs for possible solutions are proposed in annex A.

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-255162, [TP for BL CR NR Femto TS 38.413] Completion of open points of NR Femto

# 4	TP for TS 38.413 (option 1)

#### 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the NG-RAN node to confirm the release of the UE-associated logical NG-connection over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |
| Information on Recommended Cells and RAN Nodes for Paging | O |  | 9.3.1.100 |  | YES | ignore |
| PDU Session Resource List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session Resource Release Response Transfer | O |  | OCTET STRING | Containing the PDU Session Resource Release Response Transfer IE specified in subclause 9.3.4.21. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | ignore |
| Information on Neighbors | O |  | Neighbor List 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.69	Assistance Data for Paging

This IE provides assistance information for paging optimisation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Assistance Data for Recommended Cells | O |  | 9.3.1.70 |  | - |  |
| Paging Attempt Information | O |  | 9.3.1.72 |  | - |  |
| NPN Paging Assistance Information | O |  | 9.3.1.183 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | ignore |
| Neighbor Assistance Information | O |  | Neighbor List 9.3.1.x |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	Neighbor List

This IE contains information on neighbor gNBs to be used e.g. for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbor List |  | 1 |  |  |
| >>Global RAN Node ID | M | 1..<maxnoofgNBs> | 9.3.1.5 |  |

# 4	TP for TS 38.413 (option 2)

#### 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the NG-RAN node to confirm the release of the UE-associated logical NG-connection over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |
| Information on Recommended Cells and RAN Nodes for Paging | O |  | 9.3.1.100 |  | YES | ignore |
| PDU Session Resource List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session Resource Release Response Transfer | O |  | OCTET STRING | Containing the PDU Session Resource Release Response Transfer IE specified in subclause 9.3.4.21. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | ignore |
| Femto Area ID | O |  | Femto Area ID 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.6.1	NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| Femto Area ID |  | 0..1 | 9.3.1.x |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.69	Assistance Data for Paging

This IE provides assistance information for paging optimisation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Assistance Data for Recommended Cells | O |  | 9.3.1.70 |  | - |  |
| Paging Attempt Information | O |  | 9.3.1.72 |  | - |  |
| NPN Paging Assistance Information | O |  | 9.3.1.183 |  | YES | ignore |
| Paging Assistance Data for CE Capable UE | O |  | 9.3.1.141 |  | YES | ignore |
| Neighbor Assistance Information | O |  | Femto Area ID 9.3.1.x |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	Femto Area ID

This IE contains information on area to be used e.g. for paging.

This IE is transparent to the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Femto Area ID | M |  | INTEGER (0..65535) |  |