3GPP TSG-RAN WG3 Meeting #129	R3-255170

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	11.2

Source:	ZTE Corporation

Title:	Remaining issues in AI/ML assisted Network Slicing

Document for:	Discussions & Approval

- Introduction

During the last meeting, we received the LS from SA5 regarding feedback on UL/DL throughputs at the UE level, as well as the LS on how SA5 defines UL/DL packet loss at the PDCP level. Therefore, we agreed to introduce the Average Slice Packet Loss UL IE in the Slice-based UE Performance IE. However, some issues still remain in AI/ML-assisted Network Slicing, such as the semantic description of the Average Slice Packet Loss UL/DL IE, as well as the range of the Average Slice Packet Loss UL/DL.

As this is the final meeting for the Rel-19 AI/ML for NG-RAN topic, in this contribution, we present the solution to address the remaining issues in AI/ML-assisted Network Slicing and remove the relevant FFS.

- Discussion

2.1 Packet Loss per S-NSSAI

9.2.3.x4	Slice Based UE Performance

This IE represents UE performance metrics per S-NSSAI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Slice UE Throughput DL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput DL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.1. |
| Average Slice UE Throughput UL | O |  | Bit Rate 9.2.3.4 | Corresponds to Average UE Throughput UL per S-NSSAI as specified in TS 28.558 [XX] clause 6.3.1.4.2. |
| Average Slice Packet Delay | O |  | Average Packet Delay 9.2.3.187 | Corresponds to the Average Packet Delay per S-NSSAI as specified in TS 28.558 [XX], clause 6.3.1.1. |
| Average Slice Packet Loss DL | O |  | INTEGER (0..10000000, ...) [FFS on the range] | Corresponds to DL PDCP SDU Drop Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.2.1. |
| Average Slice Packet Loss UL | O |  | INTEGER (0..10000000, ...) [FFS on the range] | Corresponds to • UL PDCP SDU Loss Rate per S-NSSAI as specified in TS 28.552 [XX] clause 5.1.3.1.1. |

Editor’s note: the semantics description of Average Slice Packet Loss to be further checked based on potential LS from SA5.

Following is the current situation for UL/DL packet loss per S-NSSAI in the BLCR:

Regarding the semantic description of packet loss per S-NSSAI, we previously sent the LS (R3-253816) [2] to SA5 in the last meeting. RAN3 would like to further inquire whether it is feasible to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity at the UL and DL levels at the PDCP layer, which can also be aggregated to reflect a per-UE Packet Loss Rate metric. If SA5 confirms that Packet Loss Rate metrics at the per-DRB per-UE level can be specified and informs RAN3, then packet loss at the slice level (UL/DL) can also be derived through aggregation of these per-DRB metrics, using a method consistent with the UE throughput aggregation.

Because the semantic description of packet loss per S-NSSAI at UL and DL is pending clarification from SA5, we propose two actions to resolve the issue:

- If SA5 provides the necessary clarification in time, incorporate their input to finalize the packet loss description in TS 38.423.

- If SA5 does not respond immediately, RAN3 has two options:

  - Remove the packet loss per S-NSSAI from the current discussion and plan to address it in Rel-20 through TEI correction, along with related issues from Rel-18 and Rel-19.

  - Retain the packet loss per S-NSSAI but keep its semantic description general until SA5 provides the required details. This would mean removing specific references and simply noting that this IE corresponds to either DL PDCP SDU Drop Rate per S-NSSAI or UL PDCP SDU Loss Rate per S-NSSAI.

Proposal 1a	If SA5 provides the necessary clarification in time, incorporate their input to finalize the packet loss description in TS 38.423.

Proposal 1b	If SA5 does not respond immediately, retain the packet loss per S-NSSAI but keep its semantic description general until SA5 provides the required details.

5.1.3.1.1	UL PDCP SDU Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface.  Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered.  The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC), and subcounters per supported S-NSSAI.

b)	SI.

c)	This measurement is obtained as:  1000000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

5.1.3.2.1	DL PDCP SDU Drop rate in gNB-CU-UP

a)	This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC), and subcounters per supported S-NSSAI.

NOTE:	this measurement may include packets that were supposed to be sent via the eUtran air interface if using NR split bearer option 3, 4 or 7.

b)	SI.

c)	This measurement is obtained as: 1000000*Number of dropped DL PDCP SDU packets whose contexts are removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface, of a data radio bearer, divided by Number of DL PDCP SDU packets for data radio bearers that have entered PDCP-SAP after being decoded from GTP-U packets. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

Regarding the range of packet loss UL/DL per S-NSSAI, the same issue also exists in the Rel-18 correction. A more granular value range is needed to accurately represent packet loss, rather than the legacy range of from 0 to 100. In 28.552 [3], both UL PDCP SDU Loss Rate and DL PDCP SDU Drop Rate are described as each measurement being an integer value representing the drop rate multiplied by 1E6. Therefore, we believe that using the range from 0 to 1,000,000 is more appropriate and aligns with SA5's definition.

Proposal 2	Using the range (0…1000000) to represent the Average Slice Packet Loss DL IE and Average Slice Packet Loss UL IE.

2.2 Other issue

Since Average Slice Packet Loss DL IE and Average Slice Packet Loss UL IE can be reported separately, unlike  packet delay UL and DL which are reported together, DATA COLLECTION REQUEST message should be further extended to include a request indicator for UL packet loss per slice.

Proposal 3	Introduce the fifth bit = Slice Average Packet Loss UL in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message.

Similarly, the new bit Slice Average Packet Loss UL should also be included in the Node Measurement Failed Report Characteristics IE in the DATA COLLECTION RESPONSE message.

Proposal 4	Introduce the eleventh bit = Slice Average Packet Loss UL in the Node Measurement Failed Report Characteristics IE in the DATA COLLECTION RESPONSE message.

3	Conclusion

Following is our proposals and observations:

Proposal 1a	If SA5 provides the necessary clarification in time, incorporate their input to finalize the packet loss description in TS 38.423.

Proposal 1b	If SA5 does not respond immediately, retain the packet loss per S-NSSAI but keep its semantic description general until SA5 provides the required details.

Proposal 3	Introduce the fifth bit = Slice Average Packet Loss UL in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message.

Proposal 4	Introduce the eleventh bit = Slice Average Packet Loss UL in the Node Measurement Failed Report Characteristics IE in the DATA COLLECTION RESPONSE message.

4	Reference

[1] R3-253023, Reply LS on per-UE UE performance metrics, SA5

[2]	R3-253824, Reply LS on clarification on UE performance measurement at PDCP level, SA5

[3] R3-253816, Reply LS on per-UE UE performance metrics, RAN3

[4] TS28.552

[5] R3-255171 (TP to BL CR to 38.423) Support of AI/ML assisted Network Slicing, ZTE Corporation, Qualcomm, China Unicom