3GPP TSG-RAN WG3 Meeting #129	R3-255174

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	Way forward on Continuous MDT collection

Document for:	Discussions & Approval

- Introduction

During the last meeting, RAN3 reached some agreements on continuous MDT collection:

- Information from OAM to the NG-RAN nodes participating to Continuous MDT, allowing for the identification of a continuous MDT process, are provided e.g. in the form of specific Trace Reference(s).

- Solutions for Continuous MDT shall not have an impact on the 5GC in Rel19 from a RAN3 perspective.

- RAN3 agrees to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem.

Additionally, RAN3 has concluded several solutions on trace correlation. RAN3 informed SA5 of the agreed principles and these candidate solutions, and asked SA5 for feedback [1]. Because this meeting is the last one for Rel-19 “AI/ML for NG-RAN” topic, this contribution is provided what we consider the way forward on continuous MDT collection.

- Discussion

RAN3 concluded two aspects on continuous MDT collection to be resolved during Rel-19 WI phase, i.e., measurement continuity and trace correlation. Based on the current situation, progress on Continuous MDT collection topic depends on conclusion from SA5. We are awaiting final input or decisions from SA5 to define the next implementation steps. Because it is the last meeting for Rel-19, we need to consider how to fulfil the continuous MDT collection topic. Now based on the current progress in RAN3, we have summarized some protocol impacts that need to be specified by RAN3.

The following are the candidate solutions and agreements, along with the corresponding candidate specification impacts:

| Problem | Agreement/Candidate Solutions | Specification Impacts |
| --- | --- | --- |
| Measurement Continuity | Information from OAM to the NG-RAN nodes participating to Continuous MDT, allowing for the identification of a continuous MDT process, are provided e.g. in the form of specific Trace Reference(s). | Impacts on SA5 specification. |
|  | RAN3 agrees to use TR(s) and TRSR(s) to resolve the MDT measurement correlation problem. | Limited impacts in RAN3 specification if considering continuous MDT collection during RRC_Connected mobility. (Explicit indication, i.e., TR and TRSR should be involved in the Handover Request message to inform target node that the UE has selected for continuous MDT collection.) |
| Trace Correlation | In some solutions, a dedicated TRSR range is assigned to each NG-RAN node participating in Continuous MDT. When an NG-RAN node selects a UE for Continuous MDT (not configured so far with Continuous MDT), the NG-RAN node assigns a TRSR from such range to the UE. The same TRSR will be used for the whole duration of the Continuous MDT trace of a UE for correlation purposes. Namely, the TRSR is kept for the UE even if the UE moves to a different participating node via handover (the TR and TRSR are signalled from source NG-RAN node to target NG-RAN node) or if the UE is configured with Logged MDT and it moves to RRC_Idle/RRC_Inactive. Trace Correlation is achieved by the assigned TR/TRSR which can identify that the trace records belong to the same selected UE. Overlapping is avoided because the same TRSR from the TRSR range assigned to the NG-RAN node that initiated the Continuous MDT process will be used for the whole duration of the Continuous management-based MDT trace of a UE. Or In some other solutions, RAN3 assumes that the problem of overlapping does not occur because OAM configures gNBs in an area with same immediate MDT configuration (TR1) and same logged MDT configuration (TR2). A gNB chooses a UE for Continuous MDT and configures the UE for Immediate MDT and Logged MDT separately (with separate TR and may be also different TRSRs, i.e. TR1/TRSR1 and TR2/TRSR2). At connected-to-idle transition the serving gNB sends the Immediate MDT report (TR1/TRSR1) to TCE by also including the TR2/TRSR2 assigned for the Logged MDT. At idle-to-connected transition the (new) serving gNB sends the Logged MDT report (TR2/TRSR2) to TCE by also including the TR1/TRSR3 assigned for the next Immediate MDT. Or, In some other solutions, RAN3 assumes that the problem of overlapping does not occur because OAM configures gNBs in an area with same configuration (single TR) encompassing both immediate MDT and logged MDT. A gNB chooses a UE for Continuous MDT and configures the UE for Immediate MDT and Logged MDT with the same TR and TRSR (TRSR1). If at idle-to-connected, or at handover the new serving gNB cannot re-assign TRSR1 to the UE, it will assign a new TRSR (TRSR2) and include both the previously assigned TRSR (TRSR1) and the new TRSR (TRSR2) in the Logged MDT (TR & TRSR1/TRSR2) or Immediate MDT report (TR & TRSR1/TRSR2) to TCE. | Impacts on SA5 specification. The solutions about trace correlation are all focused on how the TCE associates measurement reports of the same UE in different states through the TR(s) and TRSR(s) in the measurement reports. No RAN3 related impacts are identified. |

Observation 1: From the above analysis, it can be seen that measurement continuity during the RRC_CONNECTED mobility scenario has the limited impact on RAN3.

In response to this, we have provided several ways forward to handle continuous MDT collection:

- Opt1: RAN3 agreed the OAM based solution in R19 as mentioned above and the normative work can be further reviewed during Rel-19 maintenance phase based on reply LS from SA5. Or

- Opt2: Continuous MDT collection can be further worked during normative phase in Rel-20 AI/ML for NG-RAN, or 6G AI discussion.

Proposal 1: Agree the first way forward proposed above to handle continuous MDT collection in R19.

3	Conclusion

Following is our observation and proposals:

Observation 1: From the above analysis, it can be seen that measurement continuity during the RRC_CONNECTED mobility scenario has the limited impact on RAN3.

Proposal 1: RAN3 agreed the OAM based solution in R19 as mentioned above and the normative work can be further reviewed during Rel-19 maintenance phase based on reply LS from SA5.

Reference

[1] R3-253958, Reply LS on Continuous MDT, RAN3