3GPP TSG-RAN WG3 Meeting #129		R3-255179

Bengaluru, India, 25th – 29th Aug, 2025

Agenda Item:	14.3

Source:	CATT, ZTE, Nokia, Nokia Shanghai Bell, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi

Title:	(TP to BL CR for TS38.300) Support of NG Suspend/Resume

Document for:	Other

- Introduction

In the last RAN3 meeting, we further discussed the issues on regenerative payload. According to the discussion, some agreements are achieved, as below:

WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure.

In this contribution, we will continue discuss the remaining issues on regenerative payload for NR NTN and work on the stage2/3 details, our views will be provided accordingly.

- Discussion

In case of NG Connection interruption due to hard feeder link switch over, the previous meeting has agreed to introduce a new IE for NG downlink transmission suspension/resumption. The corresponding description is as follows in BLCR 38.300:

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

For hard feeder link switch case (use case 1), the gNB connects with the new NTN-GW only after having disconnected from the old NTN-GW. The 5GC may not know the accurate timing of hard feeder link switch until gNB informs 5GC, thus during this period, the 5GC may continue sending the DL NG-C/U packet to the old NTN-GW, but the old NTN GW cannot deliver packets to the gNB due to the old feeder link is broken, the DL packets are lost in this case. To solve the packet loss problem, it is beneficial to introduce the new indicator to suspend or resume NG connection. Specifically, the gNB clearly informs the AMF before the hard feeder link switch, and then the AMF can suspend the NG connection and inform the corresponding SMF(s) to control the UPF(s) to suspend the DL data transmission. And when the new feeder link is activated, the gNB could notify the AMF to resume NG connection and continue the DL NG-C/U packets transmission via the new NTN-GW.

As above, for hard feeder link switch case, it is beneficial to introduce the new indicator in RAN Configuration Update to suspend or resume NG connection.

Observation 1: For hard feeder link switch case, it is beneficial to introduce the new indicator in RAN Configuration Update to suspend or resume NG connection to avoid the DL data loss.

Besides feeder link swithover case, we understand that the specified suspend/resume indicator could be generically applicable, not restricted to hard feeder link switchover case. Another case we discussed the last RAN3 meeting is to suspend/resume NG connection in case of an on-board gNB leaves/enters the AMF service area (use case 2). The on-board gNB could indicate the AMF that the current NG connection is suspended (temporarily unavailable, the logical NG interface still keeps). The gNB and AMF stores the corresponding NG context information during the NG suspension.

In previous discussion, a few satellite companies showed concerns on the limited storage space on satellites. As the on-board gNB has left the AMF service area, corresponding UEs located in the AMF service area have been handovered to the subsequent satellites, corresponding UE contexts between this on-board gNB and the AMF have been released before gNB initiate NG suspension or removal.Only the basic context of NGAP logical connection is stored in on-board gNB and the AMF during NG suspension, and the amount of data is very limted.

Figure 1. An example of an on-board gNB entering/leaving an AMF service area

As shown in the figure above, at T1, the onboard gNB serves the AMF#1 and AMF#2. With the moving of the satellite, the gNB leaves the AMF#1 service area from T2, the served UEs located in AMF#1 service area have already been handovered to the subsequent satellite(s) at T2. In antoher words, the on-board gNB will not provide any 5G services for the AMF#1 from T2. Thus, there’s no UE context need to be stored in the gNB for the AMF#1 from T2, no matter the NG connection between the gNB and AMF#1 keeps or suspended. The gNB may decide to remove/suspend the NG interface towards the AMF#1 at any time after T2 due to it leaves the AMF#1 service area. If NG connection between the gNB and AMF#1 is suspended, only the basic context of NG connection is stored in the onboard gNB.

Observation 2: No additional UE contexts stored on the on-board gNB when  it leaves an AMF service area, as the UEs located in the AMF service area have been handovered to the subsequent satellites, corresponding UE contexts have been released before gNB suspend or remove the NG connection towards an AMF.

We understand that both NG Removal/Setup and NG suspend/resume can be supported, and the two mechanisms can be used flexibly or can be used in combination, adopt NG Removal/Setup, or Suspend/Resume, or both are pending to the satellite operators. For example, if on board gNB may not return to the service area of an AMF for a long time in the future, it can choose NG Removal procedure. If on board gNB will return to the service area of an AMF in a shorter period of time, it can suspend the NG connection.

Observation 3: Support of NG Suspend/Resume provide an alternative way on NG management, which makes the NG connection establishment/recovery faster, adopt NG Removal/Setup, or Suspend/Resume, or both are pending to the satellite operators.

Base on the discussion above, the suspend/resume indication may be sent from gNB to AMF in RAN Configuration Update in case of hard feeder link switchover, or for NG connection suspension in case of on-board gNB leaves the AMF service area. Therefore, it’s proposed to turn the “WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure.” to the agreement.

Proposal 1: Turn the “WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure.” to the agreement.

Corresponding stage 2 TP for 38.300 BL CR is provided in appendix.

For the stage 3, the “Downlink NG Transmission Control” IE introduced in the RAN Configuration Update message for hard feeder link switch could be changed to a general name, e.g. “NG Connection Control”. Corresponding stage 3 TP for TS 38.413 BL CR is provided in [2].

RAN3 is requested to discuss and agree the TPs on support of NG Suspend/Resume for NR NTN.

Proposal 2: Discuss and agree the TPs on support of NG Suspend/Resume for NR NTN.

Similary, for IoT NTN, S1 Suspend/Resume should also be supported. The same design as NR NTN could be applied. Corresponding TPs to the stage 2 and stage 3 BL CR are provided in [3][4]. RAN3 is requested to discuss and agree corresponding TPs.

Proposal 3: Discuss and agree the TPs [3][4]on support of NG Suspend/Resume for IoT NTN.

- Conclusion

Based on the discussion in section 2, we have the following observations and proposals:

Observation 1: For hard feeder link switch case, it is beneficial to introduce the new indicator in RAN Configuration Update to suspend or resume NG connection to avoid the DL data loss.

Observation 2: No additional UE contexts stored on the on-board gNB when  it leaves an AMF service area, as the UEs located in the AMF service area have been handovered to the subsequent satellites, corresponding UE contexts have been released before gNB suspend or remove the NG connection towards an AMF.

Observation 3: Support of NG Suspend/Resume provide an alternative way on NG management, which makes the NG connection establishment/recovery faster, adopt NG Removal/Setup, or Suspend/Resume, or both are pending to the satellite operators.

Proposal 1: Turn the “WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure.” to the agreement.

Proposal 2: Discuss and agree the TPs on support of NG Suspend/Resume for NR NTN.

Proposal 3: Discuss and agree the TPs [3][4]on support of NG Suspend/Resume for IoT NTN.

- Reference

- RAN3_128_agenda_20250523_1530_EOM

- R3-255263 (TP to BL CR for TS 38.413) Support of NG Suspend and Resume Indicator (ZTE Corporation, CATT, Nokia, Nokia Shanghai Bell, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi)

- R3-255311(TP for TS 36.300) Support for Regenerative Payload for IoT NTN (Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, ZTE, CATT, China Telecom, CMCC, LG Electronics, Xiaomi)

- R3-255295 (TP to BL CR for TS 36.413) Support of regenerative payload (Nokia, Nokia Shanghai Bell, ZTE Corporation, CATT, Qualcomm, CMCC, Samsung, China Telecom, LG Electronics, Xiaomi)

- TP for TS38.300

START OF CHANGES

16.14.4	Switchover

16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN payloads.

16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links.

16.14.4.3	Procedures

The NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, PDU Session Resource Modify Indication procedure) may be used.

For regenerative payload, an indication to suspend or resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs, or for NG connection suspend/resume.

END OF CHANGES