3GPP TSG-RAN WG3 Meeting #129	R3-255180

Bangaluru, India, 25th – 29th August, 2025

Agenda Item:	14.3

Source:	CATT, Nokia, Nokia Shanghai Bell, ZTE, Samsung, CSCN, CMCC, China Telecom, LG Electronics, NEC, Xiaomi

Title:	(TP to BLCR for TS 38.300) OAM configuration for supported TAI list

Document for:	other

1	Introduction

In previous RAN3 meetings, its agreed that “The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration”. However, nothing has been captured in our stage 2 and stage 3 specifications. There’re some liaisons between RAN3 and SA2/SA5, the latest LS from SA2 [5] shows that no consensus to take any enhancement in their specifications.

To make it clear, we try to further clarify the relationship between OAM based solution, and signalling based solution (via legacy signalling) for supported TAI list. And we try to add some stage 2 texts into TS 38.300 to reflect our agreement that OAM based configuration could be used to provide supported TAI list to AMF.

2	Discussion

In RAN3#127 meeting, SA2 raised some concerns on OAM based solution on provision of supported TAI list to AMF in the LS [1]. RAN3 further discussed and provided the clarifications in the reply LS to SA2 [3], as highlighted below:

The existing NG signalling procedures for supported TAI list update are expected to work as legacy. However, some NTN scenarios (e.g. NGSO NTN scenarios) benefit from the RAN3 agreements on OAM based solution.

RAN3 discussed and would like to provide the following clarifications:

- In general, satellite orbital movement is periodic and predictable; therefore, any resulting change to node parameters will result in the same information being frequently exchanged over network interfaces at regular intervals.

- In the solution agreed by RAN3, both AMF and gNB share the same OAM configuration on supported TAI list of the on-board gNB, this is considered as an optimization.

- NG Setup procedure including the supported TAs is always expected to take place according to TS38.413.

- The solution agreed by RAN3 avoids the need for periodic and frequent RAN Configuration Update procedures. Any potential discrepancy between OAM configuration and parameters received via NG signalling, if present, is expected to be resolved via implementation.

- With respect to the frequency of updates by OAM, RAN3 believes this is in SA5 scope.

As highlighted above, the existing NG signalling procedures for supported TAI list update are expected to work as legacy, no enhancement is adopted. Which means it’s not precluded to provide and update the supported TAI list of an on-board gNB to AMF via legacy N2 signallings (i.e. NG Setup Request, RAN Configuration Update).

An on-board gNB should be able to know whether to update the supported TAI list via RAN Configuration Update(s) towards the AMF. Adopt OAM based solution, or signalling based solution is up to configuration (e.g. operator’s policy).

Observation 1: On provision of supported TAI list to AMF, OAM based solution and signalling based solution (legacy signalling) are both feasible, which one to be adopted is up to operator’s policy/configuration.

In the last SA2 meeting, they discussed the issue on top of RAN3 and SA5 feedback. They further replied the LS to RAN3 in [5].

SA2 discussed the solution proposed by RAN3/SA5 in their LS and did not reach consensus to introduce any enhancement to SA2’s specification.

Observation 2: SA2 did not reach consensus to introduce any enhancement to SA2’s specification.

As RAN3 and SA5 already confirmed that the OAM based solution is feasible for NTN to provide supported TAI list of the on-board gNB to AMF. We do not see any issue to support the OAM based solution without any change to SA2 specification.

Observation 3: Without SA2 change, there’s no issue to provide the supported TAIs of an on-board gNB to AMF via OAM/pre-configuration.

From RAN3 perspective, it’s beneficial to capture some stage 2 to reflect the RAN3 agreements that the supported TAIs of an on-board gNB may be provided to AMF by OAM/pre-configuration, which is missed in the current BL CR. The stage 2 TP is provided in the Annex, RAN3 is requested to discuss and agree the TP.

Proposal 1: RAN3 to discuss and agree the stage 2 TP in the Annex to capture the OAM based configuration for supported TAI list.

3	Conclusion

Base on the discussion above, we provide the following observations and proposal:

Observation 1: On provision of supported TAI list to AMF, OAM based solution and signalling based solution (legacy signalling) are both feasible, which one to be adopted is up to operator’s policy/configuration.

Observation 2: SA2 did not reach consensus to introduce any enhancement to SA2’s specification.

Observation 3: Without SA2 change, there’s no issue to provide the supported TAIs of an on-board gNB to AMF via OAM/pre-configuration.

Proposal 1: RAN3 to discuss and agree the stage 2 TP in the Annex to capture the OAM based configuration for supported TAI list.

4	Reference

- R3-250023, LS on reply to LS on OAM requirements to support regenerative payload, SA2(Huawei)

- R3-250030, LS reply on OAM requirements to support regenerative payload, SA5(CATT)

- R3-250895, Reply LS on OAM requirements to support regenerative payload transport links, RAN3#127

- R3-251528, Reply LS to LS on reply to LS on OAM requirements to support regenerative payload, RAN3#127b

- R3-255024, Reply LS on OAM requirements to support regenerative payload transport links, SA2

TP to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<<<< Changes begin>>>>>>>>>>>>>>>>>>>>

### 16.14.7	O&M Requirements

The following NTN related parameters shall be provided by O&M to the gNB providing NTN access:

-	Ephemeris information describing the orbital trajectory information or coordinates for the NTN payload. This information is provided on a regular basis or upon demand to the gNB;

-	Two different sets of ephemeris format shall be supported:

-	Set 1: NTN payload position and velocity state vectors:

-	Position;

-	Velocity.

-	Set 2: At least the following parameters in orbital parameter ephemeris format, as specified in NIMA TR 8350.2 [51]:

-	Semi-major axis;

-	Eccentricity;

-	Argument of periapsis;

-	Longitude of ascending node;

-	Inclination;

-	Mean anomaly at epoch time.

-	The explicit epoch time associated to ephemeris data;

-	The location of the NTN Gateways;

NOTE 1:	The ephemeris of the NTN payloads and the location of the NTN Gateways, are used at least for the Uplink timing and frequency synchronization. It may also be used for the random access and the mobility management purposes.

-	Additional information to enable gNB operation for feeder/service link switch overs.

NOTE 2:	The NTN related parameters provided by O&M to the gNB may depend on the type of supported service links, i.e., Earth-fixed, quasi-Earth-fixed, or Earth-moving.

For regenerative payload, the supported TAIs of an on-board gNB can be configured to the AMF(s) by O&M, as in TS 28.541 [49].

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>