3GPP TSG-RAN WG3 Meeting #129	R3-255182

Bangaluru, India, 25 – 29 August, 2025

Agenda Item:	16.2

Source:	CATT

Title:	(TP to BLCR for TS 38.300) Support of A-IoT

Document for:	other

1	Introduction

This contribution provides the TP to BL CR for TS 38.300, addressing the RAN3 remaining issues on support of A-IoT.

2	(TP to BL CR for TS 38.300) Support of A-IoT

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the Expected D2R message size and may include Inventory Assistance Information e.g. Approximate number of Target A-IoT devices.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 1:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices, or multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

7.	Upon completion of the inventory procedure the gNB may include an Inventory Complete Indication in the Inventory Report message.

16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 1: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices or in parallel to other command procedures targeting other A-IoT devices within the same session.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB in order to send a command to a single A-IoT device. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU corresponding to the targeted A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size, command type.

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response messageincluding the AIOTF Identifier  the Correlation ID and the RAN A-IoT device NGAP ID received in the Command Request, and the A-IoT NAS PDU received from the device.

NOTE 2:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

### 16.xx.x5	A-IoT CN initiated A-IoT Session Release procedure

Figure 16.xx.x5-1: A-IoT CN initiated A-IoT Session Release procedure

1.	The A-IoT CN node initiates the A-IoT Session Release procedure over NG-C by sending the A-IoT Session Release Command message to the gNB. The A-IoT Session Release Command message includes the Correlation ID and the AIOTF Identifier to identify the session to be released.

2.	The gNB releases the session context, the associated NG resources and the A-IoT radio resources.

3.	The gNB confirms the release to the A-IoT CN node by replying with the A-IoT Session Release Complete message. The A-IoT Session Release Complete message includes the AIOTF Identifier and the Correlation ID received in the A-IoT Session Release Command message.

### 16.xx.x6	gNB initiated A-IoT Session Release procedure

Figure 16.xx.x6-1: gNB initiated A-IoT Session Release procedure

1.	The gNB initiates the A-IoT Session Release Request procedure over NG-C by sending the A-IoT Session Release Request message to the A-IoT CN node in order to request the release of an A-IoT session. The A-IoT Session Release Request message includes the Correlation ID and the AIOTF Identifier to identify the session requested to be released.

2.	The A-IoT CN node accepts the request and triggers the CN initiated A-IoT Session Release procedure.

3.	The CN initiated A-IoT Session Release procedure is performed as described in section 16.xx.x5.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x7 Locating A-IoT devices

In this version of specification, the A-IoT device location is reported by the gNB at Reader ID granularity to the AIOTF via Inventory Report procedure. The AIOTF may use the A-IoT device location information in performing A-IoT operations.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>