3GPP TSG-RAN WG3 #129	R3-255199

Bengaluru, India, 25th – 29th August 2025

Agenda item:		13.3 Support for Conditional LTM

Title:		(TP to BL CR for 38.473 on conditional intra-CU LTM) Remaining issues of Rel-19 intra-CU Conditional LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

Last RAN3#128 meeting we have reached some agreements, and baseline CR to 38.401 was endorsed and latest in R3-254008.  TP to 38.473 for intra-CU conditional LTM was also provided and agreed in R3-253960, while still some detail FFS are remaining.

In this contribution the remaining issues for detail specification are discussed.

9.3.1.XXX		L1 Execution Condition List

This IE indicates the list of conditional LTM L1 execution conditions to be used by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| L1 Execution Condition Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Execution Condition | M |  | OCTET STRING | Includes the LTM-CSI-ReportConfigId-r18 IE as defined in subclause 6.3.2 in TS 38.331 [8]. FFS:to be checked in RRC running CR. | - |  |

13. NR Mobility Enhancements WI WID [NR_Mob_Ph4-Core]: RP-250339 (target: RAN #109) [TU: 1 ( 1, 1, 0.5)] QUOTA: 3

13.3. Support for Conditional LTM From RAN3#128: RAN3 confirms that the network can trigger a LTM Cell Switch Command MAC CE towards a candidate cell with C-LTM candidate configuration. The candidate (initial source) DU need to know the L1 C-LTM and the L3 C-LTM. The DU needs to know to which other candidate cell(s) to generate the L1 execution condition for L1 C-LTM. WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”. WA: To introduce a new IE with a list of candidate cells for L1 execution condition. It is acknowledged that the C-LTM indicator needs to be sent to the initial source DU for the very first C-LTM. The candidate DU generates the TAT for its own candidate cells.

2. Discussion

2.1	Support for Conditional LTM

Most of the information for the Conditional LTM have been reflected in the baseline CR of 38.473 (latest in R3-254011 after last RAN3#128). The content of the L1 Execution Condition remains “FFS:to be checked in RRC running CR.”.

The content of the L1 Execution Condition Information provided by the DU, following what RAN2 has defined, is to be the LTM-CSI-ReportConfigId-r18 i.e. the ID corresponds to the LTM-CSI-ReportConfig-r18, which will then need CU to set in part of the newly defined LTM-ExecutionCondition-r19 in the RRCReconfiguration message to be sent to the UE.

From the latest RRC running CR (after RAN2#130):

– LTM-ExecutionConditionList The IE LTM-ExecutionConditionList is used to configure LTM cell switch conditions. LTM-ExecutionConditionList information element -- ASN1START -- TAG-LTM-EXECUTIONCONDITIONLIST-START LTM-ExecutionConditionList-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-ExecutionCondition-r19 LTM-ExecutionCondition-r19 ::= SEQUENCE { ltm-CandidateId-r19 LTM-CandidateId-r18, executionCondition-r19 CHOICE { l1-Conditions-r19 LTM-CSI-ReportConfigId-r18, l3-Conditions-r19 SEQUENCE (SIZE (1..2)) OF MeasId } OPTIONAL, -- Need R ... } -- TAG-LTM-EXECUTIONCONDITIONLIST-STOP -- ASN1STOP

So the content of the L1 Execution Condition Information IE in F1AP, follow the RAN2 definition, has a list of cell and will be each ID (LTM-CSI-ReportConfigId-r18) correspond to each cell. This is already in the baseline CR of 38.473 (latest in R3-254011 after last RAN3#128), then only need to remove the FFS.

Proposal 1: To specify In F1 interface the L1 Execution Condition Information IE provided by the DU to CU, a list of cell and correspond to each cell the LTM-CSI-Report-Config-ID as OCTET STRING (includes the LTM-CSI-ReportConfigId-r18 as defined in subclause 6.3.3 in TS 38.331), this is already in the baseline CR of 38.473 (latest in R3-254011 after last RAN3#128), then only need to remove the FFS.

3. Conclusion and proposals

Proposal 1: To specify In F1 interface the L1 Execution Condition Information IE provided by the DU to CU, a list of cell and correspond to each cell the LTM-CSI-Report-Config-ID as OCTET STRING (includes the LTM-CSI-ReportConfigId-r18 as defined in subclause 6.3.3 in TS 38.331), this is already in the baseline CR of 38.473 (latest in R3-254011 after last RAN3#128), then only need to remove the FFS.

Proposal 2: There are two similar IEs in the baseline CR to 38.473 (R3-254011->R3-255065):

“9.3.1.XXX	L1 Execution Condition List” and

“9.3.1.XXX	Conditional LTM Execution Condition List”

Need to remove the “9.3.1.XXX	Conditional LTM Execution Condition List”.

All above proposals their TP are provided.

# References

[1] RP-250339 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-253788 Summary of Offline Discussion on intra-CU Conditional LTM

[3] R3-254006 -> R3-255060 (BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC

[4] R3-253085 -> R3-255057 (BL CR to 37.483) Introducing Rel-19 Mobility Enhancement

[5] R3-254007 -> R3-255061 (BL CR to 38.300) Inter-CU LTM

[6]R3-254008 -> R3-255062 (BL CR to 38.401) on conditional intra-CU LTM

[7] R3-253088 -> R3-255058 (BL CR to 38.420) Support for Inter-CU LTM

[8] R3-254009 -> R3-255063 (BL CR to 38.423) Xn Support for Inter-CU LTM

[9] R3-253091 -> R3-255059 (BL CR to 38.470) Introducing Rel-19 Mobility Enhancement

[10] R3-254010 -> R3-255064 (BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC

[11] R3-254011 -> R3-255065 (BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM

[12] R3-253009 (==R2-2503012) LS on RAN2 agreements for security key handling in inter-CU LTM

[13] R3-253895 (TP for LTM BLCR for TS38.473): Update on inter-CU LTM procedure

[14] R3-253960 (TP for LTM BLCR for TS38.473): Update on intra-CU conditional LTM

[15] R2-2504125 Running RRC CR for inter-CU and conditional LTM (RAN2#130)

[16] R2-250xxxx (For RAN2#131) Running RRC CR for inter-CU and conditional LTM

[17] Draft_RAN2_130_Meeting_Report_v2

# TP to 38.473 (based on R3-254011 -> R3-255065)

SKIP ALL UNAFFECTED PARTs.

/*********************Next change***********************/

#### 9.3.1.XX	LTM Security Information

This IE contains the security related information for LTM candidate cell(s) to support the UE in generating the key material for AS security during an inter-CU LTM cell switch.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Next Hop Chaining Count | M |  | INTEGER (0..7) | Next Hop Chaining Count (NCC) defined in TS 33.501 [Y] |
| Security Change Serving Cell Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellNoSecurityChangeID IE as defined in TS 38.331 [8], for the current serving cell. |
| Security Change Candidate Cell Information List |  | 0..1 |  |  |
| >Security Change Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>Security Change Candidate Cell Configuration | M |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

/*********************Next change***********************/

/******************Next change*******************************/

9.3.1.x1	CSI-RS Resource Configuration

This IE contains the CSI-RS resource configuration used for LTM.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI-RS Resource Configuration To AddModList | O |  | OCTET STRING | Contains the ltm-NZP-CSI-RS-ResourceToAddModList as defined in TS 38.331 [8]. |
| CSI-RS Resource Configuration To Release List | O |  | OCTET STRING | Includes the ltm-NZP-CSI-RS-ResourceToReleaseList contained in the LTM-Config IE as defined in TS 38.331 [8]. |

/******************Next change*******************************/

9.3.1.x	Request for L1 Execution Condition

This IE indicates the list of LTM candidate cells requested for generating conditional LTM L1 execution conditions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Request for L1 Execution Condition Candidate Cell List Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

/*********************Next change***********************/

9.3.1.XXX		L1 Execution Condition List

This IE indicates the list of conditional LTM L1 execution conditions to be used by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| L1 Execution Condition Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Execution Condition | M |  | OCTET STRING | Includes the LTM-CSI-ReportConfigId-r18 IE as defined in subclause 6.3.2 in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

/*********************Next change***********************/