3GPP TSG-RAN WG3 Meeting #129	R3-255200

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	20.2

Title:	[TP for BL CR to TS 38.305& TP for BL CR to TS 38.455] Discussion on AI/ML for positioning Case 3a

Source:	NEC

Document for:	Discussion and Decision

- Introduction

The work items pertaining on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface were officially approved in RAN #102 and updated in RAN #107 finally with the removal of 2nd priority cases. The details concerning the positioning use case after RAN #107 have been finalized as follows[1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

Based on the previous progress, this contribution addresses the remaining issues of Case 3a, NG-RAN node assisted positioning with gNB-side model.

- Discussion

  - Ground truth label provided by LMF to gNB

For Case 3a training data collection, the gNB can generate the measurement, while the ground truth label can be generated by at least the LMF. The gNB can request the LMF to provide the ground truth label, pair it with the measurement to form a training data element, and train the model based on the paired dataset. Regarding the transmission of the ground truth label from the LMF to the gNB, RAN3 reached consensus on this in the last meeting:

Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator. FFS if UE location can be requested.

For the FFS, RAN1#120b addressed a similar issue and reached the following agreement:

Agreement (RAN1#120bis) For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes timing information, support the following for providing label and quality indicator of label, - The corresponding label is provided by LMF to gNB in the format of timing information. “Timing Measurement Quality” in 38.455 can serve as quality indicator of the label. Note: It is assumed that user data privacy of non-PRU UE is preserved. Conclusion (RAN1#120bis) For training data collection of Part B in AI/ML based positioning Case 3a, for the case when Part B label includes the LOS/NLOS indicator, - The corresponding label is provided from LMF to the gNB by using the existing IE LOS-NLOS-Indicator-r17. There is no need of a separate field for quality indicator of LOS/NLOS indicator.

This agreement clarifies that for Case 3 training data collection, the gNB will receive timing information or an LOS/NLOS indicator, but not UE location. The primary reason for not providing UE location from the LMF to the gNB is to protect UE privacy; this sensitive information should not be directly exposed to the gNB. It is important to note that in existing positioning procedures, neither the LMF nor the UE discloses the target UE’s location to the gNB. This principle remains unchanged for AI/ML-based positioning, and duplicative discussions in RAN3 should be avoided. Therefore, the FFS should be resolved by stipulating that UE location cannot be requested.

Additionally, to prevent misinterpretation in TS38.305 BL CR[2], we propose revising the phrase "information related to UE location" to "timing information" in alignment with the aforementioned RAN1 agreement.

For data collection of Case 3a, “FFS if UE location can be requested” in RAN3#128 agreement is resolved by:

UE location cannot be requested.

In TS 38.305 BLCR, change "information related to UE location" to "timing information".

A TP for the AI/ML BLCR to TS 38.305 is provided in Annex 1.

  - LMF requested measurement

During RAN3#128, it was agreed that an indicator is needed to denote whether the gNB reported measurement was generated via AI/ML inference. It was FFS whether the LMF should specifically request AI/ML inferred measurements.

Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion FFS: if LMF should specifically request for AIML inferred measurement.

In our view, requiring the LMF to request AI/ML-inferred measurements is unnecessary. The reasons are:

- Enabling the LMF to request AI/ML-inferred measurements implies the simultaneous availability of both legacy and inferred measurements. However, there is no clear justification for maintaining both. If the AI/ML-based measurement is activated, the legacy measurement may become unreliable – precisely why AI/ML-based positioning is needed. This aligns with the WID objective of "Positioning accuracy enhancements".

- The LMF is likely aware of AI/ML model activation status, as it may manage the functionality (a similar agreement was reached for Case 1 in RAN2). Consequently, the LMF can determine whether a reported measurement originates from AI/ML inference or the legacy method.

Hance, we propose that there is no need to enable LMF request for AIML inferred measurement.

Regarding the measurement report of Case 3a, resolve the FFS item from RAN3#128 ("FFS: if LMF should specifically request for AIML inferred measurement") as follows:

It is not necessary to enable LMF to request AIML inferred measurement

  - Add “Time stamp” in NRPPa MEASUREMENT RESPONSE message

In the latest BL CR to TS 38.305[2], the following call flow was agreed as part of the AI/ML model training.

Regarding Positioning Data Collection in gNB, it was agreed that

- in step 3 the gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

- in step 4 the LMF sends a NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s).

In RAN1#120 meeting, it was agreed that time stamp of Part A (i.e., time stamp of the channel measurement) and time stamp of Part B (i.e., time stamp of ground truth label) are needed for pairing between Part A and Part B. The related agreement is:

Agreement (RAN1#120) - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above.

However, as explained above, current information transfer between gNB and LMF lacks time stamp information for both Part A and Part B. Consequently, LMF may provide improper Part B to gNB because the time stamps of Part A and Part B could be significantly mismatched. This leads to inefficient data collection, significantly impacting system performance.

Hence, to solve this problem, we propose to include a “time stamp” in the NRPPa MEASUREMENT RESPONSE message.

Furthermore, according to "RAN3 confirms the WA: For case 3a data collection, Part A is known internally by the gNB and is not signaled over any network interface." agreed in the last RAN3 meeting, gNB knows Part A (containing time stamp of measurement) internally, it's easy for gNB to provide time stamp of Part A to LMF as a "time stamp" indication. LMF can have flexibility to provide proper Part B by utilizing this indication as reference. Otherwise, if gNB request "time stamp" of Part B, gNB needs to firstly determine a proper time stamp indication and it may be occurred that LMF cannot provide the Part B with the requested time stamp. Things become complicated.

In NRPPa MEASUREMENT RESPONSE message, gNB provides "time stamp" indication to LMF.

"time stamp" could be the time stamp of Part A at gNB.

  - Report on model performance monitoring outcome

During recent meetings, it was agreed that the NG-RAN node calculates monitoring metrics for its own model in Case 3a. However, subsequent operations after metric calculation require clarification:

- What monitoring outcome should the gNB report to the LMF?

In general, the monitoring outcomes should include two types: the calculated metric value, and the monitoring decision based on this metric. The similar issue for Case 1 was discussed in RAN1, and it has agreed that the UE monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method. The detailed agreements are:

Agreement RAN1#119 For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B Agreement RAN1#120b For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

Since monitoring metrics remain unspecified for Case 3a, we propose adopting this same design. Upon receiving the outcome, the LMF will determine whether to suspend AI/ML positioning and initiate NRPPa messaging for deactivation.

For model performance monitoring of positioning Case 3a, the gNB may signal the monitoring outcome to LMF.

The monitoring outcome is determined based on calculated metric.

The monitoring outcome includes at least an indication that the gNB cannot perform the Case 3a positioning method.

- How should the gNB report monitoring outcomes?

A new NRPPa procedure should be introduced to convey monitoring outcomes. This message should be triggered when:

• The gNB completes monitoring metric calculation, and

• The metric indicates unreliable positioning performance.

Define new NRPPa procedure to indicate the monitoring outcome that the gNB cannot perform the Case 3a positioning method.

A TP for the AI/ML BLCR to TS 38.455 is provided in Annex 2.

- Conclusion

In this contribution, we discussed the issues of AI/ML for positioning accuracy enhancement. Observations and proposals are summarized as following:

Proposal 1:	For data collection of Case 3a, “FFS if UE location can be requested” in RAN3#128 agreement is resolved by:

−	UE location cannot be requested.

Proposal 2:	In TS 38.305 BLCR, change "information related to UE location" to "timing information".

Proposal 3:	Regarding the measurement report of Case 3a, resolve the FFS item from RAN3#128 ("FFS: if LMF should specifically request for AIML inferred measurement") as follows:

−	It is not necessary to enable LMF to request AIML inferred measurement

Proposal 4:	In NRPPa MEASUREMENT RESPONSE message, gNB provides "time stamp" indication to LMF.

−	"time stamp" could be the time stamp of Part A at gNB.

Proposal 5:	For model performance monitoring of positioning Case 3a, the gNB may signal the monitoring outcome to LMF.

−	The monitoring outcome is determined based on calculated metric.

−	The monitoring outcome includes at least an indication that the gNB cannot perform the Case 3a positioning method.

Proposal 6:	Define new NRPPa procedure to indicate the monitoring outcome that the gNB cannot perform the Case 3a positioning method.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #107.

- R3-253977, (BL CR to 38.305) Introduction of AI/ML air, RAN3#128

- RAN1 Chairman’s notes, 3GPP RAN1 #120

# Annex 1 – TP for the AI/ML BLCR to TS 38.305

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

## 7.x	AI/ML assisted positioning with gNB side model

### 7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML model.

7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to UL related positioning procedures described in Clause 8.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clause 8.

4.	The LMF sends a NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated in Step 3 that positioning data collection is needed. The message includes timing information and correlation information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

Editor’s Note: The texts above could be further refined, if needed.

Editor’s Note: FFS on details of the signallings and IEs.

<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>

# Annex 2 – TP for the AI/ML BLCR to TS 38.455

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |
| SRS Information Reservation Notification | SRS INFORMATION RESERVATION NOTIFICATION |
| Positioning Data Collection Report | POSITIONING DATA COLLECTION REPORT |
| Performance Monitoring Report | PERFORMANCE MONITOING REPORT |

Next change

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

If the Time Window Information Measurement List IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider this as the time window(s) within which to perform the positioning measurements, as specified in TS 38.305 [18].

If the Inferred Measurement IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF shall, if supported, consider that the received positioning information is an inferred version of the measurement requested.

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

Interaction with the Positioning Data Collection Report procedure:

If the Positioning Data Collection Needed IE is included in the MEASUREMENT RESPONSE message, the LMF shall, if supported, take it into account for providing the requested positioning data in the POSITIONING DATA COLLECTION REPORT message, as specified in TS 38.305 [18].

Interaction with the Performance Monitoring Report procedure:

If the Performance Monitoring Report Needed IE is included in the MEASUREMENT RESPONSE message, the NG-RAN node shall, if supported, take it into account for sending the PERFEREMANCE MONITORING REPORT message.

Next change

### 8.5.2	Measurement Report

#### 8.5.2.1	General

The Measurement Report procedure allows the NG-RAN node to report positioning measurements to the LMF. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.2.2	Successful Operation

Figure 8.5.2.2.1: Measurement Report procedure. Successful operation.

The NG-RAN node initiates the procedure by sending a MEASUREMENT REPORT message to the LMF. The MEASUREMENT REPORT message contains the measurement results according to the associated measurement configuration.

Interaction with the Positioning Data Collection Report procedure:

If the Positioning Data Collection Needed IE is included in the MEASUREMENT REPORT message, the LMF shall, if supported, take it into account for providing the requested positioning data in the POSITIONING DATA COLLECTION REPORT message, as specified in TS 38.305 [18].

Interaction with the Performance Monitoring Report procedure:

If the Performance Monitoring Report Needed IE is included in the MEASUREMENT RESPONSE message, the NG-RAN node shall, if supported, take it into account for sending the PERFEREMANCE MONITORING REPORT message.

Next change

## 8.X	Performance Monitoring Result Transfer

#### 8.X.1	Performance Monitoring Result Report

#### 8.X.1.1	General

The Performance Monitoring Result Report procedure allows the NG-RAN to provide positioning performance monitoring result to LMF.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2.1: Performance Monitoring Report procedure. Successful operation.

The NG-RAN node initiates the procedure by sending a PERFORMANCE MONITORING REPORT message to the LMF. The PERFORMANCE MONITORING REPORT message contains monitoring output that indicate the NG-RAN node cannot infer the measurement.

#### 8.X.1.3	Unsuccessful Operation

Not applicable.

#### 8.X.1.4	Abnormal Conditions

Void.

Next change

### 9.1.X	Messages for Performance Monitoring Result Transfer Procedures

#### 9.1.X.1	PERFORMANCE MONITORING REPORT

This message is sent to report performance monitoring for inferred measurement.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Performance Monitoring Outcome | O |  | ENUMERATED(true, …) | Indicates the NG-RAN node cannot infer the measurement. |  |  |

Next change

<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>