3GPP TSG-RAN WG3 Meeting #129	R3-255201

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	20.2

Title:	[TP for BL CR to TS 38.455] Discussion on AI/ML for positioning Case 3b

Source:	NEC

Document for:	Discussion and Decision

- Introduction

The work items pertaining on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface were officially approved in RAN #102 and updated in RAN #107 finally with the removal of 2nd priority cases. The details concerning the positioning use case after RAN #107 have been finalized as follows [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

Based on the previous progress, this contribution addresses the remaining issues of Case 3b, LMF based positioning with LMF side model.

- Discussion

  - Signalling on unavailable measurement

Regarding the measurement for Case 3b, two alternatives, i.e., sample-based measurement and path-based measurement were discussed in RAN1. In RAN1 #119, it agreed that the enhanced measurement (sample-based measurement) in additional to path-based measurement can also be supported at least for case 3b in Rel-19. During RAN1#120 meeting, a discussion regarding whether the gNB can report the measurements different with what LMF required. The final conclusion is [2]:

Conclusion For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

From RAN3 perspective, a problem that needs to be solved is how to signal the details when the gNB cannot provide measurement with the requested measurement type indicated by LMF. In current NRPPa procedure, if the gNB cannot provide any of the requested measurement for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, this gNB responds with a MEASUREMENT FAILURE message with an appropriate cause value to LMF. The same behavior should be reused when the gNB cannot provide the required measurement type from LMF. That is, the gNB also responds with MEASUREMENT FAILURE message with an appropriate cause value to LMF.

If the gNB cannot provide measurement with the requested measurement type required by LMF, the gNB responds with MEASUREMENT FAILURE message with an appropriate cause value to LMF.

A TP for the AI/ML BLCR to TS 38.455 is provided in Annex 1.

  - Entity for performing model performance monitoring for Case 3b

Regarding model performance monitoring of Case 3b, RAN1 reached the following agreement:

Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

For Case 3b, the model is deployed at LMF side. In model inference stage, the gNB generates the timing information or paired timing information and power information (and phase information maybe) and reports it to LMF to further extract input data. In model performance monitoring stage, LMF can use gNB reported measurement to obtain the model output, and contrasts this output with ground truth label(i.e., UE location) to evaluate the model performance. Whether to keep this model or deactivate this model depend on the contrast result, for example, if this model output deviates the ground truth a lot, the LMF think this model is inappropriate, otherwise the LMF still keep this model at least until the enforcement of next model performance monitoring. Therefore, the model performance monitoring for Case 3b can be performed at LMF fully.

For Case 3b, LMF is the only entity to perform model performance monitoring, including monitoring metric calculation and monitoring decision.

- Conclusion

In this contribution, we discussed the issues of AI/ML for positioning accuracy enhancement. Observations and proposals are summarized as following:

Proposal 1:	If the gNB cannot provide measurement with the requested measurement type required by LMF, the gNB responds with MEASUREMENT FAILURE message with an appropriate cause value to LMF.

Proposal 2:	For Case 3b, LMF is the only entity to perform model performance monitoring, including monitoring metric calculation and monitoring decision.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #107.

- RAN1 Chairman’s notes, 3GPP RAN1 #120

# Annex 1 – TP for the AI/ML BLCR to TS 38.455

<<<<<<<<<<<<<<<<<< Start of the Changes >>>>>>>>>>>>>>>>>>

## 9.2.1	Cause

The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Requested Item not Supported, Requested Item Temporarily not Available, ..., Serving NG-RAN node changed, Requested Item not Supported on Time, Requested Measurement Type not Supported, Requested Measurement Type Temporarily not Available ) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Unspecified, Abstract Syntax Error (Falsely Constructed Message), ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related |
| Requested Item not Supported | The NG-RAN node does not support the requested measurement object, or cannot provide the requested information item. |
| Requested Item Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object or information item. |
| Serving NG-RAN node changed | The UE has moved to another serving NG-RAN node. |
| Requested Item not Supported on Time | The NG-RAN node is unable to provide the measurement results on time. |
| Requested Measurement Type not Supported | The NG-RAN node does not support the requested measurement type. |
| Requested Measurement Type Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement type. |

<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>