3GPP TSG-RAN WG3 Meeting #129	R3-255204

Bangalore, India, 25th – 29th May 2025

Agenda Item:		11.3

Source:		NEC

Title:	Clean-up for AI/ML-based CCO

Document for:		Discussion and decision

- Introduction

In this contribution, we further discuss and clean up the following open issues for AI/ML-based CCO.

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1) to update/cancel a prediction?

Timing information for predicted CCO issue over Xn?

Whether/what additional UE performance measurement metrics?

- Discussion

2.1	Predicted CCO issue cancel

In last meeting, one scenario about to cancel a predicted CCO issue was raised and we achieved the following WA.

WA: Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue).

Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

In general, if a new prediction is generated even for the same issue or task, instead of updating the previous predication, a separate Data Collection Reporting procedure should be triggered. The receiving node can consider both the new and the previous predictions together in order to make a proper future action, which is up to implementation.

Besides, if the AI model is well training, when it performs model inference, it should already take into account all possible variables as in the input data, and then generate a trustable prediction. And as we already agreed that the maximum value of the Requested Prediction Time is 60 seconds, the possibility of an invalid prediction in such a short time is quite low.

Based on the above two points, we do not see the need to add any new IE for this corner case. If some companies still prefer to address this cancel issue for CCO, a simple way is to add a procedure text for the existing Predicted Coverage Modification Cause IE to implicitly indicate that a previous predicted CCO issue is invalid, for example, “If the Predicted Coverage Modification Cause IE is not present, the NG-RAN node2 may consider the previous predicted CCO issue is invalid.”

Proposal 1: Add a procedure text for the existing Predicted Coverage Modification Cause IE to implicitly indicate that a previous predicted CCO issue is invalid.

2.2	Timing information for predicted CCO issue over Xn

For CCO, the following two timing information were introduced:

Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1.

Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node config Update message over Xn.

In XnAP NG-RAN NODE CONFIGURATION UPDATE message, a Time for Future Coverage State IE can be included for each Future Coverage Modification Item. As explained in the semantics description, this time information indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the NG-RAN node1 relative to the time of receiving this information [1].

As for timing information for predicted CCO issue, it indicates the time when the predicted CCO issue will happen in the gNB who performed this prediction. gNB uses this timing information and the predicted CCO issue to generate the corresponding future coverage configuration and timing information for future coverage state in legacy way or using AI/ML model. If in CU/DU split, gNB-CU will send this timing information together with the predicted CCO issue to gNB-DU, which can assist gNB-DU to select corresponding future coverage configuration and time for future coverage modification.

As described above, timing information for predicted CCO issue is used to generate corresponding future coverage configuration and time for future coverage state. For neighbour nodes, this timing information for predicted CCO issue is useless. The neighbour nodes only need to know when the future coverage state(s) will be applied, which can allow the neighbour nodes to adapt to this CCO coverage change.

Proposal 2: Not support transfer timing information for predicted CCO issue over Xn.

Consequently, the semantics description of Time for Predicted CCO Issue IE in [2] needs to be updated to clarify that it is only present from gNB-CU to gNB-DU in the sending side.

Proposal 3: Add a semantic text to clarify the Time for Predicted CCO Issue IE is only present from gNB-CU to gNB-DU in the sending side.

2.3	UE performance feedback for CCO

RAN3 already agreed that legacy UE performance measurement metrics can be reused for CCO, and the only open issue for UE performance feedback is whether additional measurement metrics is needed.

For AI/ML based CCO, UE performance feedback is used to evaluate the AI/ML model by comparing UE performance before and after a CCO action that is triggered based on the output from AI/ML Model Inference. Since the predicted CCO issue can be avoided or mitigated by advance CCO action triggered by the AI/ML Model Inference output, UE performance should be not (much) impacted due to the coverage adjustment, especially for the UE in the cell border area. Therefore, it is reasonable that gNB needs to evaluate UE performance for UEs in the cell border area that could be affected by the CCO action. To save standard effort, we prefer this can be achieved by gNB implementation, for example, neighbour gNB can report only the UE performance of the UEs in the cell border area based on UE location information.

Observation 1: UE performance collection based on UE location information can be achieved by gNB implementation and UE location information is not needed to be reported as feedback.

Some companies also proposed to report UE radio measurements (cell level RSRP, RSRP, SINR) as UE performance feedback. However, as we discussed in SI phase, besides legacy UE performance metrics, SON Reports including RLF report, RACH report can also be used as feedback, which already can reflect the radio condition. Therefore, it is no need to report additional UE radio measurements as feedback.

Observation 2: SON report can be used to evaluate radio condition and UE radio measurements is not needed to be reported as feedback.

In summary, we think legacy UE performance measurement metrics are already enough for CCO and no additional metric is needed.

Proposal 4: No additional UE performance measurement metric is needed for CCO.

Based on the above proposals, for AI/ML-based CCO, we provided a TP to BLCR TS38.423 in [3], a TP to BLCR TS38.473 in [4] and a TP to BLCR TS38.300 on in [5].

Proposal 5: To agree the TP to BLCR TS38.423, BLCR TS38.473, BLCR TS38.300 in [3], [4] and [5] respectively.

- Proposals

In this contribution, we provide our analysis on open issues for AI/ML-based CCO and made the following observations and proposals:

Observation 1: UE performance collection based on UE location information can be achieved by gNB implementation and UE location information is not needed to be reported as feedback.

Observation 2: SON report can be used to evaluate radio condition and UE radio measurements is not needed to be reported as feedback.

Proposal 1: Add a procedure text for the existing Predicted Coverage Modification Cause IE to implicitly indicate that a previous predicted CCO issue is invalid.

Proposal 2: Not support transfer timing information for predicted CCO issue over Xn.

Proposal 3: Add a semantic text to clarify the Time for Predicted CCO Issue IE is only present from gNB-CU to gNB-DU in the sending side.

Proposal 4: No additional UE performance measurement metric is needed for CCO.

Proposal 5: To agree the TP to BLCR TS38.423, BLCR TS38.473, BLCR TS38.300 in [3], [4] and [5] respectively.

- Reference

[1] R3-253986, (TP for AI/ML BLCR to TS 38.423) Discussion on the AI/ML-based Coverage and Capacity Optimization

[2] R3-253988, (TP for AI/ML BLCR to TS 38.473) Discussion on the AI/ML-based Coverage and Capacity Optimization

[3] R3-255205 (TP to BLCR TS38.423 and TS38.473) AIML-based CCO

[4] R3-255206 (TP to BLCR for TS38.300) AIML-based CCO