3GPP TSG-RAN WG3 Meeting #129	R3-255210

Bangalore, India, 25th – 29th May 2025

Agenda Item:		17.3

Source:		NEC

Title:			(TP to BLCR TS38.473) Support on-demand SIB1

Document for:		Discussion and decision

- Introduction

In RAN3#126 meeting, one new class 1 procedure for providing UL WUS Configuration from NES Cell gNB to Cell A gNB and one new class 2 procedure for informing UL WUS configuration broadcast stop from Cell A gNB to NES Cell gNB were agreed.

In RAN3#128 meeting, we further agreed the followings and all the agreements are reflected in the BLCRs in [1] [2].

One “Provision Request message includes one “OD-SIB1 config R19” referring to the TS 38.331 definition, it is a RRC Container in octet string (presence M)  +  one NES Cell ID (presence M ) +  one Cell-A ID (presence O )

Cell A gNB-CU encoding the SIBxx.

The NES gNB-CU sends the indication to NES gNB-DU. The NES gNB-DU MAY go to OD-SIB 1 operation up to gNB-DU decision.

This contribution further discusses the open issues for OD-SIB1.

- Discussion

  - Procedure naming

Regarding the new defined class 1 procedure and the new class 2 procedure, the names of the procedures and messages is still FFS.

RAN2 agreed in RAN2#129bis meeting that

Define features, not devices. That is, definition of OD-SIB1, OD-SSB, paging adaptation etc in 38.300 and in other specifications refer to 38.300:

- Avoid NES, Cell A, UL-WUS

- “UE supporting OD-SIB1” where/if needed, and to replace with the UE capability name

- UL-WUS is replaced with SIB1 request in e.g. field descriptions

- ul-WUS-Config renamed as od-sib1-Config

According to the purpose of the class 1 procedure, i.e. providing UL WUS Configuration from NES Cell gNB to Cell A gNB, and also taking into account RAN2’s agreement, it is suitable to name it as “OD-SIB1 Configuration Provision” procedure. The three corresponding messages should be named as

- OD-SIB1 Provision Request

- OD-SIB1 Provision Response

- OD-SIB1 Provision Failure

Proposal 1: Rename the class 1 procedure as “OD-SIB1 Configuration Provision” procedure and the three corresponding messages as “OD-SIB1 Configuration Provision Request”, “OD-SIB1 Configuration Provision Response” and “OD-SIB1 Configuration Provision Failure”.

Regarding the new class 2 procedure, according to the agreement made in last meeting, “A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB”, therefore, we suggest to directly name the procedure as “OD-SIB1 Configuration Broadcast Stop Indication”.

Proposal 2: Rename the class 2 procedure and the corresponding message as “OD-SIB1 Configuration Broadcast Stop Indication”.

  - Choice “pause/resume”

For the new Class 1 message (OD-SIB1 configuration Provision Request), RAN3 agreed to use a CHOICE structure  and include two codepoints i.e. “Start” and “Stop”. However, there is another one “pause/resume”, which means the Cell A gNB remembers the OD-SIB1 configuration when it receive the “Pause” and NES uses “resume”, was proposed, but still FFS.

In our view, “pause/resume” is not needed. If the OD-SIB1 configuration broadcasting in Cell A just needs to be stopped for a short time, then it is not an issue if there is no any indication from NES cell and Cell A just keeps providing OD-SIB1 configuration. RAN2 already agreed that “If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell”. If the NES Cell goes back to SIB1 broadcasting, UE can receive SIB1 in legacy way and thus there should be no impact to UE. As more than one OD-SIB1 configurations from several NES cells can be incorporated into one new SIBx, in this case, there should be no additional signalling waste even if Cell A still provides the OD-SIB1 configuration for that NES cell. While, if the OD-SIB1 configuration broadcasting in Cell A needs to be stopped for a long time, then it is not reasonable to let Cell A keep the configuration and the Choice “Stop” can be used to inform Cell A to release the configuration and stop the broadcasting.

If NES Cell needs to update its OD-SIB1 configuration, OD-SIB1 configuration Provision procedure can be reused with a new OD-SIB1 configuration included.

Therefore, we think “pause/resume” is not needed in OD-SIB1 configuration Provision Request message.

Proposal 3: One Choice “pause/resume” is not needed in the OD-SIB1 configuration Provision Request message.

  - OD-SIB1 configuration IE

The RAN2 defined SIBxx for OD-SIB1 configuration broadcasting is showing as below. The top level IE OD-SIB1-CellConfig-r19 includes not only od-SIB1-Config, but also carrierFreq and physCellId, and the field description of this IE states “Provides a configuration to request SIB1 for serving cell and neighbor cells” [3]. Therefore, for the RAN3 defined OD-SIB1 configuration IE (former UL WUS configuration IE) in the OD-SIB1 CONFIGURATION  PROVISION REQUEST message, it should refer to OD-SIB1-CellConfig-r19 IE in TS38.331.

#### –	SIBxx

SIBxx contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells.

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxCellODSIB1-r19))  OF OD-SIB1-CellConfig-r19    OPTIONAL,        -- Need R

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellId-r19                           SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId,

od-SIB1-Config-r19                       OD-SIB1-Config-r19                               OPTIONAL,        -- Need R

...

}

OD-SIB1-Config-r19  ::=       SEQUENCE {

sib1-rsrp-ThresholdSSB-r19                       RSRP-Range                                                              OPTIONAL,   -- Need R

prach-RootSequenceIndex-r19        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                                   OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19               ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2}  OPTIONAL, -- Need R

sib1-tdd-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                                     OPTIONAL, -- Cond TDD

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}       OPTIONAL, -- Need R

offsetToCarrier-r19                          INTEGER (0..2199)                                                      OPTIONAL, -- Need R

absoluteFrequencyPointA-r19                  ARFCN-ValueNR                                                          OPTIONAL, -- Need R

frequencyBandList-r19                        MultiFrequencyBandListNR                                               OPTIONAL, -- Need R

p-Max-r19                                    P-Max                                                                  OPTIONAL, -- Need R

ss-PBCH-BlockPower-r19                       INTEGER (-60..50)                                                      OPTIONAL, -- Need R

ssb-PositionsInBurst-r19                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

ssb-Periodicity-r19                         ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2}      OPTIONAL, -- Need R

od-sib1-WindowDuration-r19                   FFS,

od-sib1-windowStartOffset-r19                FFS,

k-ssb-r19                                    INTEGER (1..23),

sib1-RequestConfig-r19                   SIB1-RequestConfig-r19                                                    OPTIONAL -- Need R

}

<Not relevant part omitted>

Proposal 4: Refer to OD-SIB1-CellConfig-r19 IE in TS38.331 for the OD-SIB1 configuration IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message in TS38.423 and in the Served Cell Information IE in in TS38.473.

  - IE presence

NR CGI at NG-RAN node2 IE in OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message is FFS. Since we already agreed that NR CGI at NG-RAN node2 IE (Cell A ID) in the OD-SIB1 CONFIGURATION PROVISION REQUEST message is optional. If this IE is not included, Cell A gNB can select one of its Cell A for this request, and NES cell only know that there is one Cell A in Cell A gNB accepting its request but does not know the Cell A ID. In this case, when Cell A needs to inform NEC cell that the OD-SIB1 configuration cannot be provided in it, Cell A ID is not needed to be indicated. Therefore, this NR CGI at NG-RAN node2 IE should be optional.

Proposal 5: The presence of NR CGI at NG-RAN node2 IE in OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message is optional.

  - OD-SIB1 configuration broadcasting capability transfer

The NES cell needs to select a cell which can work as Cell A, which means it can support OD-SIB1 configuration broadcasting for the NEC cell, or to select a gNB which has a cell can worked as Cell A. Some companies think that this can be achieved by OAM configuration. However, OAM configuration is quite static, while a change between Cell A and normal cell can be semi-static. Therefore, besides OAM configuration, we suggest also to support the exchange of OD-SIB1 configuration broadcasting capability between gNBs, e.g. during Xn setup procedure. This can greatly reduce the OD-SIB1 Configuration Provision request failure rate.

Proposal 6: Support the exchange of OD-SIB1 configuration broadcasting capability between gNBs during Xn setup procedure.

  - Multiple Cell A scenario

There is one possible scenario that, for one NES Cell, multiple Cell A(s) are applicable. In this case, the NES Cell can select which Cell A(s) to broadcast OD-SIB1 configuration for it, and it can select only one or several or all Cell A(s) from the applicable Cell A(s). Since we agreed in last RAN3 meeting that only one NEC cell can be carried in one OD-SIB1 Configuration Provision Request message, there may be several OD-SIB1 Configuration Provision Request messages need to be sent form NES cell gNB to Cell A gNB(s). Afterwards, the NES cell can decide how many OD-SIB1 Configuration Provision Response message(s) it needs to receive before activating OD-SIB1.

Proposal 7: The following NES cell gNB behaviours can be left to implementation:

- NES Cell can select which Cell A(s) for OD-SIB1 configuration provision.

- NES cell gNB can send OD-SIB1 configuration Provision Request to Cell A gNB(s) one by one or at the same time

- NES cell can decide how many WUS Configuration Provision Response message it needs to receive before activating OD-SIB1

In the case of split architecture, the simple and straightforward way is, after NES Cell gNB-CU receives one OD-SIB1 Configuration Provision Response message, it will send the On-Demand SIB1 Cell ID via GNB-CU CONFIGURATION UPDATE message to NES Cell gNB-DU.

Proposal 8: NES Cell gNB-CU sends the On-Demand SIB1 Cell ID via GNB-CU CONFIGURATION UPDATE message to NES Cell gNB-DU once it receives one OD-SIB1 Configuration Provision Response message from a Cell A gNB.

TP to BLCR TS38.423 and TS38.473 reflecting the above proposals are in [4] and in Annex respectively.

Proposal 9: Agree the TP to BLCR TS38.423 and TS38.473 in [4] and in Annex.

- Proposals

In this contribution, we addressed the open issues for OD-SIB1 of a NES Cell based on the latest agreements and the following proposals were drawn from the analysis.

Proposal 1: Rename the class 1 procedure as “OD-SIB1 Configuration Provision” procedure and the three corresponding messages as “OD-SIB1 Configuration Provision Request”, “OD-SIB1 configuration Provision Response” and “OD-SIB1 Configuration Provision Failure” (to remove corresponding Naming FFS in [1]).

Proposal 2: Rename the class 2 procedure and the corresponding message as “OD-SIB1 Configuration Broadcast Stop Indication”.

Proposal 3: One Choice “pause/resume” is not needed in the OD-SIB1 configuration Provision Request message.

Proposal 4: Refer to OD-SIB1-CellConfig-r19 IE in TS38.331 for the OD-SIB1 configuration IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message in TS38.423 and in the Served Cell Information IE in in TS38.473.

Proposal 5: The presence of NR CGI at NG-RAN node2 IE in OD-SIB1 CONFIGURATION PROVISION STATUS UPDATE message is optional.

Proposal 6: Support the exchange of OD-SIB1 configuration broadcasting capability between gNBs during Xn setup procedure.

Proposal 7: The following NES cell gNB behaviours can be left to implementation:

- NES Cell can select which Cell A(s) for OD-SIB1 configuration provision.

- NES cell gNB can send OD-SIB1 configuration Provision Request to Cell A gNB(s) one by one or at the same time

- NES cell can decide how many WUS Configuration Provision Response message it needs to receive before activating OD-SIB1

Proposal 8: NES Cell gNB-CU sends the On-Demand SIB1 Cell ID via GNB-CU CONFIGURATION UPDATE message to NES Cell gNB-DU once it receives one OD-SIB1 Configuration Provision Response message from a Cell A gNB.

Proposal 9: Agree the TP to BLCR TS38.423 and TS38.473 in [4] and in Annex.

- Reference

- R3-253974, (BL CR to 38.423) Introduction of Network Energy Saving Enhancement

- R3-253975, (BL CR to 38.473) Introduction of Network Energy Saving Enhancement

- R2-2503607, Running RRC CR for enhancements for network energy efficiency

- R3-25xxxx, TP to BLCR TS38.423 on support on-demand SIB1

Annex TP to BLCR TS38.473

<Start of change>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| CHOICE on-demand SIB1 | O |  |  |  | YES | ignore |
| >Provision |  |  |  |  |  |  |
| >>On-demand SIB1 Config | M |  | Octet String | Includes the OD-SIB1-CellConfig-r19 contained in the SIBxx message as defined in TS 38.331 [8]. | - |  |
| >Stop provision |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

<End of change>