3GPP TSG-RAN WG3 #129	R3-255212

Bengaluru, IN, Aug 25th – 29th, 2025

Agenda item:		14.3

Title:		(TP to BL CR for TS 38.300 on NR_NTN_Ph3) Discussion on NTN leftover issue

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the previous meeting, RAN3 made the following progress on NR NTN regenerative payload enhancement:

WA: For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure. Check the use cases including Hard Feederlink Switch, other use cases to be further justified.

One open issue remains in RAN3, which this paper will discuss.

# 2. Discussion

2.1 The suspend/resume indication for Hard Feeder link Switch

Whether to support the suspend/resume indication for the NG connection during Hard Feeder Link Switch remains highly controversial, as the objecting companies have concerns about the benefits of the suspend/resume indication.

During a hard feeder link switch, the NG interface (NG-C and NG-U) is interrupted; this interruption causes packet loss, which directly impacts user experience. Furthermore, due to the NG interface interruption, procedures over the NG interface may exceed their maximum allowed time, which leads to NG procedure failure. Such a failure may lead to greater latency than the interruption duration caused by the hard feeder link switch itself.

For example, the timer TNGRELOCprep and TNGRELOCoverall that defined in TS 38.413 that

## 9.6 Timers TNGRELOCprep - Specifies the maximum time for the Handover Preparation procedure in the source NG-RAN node. TNGRELOCoverall - Specifies the maximum time for the protection of the overall handover procedure in the source NG-RAN node.

Example 1: When the source NG-RAN node sends the Handover Required message to the AMF, it shall start timer TNGRELOCprep. If the NG interface is suspended (due to hard feeder link switchover) before the NG-RAN node receives the Handover Command message and the AMF is not aware of this, TNGRELOCprep may expire before the NG interface resumes. Consequently, the source NG-RAN node will consider the handover preparation procedure to have failed and initiate the Handover Cancel procedure towards the AMF with the cause value "TNGRELOCprep expiry".

Example 2: Timer TNGRELOCoverall starts upon the source NG-RAN node's reception of the Handover Command message from the AMF, and stops upon its reception of the UE Context Release Command message from the AMF. If the NG interface is suspended before TNGRELOCoverall stops and the AMF is not aware of this, the timer may expire due to the NG interface suspension. As a result, the source NG-RAN node will determine that the handover procedure has failed and shall initiate the UE Context Release procedure towards the AMF with the cause value "TNGRELOCoverall expiry".

Therefore, if the AMF is unaware that the NG interface is suspended or resumed, it may cause timers TNGRELOCprep and TNGRELOCoverall to expire, which leads to failure of the handover procedure. If the AMF is aware of the NG interface being suspended or resumed, it may pause the TNGRELOCoverall and TNGRELOCprep timers during NG interface suspension, which can be based on NG-RAN/AMF implementation. These timers may then be resumed once the AMF receives the NG interface resume indication. This prevents unnecessary handover failures during hard feeder link switch.

Moreover, other NG signaling procedures may also have maximum operation times, which are not specified in TS but may be utilized in actual implementations. Thus, the suspend/resume indication could potentially help reduce other NG procedure failures caused by hard feeder link switches.

Observation 1: The suspend/resume indication for NG connection can help reduce procedure failures caused by hard feeder link switches.

Proposal 1: Adopt the following Work Agreement (WA) as agreed: “For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure”.

Furthermore, in the previous CR, it was agreed to implement NG interface suspension and resume indications in the Downlink NG Transmission Control IE from the onboard gNB to AMF. This mechanism suspends downlink NG transmission during hard feeder link switchover that causes NG interface interruptions.

While it should be noted that uplink NG transmission is equally affected by the hard feeder link switchover. The onboard gNB is aware of the NG interface interruption timing, which can implement uplink signaling buffering and transmit the buffered signaling once the NG interface resumes. Therefore, for the UL transmission, stage 2 statement is needed and no further enhancement on stage 3. The corresponding TP is provided in Annex A.

Observation 2: The Uplink NG transmission is equally affected by the hard feeder link switchover.

Proposal 2: For the UL NG transmission, stage 2 statement is needed and no further enhancement on stage 3. The corresponding TP is provided in Annex A.

# 3. Conclusion

Observation 1: The suspend/resume indication for NG connection can help reduce procedure failures caused by hard feeder link switches.

Observation 2: The Uplink NG transmission is equally affected by the hard feeder link switchover.

Proposal 1: Adopt the following Work Agreement (WA) as agreed: “For regenerative payload, an indication to suspend or resume NG connection may be sent to AMF from gNB in the RAN Configuration Update procedure”.

Proposal 2: For the UL NG transmission, stage 2 statement is needed and no further enhancement on stage 3. The corresponding TP is provided in Annex A.

# References

# Annex A: TP to BLCR for TS 38.300

--------------------------------------------------------------Start of the change-----------------------------------------------------------

## 16.14	Non-Terrestrial Networks

/******* Unchanged part skipped ******/

### 16.14.4	Switchover

#### 16.14.4.1	Definitions

A feeder link switchover is the procedure where the feeder link is changed from a source NTN Gateway to a target NTN Gateway for an NTN payload. The feeder link switchover is a Transport Network Layer procedure. Service link switch refers to a change of the serving NTN payload.

Both hard and soft feeder link switchover are supported in NTN payloads.

#### 16.14.4.2	Assumptions

A feeder link switch over may result in transferring the established connection for the affected UEs between two gNBs.

For soft feeder link switch over, an NTN payload is able to connect to more than one NTN Gateway during a given period, i.e. a temporary overlap can be ensured during the transition between the feeder links.

For hard feeder link switch over, an NTN payload connects to only one NTN Gateway at any given time, i.e. a radio link interruption and/or NG link interruption may occur during the transition between the feeder links. For regenerative payload, the NG link interruption cause data and signaling procedure over NG interface to suspend.

#### 16.14.4.3	Procedures

he NTN Control function (see Annex B.4) determines the point in time when the feeder link switch over is performed. The transfer of the affected UE(s) at feeder link switch over is performed by means of handover or TNL functionality, and it depends on the gNBs' implementation and configuration information provided to the gNBs by the NTN Control function.

NOTE:	In case of TNL address change due to feeder link switchover, existing procedures (e.g., Path Switch Request procedure, PDU Session Resource Modify Indication procedure) may be used.

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

--------------------------------------------------------------End of the change----------------------------------------------------------