3GPP TSG-RAN WG3 #129	R3-255213

Bengaluru, IN, Aug 25th – 29th, 2025

Agenda item:		16.2

Title:		Discussion on A-IoT leftover issues

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The following progress was made during the last RAN3 meeting:

The new A-IoT Area is encoded as an A-IoT Area ID. A-IoT Area ID = PLMN ID +NID(optional) + A-IoT Area Code (OCTET STRING (SIZE(3))) One reader only belongs to one gNB. One reader can map to one or multiple A-IoT Area ID(s). One A-IoT Area may include readers belong to the same or different gNBs. One gNB may serve multiple readers which belong to the same or different A-IoT Areas. OAM configures in the AIOTF the mapping relationships among gNBs, readers and A-IoT areas, as needed. Non-UE associated signalling principle and requirements applies to A-IoT related signalling. Introduce new CN triggered Class 1 Session Release procedure, which includes A-IoT SESSION RELEASE REQUEST message and A-IoT SESSION RELEASE RESPONSE message. Turn WA to agreement “NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.” In case of indirect connectivity, allow parallel sessions between gNB and AMF. In case of indirect connectivity, allow parallel Command procedures for different devices between gNB and AMF within the same session. CN A-IoT Device NGAP ID is not needed. Define the Expected D2R Message Size IE in Inventory Request Transfer IE. FFS on the detail encoding. Remove the FFSs on the presence of Cause IE in the INVENTORY FAILURE message and the COMMAND FAILURE message. FFS on the detail encoding of the A-IoT Correlation Identifier IE? Wait for further inputs from CT4. FFS on the detail encoding of the AIOTF Identifier IE? Refer to the NfInstanceId IE defined in TS 29.571? Define the A-IoT Device Identification Requested IE as CHOICE type, with three branches: - single device inventory: OCTECT STRING refer to Device Identifier - group devices inventory: FFS on the encoding - all devices inventory: NULL Define the Estimate of Expected D2R Message Size IE in the Command Assistance Information IE as INTEGER (1..128, …) unit: byte. Add the semantic description that this IE refers to NAS PDU size. Details of the Reader Location is out the scope of RAN3. Reader Selection refers to SA2 TS 23.369 clause 5.3.3. Further check is needed. It is FFS to introduce Command Type (write, read, disable) in Command Request Transfer. It is FFS on the Time Interval as assistance information in the Inventory Request message. Clarify with SA2 on the usage of this information in NG-RAN node. RAN A-IoT Information will not be provided over NGAP from NG-RAN node to AIoT CN. Turn WAs to agreement “Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.” “Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.” Include RAN NGAP Device ID outside of the containers in the Command related messages. Mandatorily include the Expected D2R Message Size in Inventory Request Transfer? Check progress in SA2. In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message. In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked. Introduce a class2 procedure to allow NG-RAN node to trigger Session Release procedure towards AIoT CN. WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …) Whether to have A-IoT Support Indicator in the NG SETUP RESPONSE message?

This contribution will discuss the open issues of Ambient IoT.

# 2. Discussion

2.1 Discussion on the Time Interval

From the last meeting, there is an FFS item regarding the Time Interval, as follows:

It is FFS on the Time Interval as assistance information in the Inventory Request message. Clarify with SA2 on the usage of this information in NG-RAN node. Mandatorily include the Expected D2R Message Size in Inventory Request Transfer? Check progress in SA2.

In SA2, the time interval has already been agreed as AIoT aggregation assistance information and is captured in TS 23.369:

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests: a) AIoT service type (e.g. Inventory, Command). b) Optionally, approximate number of AIoT devices based on AF request. c) Optionally, approximate D2R message size based on AF request. d) Optionally, time interval as AIoT aggregation assistance information. Editor's note: Other assistance information may be added later if necessary. Editor's note: It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause. If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF. NOTE: Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

The time interval is configured by the AF entity, and NG-RAN shall send the inventory report within or shorter than this time interval. This is beneficial for optimizing RAN reader reporting and reflecting the latency requirements of AIoT services. Therefore, the Time Interval should be included as optional assistance information in the Inventory Request message.

Proposal 1: The Time Interval shall be included as optional assistance information in the Inventory Request message.

Additionally, as the SA2 statement above, the approximate D2R message size is an optional IE from AIoTF to NG-RAN. To align with SA2, the Expected D2R Message Size IE should not be mandatorily included in the Inventory Request Transfer.

Proposal 2: The Expected D2R Message Size IE shall not be mandatorily included in the Inventory Request Transfer.

2.2 Whether to introduce Command Type in Command Request Transfer

It is FFS to introduce Command Type (write, read, disable) in Command Request Transfer.

The only function of the Command Type IE is to indicate RAN reader resource allocation for D2R responses. The “Write” and “Disable” types mean only R2D NAS PDU are expected (with no D2R NAS PDU expected), while the “Read” type means both R2D NAS PDU and D2R NAS PDU are expected.

However, this function can be realized via the Estimate of Expected D2R Message Size IE, which was agreed to be a Command Assistance Information IE in the Command Request message in the last meeting. The Estimate of Expected D2R Message Size IE can implicitly indicate the “Write” and “Read” types: the presence of this IE indicates a “Read” command, while its absence indicates a “Write” or “Disable” command. Therefore, RAN3 does not need to define the Command Type (Write, Read, Disable) in the Command Request Transfer.

Proposal 3: RAN3 does not need to define the Command Type (Write, Read, Disable) in the Command Request Transfer.

2.3 Complete indication for Inventory report

In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message. In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

For the Inventory Complete Indication to the A-IoT CN in the inventory only scenario, three available options are as follows:

- Option 1: Include the Inventory Complete Indication in the Inventory Report message to the A-IoT CN.

- Option 2: Include the Inventory Complete Indication in the A-IoT Session Release Request message (gNB -> A-IoT CN).

- Option 3: Define a new message to carry the Inventory Complete Indication from the gNB to the A-IoT CN.

Since this indication is used for the inventory only scenario, its purpose is to inform the A-IoT CN that the A-IoT session has been completed, prompting the A-IoT CN to initiate sending the A-IoT Session Release Command message to the gNB to release the A-IoT session. This function is similar to that of the A-IoT Session Release Request message. Therefore, including the Inventory Complete Indication in the A-IoT Session Release Request message is more appropriate.

Regarding to command after inventory scenario, the complete indication also means the A-IoT session is completed, which plays the same role as the Inventory Complete Indication in the Inventory only scenario.

Overall, a session complete indication should be used for both the inventory-only scenario and the command after inventory scenario, rather than using separate indications for each. The session complete indication can thus be included in the A-IoT Session Release Request message to the A-IoT CN.

Proposal 4: A session complete indication should be used for both inventory only scenario and command after inventory scenario, rather than using separate indications for inventory only scenario and command after inventory scenario.

Proposal 5: The session complete indication can be included in the A-IoT Session Release Request message to the A-IoT CN.

2.4 Encoding related issues

Define the A-IoT Device Identification Requested IE as CHOICE type, with three branches: - single device inventory: OCTECT STRING refer to Device Identifier - group devices inventory: FFS on the encoding - all devices inventory: NULL

Regarding the filtering information for group device inventory, SA2 has a clear definition as follows:

## 5.8 Filtering Information The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components of the AIoT Device Permanent Identifier as defined in clause 5.7. The filtering information includes a list of filtering element information. The filtering element information includes: - Information indicating a which component of the AIoT Device Permanent Identifier that is used to match the bitstring. - A bitstring which is used to compare with the component. Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier. NOTE: The bitstring match can start at any position in the corresponding component. Editor's note: Whether and how to secure the filtering information is up to SA WG3. To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every filtering element information within filtering information by comparing the bitstring in a filtering element information with the indicated component of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

The filtering information for group device inventory corresponds to one component of the A-IoT Device Permanent Identifier. Since the A-IoT Device Permanent Identifier is transparent to the RAN reader and defined as an OCTET STRING, the filtering information for group device inventory should also be defined as an OCTET STRING.

Proposal 6: The filtering information for group device inventory should also be defined as an OCTET STRING.

2.5 NG Interface management for direct connection between Reader and AIoTF

RAN3 has some agreements on the session release procedure as follows:

Introduce new CN triggered Class 1 Session Release procedure, which includes A-IoT SESSION RELEASE REQUEST message and A-IoT SESSION RELEASE RESPONSE message. Introduce a class2 procedure to allow NG-RAN node to trigger Session Release procedure towards AIoT CN.

For the direct A-IoT NG interface between gNB and AIoTF, the interface is only utilized during the A-IoT service is triggered by the AIoTF and will not be used after the A-IoT service released. This differs from the NG interface between the gNB and AMF. Therefore, RAN3 needs to determine an appropriate management approach for this interface.

Two options are under our consideration:

- Option 1: The A-IoT NG interface is established when the A-IoT service is initiated and released when the A-IoT service is released.

- Option 2: Follow the same legacy NG interface management as between the gNB and AMF. The A-IoT NG interface is established when the TNL association becomes operational and maintained continuously.

The legacy NG interface between the gNB and AMF requires persistent maintenance because the RAN and AMF cannot predict downlink and uplink transmissions between the UE and CN, and real-time downlink and uplink data delivery must be guaranteed.

However, the A-IoT NG interface is different: communication between the A-IoT device and AIoTF only occurs when A-IoT service sessions are configured, and there are no strict latency requirements for A-IoT UL/DL data transmission. Additionally, continuous interface maintenance consumes unnecessary network resources and generates excessive signaling overhead.

Given these factors, we recommend that the A-IoT NG interface be established when the AIoTF initiates a service and released when no further services are configured.

Proposal 7: The establishment of the direct A-IoT NG interface between the gNB and AIoTF shall be triggered by the initiation of an AIoT service by the AIoTF, and the interface shall be released when the AIoT service is released.

# 3. Conclusion

Proposal 1: The Time Interval shall be included as optional assistance information in the Inventory Request message.

Proposal 2: The Expected D2R Message Size IE shall not be mandatorily included in the Inventory Request Transfer.

Proposal 3: RAN3 does not need to define the Command Type (Write, Read, Disable) in the Command Request Transfer.

Proposal 4: A session complete indication should be used for both inventory only scenario and command after inventory scenario, rather than using separate indications for inventory only scenario and command after inventory scenario.

Proposal 5: The session complete indication can be included in the A-IoT Session Release Request message to the A-IoT CN.

Proposal 6: The filtering information for group device inventory should also be defined as an OCTET STRING.

Proposal 7: The establishment of the direct A-IoT NG interface between the gNB and AIoTF shall be triggered by the initiation of an AIoT service by the AIoTF, and the interface shall be released when the AIoT service is released.

# References