3GPP TSG-RAN WG3 #129	R3-255214

Bengaluru, India, 25th – 29th August 2025

Source:				NEC

Agenda item: 	10.2. MRO Enhancements

Title: 	(TP to BLCR of 38.473) Discussion on MRO for LTM

Document for:				Discussion and decision

- Introduction

- 10. Data Collection for SON_MDT in NR standalone and MR-DC WI (RAN3-led) WID [NR_ENDC_SON_MDT_Ph4-Core]: RP-234038 (target: RAN #109) [TU: 1.5 (1.5, 1.5)] QUOTA: 4

10.2. MRO Enhancements MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2] From RAN3#128: BFR shortly after successful LTM cell switch to the wrong beam(Case 1) Use DU-CU ACCESS AND MOBILITY INDICATION message for target DU send recovery beam information to the CU. Only one recovery beam information included. Additional Information with recovery beam information from target DU to CU: - CU UE F1AP ID - Failure type (for case 1, case 2) in an implicit way or explicit way - beam failure indicator in an implicit way or explicit way Use ACCESS AND MOBILITY INDICATION message for CU forward recovery beam information to the source DU. Only one recovery beam information included. Additional Information with recovery beam information from target CU to DU: - Source DU UE F1AP ID - Failure type (for case 1, case 2) in an implicit way or explicit way - beam failure indicator in an implicit way or explicit way No new failure type for beam failure recovery is needed for TS38.300. The Timer configured in DU (e.g., Tstore_UE_cntxt) used for the DU to detect BFR shortly after successful LTM cell switch to the wrong beam is up to implementation. UHI and ping-pong issue CU informs relevant information to the target DU via CU-DU Cell Switch Notification. Down selection of following solutions: 1a: CU provide full UHI list with L1/L3 type to the DU (maximum item number is 16). 1b: CU provide full UHI list (except PScell) with L1/L3 type to the DU (maximum item number is 16). 2a: CU provide last consecutive L1 entries to the DU. 2b: CU provide FFS number of last consecutive L1 entries to the DU. No stage2, work in stage3 with 1a. If the above 1a information is included in F1AP message from CU to DU, the DU may consider the potential mobility issue detected including ping-pong. FFS on the extra indication for ping-pong case. RACHless LTM cell switch failure due to wrong beam (Case 2) Network-based solution for the LTM cell switch failure due to wrong beam using the same signaling flow as for Case1. In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization. In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization. Target DU needs to send the reconnect/re-established/recovery beam information to CU and CU forwards it to source DU. CU does not need to send the old beam information to source DU. Reply LS to RAN2 on the network based solution for case1 and case2. Work on stage3 semantic description for C-RNTI IE in ACCESS AND MOBILITY INDICATION message. New BL CR for TS38.420 Samsung

In this contribution, we would like to provide our further consideration on MRO for LTM.

- Discussion

2.1 Ping-pong avoidance for LTM

During last RAN3 meeting, it was agreed that CU informs relvevant information to the target DU via CU-DU Cell switch Notification message. However, whether the CU sends the information for cases other than ping-pong issue, and what is the exact information transmitted from CU to DU is still FFS.

From our perspective, as a compromise, the CU can send the information if the CU feels that there is the need, i.e., it is not only for ping-pong case. For example, the information may be used for the DU to determine to adjust the LTM prameters or the candidate cells. And an extra ping-pong indicator can be included in the the message, such that the DU can pay attention to ping-pong issue. Otherwise, if a ping-pong indication is not included, it is up to the DU whether to check the information and optimize the LTM based on it. For the content of the information, we think the existing UHI information is fine, since the sigalling overhead is not very big, and the existing IE for UHI can be reused directly.

Proposal 1: The CU sends, full UHI list with L1/L3 type with one optional ping-pong indication, to the gNB-DU by CU-DU Cell Switch Notification message. An extra ping-pong indication is included in the message, if the gNB-CU detects a potential ping-pong occurrence.

2.2 MRO enhancement in case of RLF-report absent

For conventional handover procedure, SON MRO support the case that the RLF-report from the UE is available or not avaliable, i.e., the Failure Indication message can be with or without RLF-report. However, for MRO of LTM, we only support the case of LTM failure with RLF-report, while the case of without RLF-report is not specified.

The same as conventional handover procedure, the RLF-report is not availabe because the UE does not have the RLF-report, i.e., before being reported to the network, the content of the RLF-report associated with the LTM failure is replaced/refeshed by a following connection failure. This is because according to TS 38.331, upon a connection failure (radio link failure or handover falure), the UE always clears the content of the RLF-report which is not reported to the network first, and then set it with the information associated with the lastest connection failure. For another example, the RLF-report may be not obtained by the gNB-CU due to network congestion, and finally discarded by the UE after 48 hours.

Observation: Upon connection failure, the UE clears the content of the RLF-report which is not reported to the network yet and sets with new information, which results in RLF-report absent for the previous failure.

When the RLF-report is not available in the gNB-CU, the gNB-CU can still identify the happening of LTM failure event:

- Too late LTM:  after the configuring of LTM configuration to the UE, if the gNB-CU finds that LTM cell switch is not triggered by the gNB-DU, and the UE re-establishes/re-connectes to another cell, the gNB-CU can identify the happening of too late LTM cell switch.

- Too early LTM: if the gNB-CU finds that a LTM cell switch is triggered by the gNB-DU, the UE does not access to the target LTM cell or soon after the UE accesses to the arget LTM cell, and the UE re-establishes/re-connects to the source cell of the LTM cell switch procedure, the gNB-CU can identify the happening of too early LTM cell switch.

- LTM to wrong cell:  if the gNB-CU finds that a LTM cell switch is triggered by the gNB-DU, the UE does not access to the target LTM cell or soon after the UE accesses to the target LTM cell, and the UE re-establishes/re-connects to a cell which is neither the source cell nor the target cell of the LTM cell switch procedure, the gNB-CU can identify the happening of LTM cell switch to wrong cell.

For the cases of LTM failure without RLF-report, the gNB-CU can also indicate to gNB-DU the failure information of LTM cell switch, such that the gNB-DU can performs the corresponding MRO optimization, e.g., to adjust the tiggering condition of LTM cell switch. In addition to the failure type (i.e., too late LTM, too early LTM and LTM to wrong cell) and UE ID (including gNB-DU F1AP ID and source C-RNTI), some basic information can also be provided to the gNB-DU by the gNB-CU, for example the ID of the source cell/failed cell of the gNB-DU, target cell of the LTM cell switch procedure, and ID of the re-established/re-connected cell.

Proposal 2: When RLF-report is absent in gNB-CU, gNB-CU can indicate to gNB-DU the following LTM failure information:

- Failure type (i.e., too late LTM, too early LTM and LTM to wrong cell)

- UE ID (gNB-DU F1AP ID, source C-RNTI)

- ID of the source/failed cell of the gNB-DU, and ID of target LTM cell

- ID of re-established/re-connected cell

# 3. Conclusion

In this paper, we provide our further consideration on MRO for LTM, and have the following proposals:

Proposal 1: The CU sends, full UHI list with L1/L3 type with one optional ping-pong indication, to the gNB-DU by CU-DU Cell Switch Notification message. An extra ping-pong indication is included in the message, if the gNB-CU detects a potential ping-pong occurrence.

Observation: Upon connection failure, the UE clears the content of the RLF-report which is not reported to the network yet and sets with new information, which results in RLF-report absent for the previous failure.

Proposal 2: When RLF-report is absent in gNB-CU, gNB-CU can indicate to gNB-DU the following LTM failure information:

- Failure type (i.e., too late LTM, too early LTM and LTM to wrong cell)

- UE ID (gNB-DU F1AP ID, source C-RNTI)

- ID of the source/failed cell of the gNB-DU, and ID of target LTM cell

- ID of re-established/re-connected cell

Annex	TP for LTM for 38.473

### 8.3.10	CU-DU Cell Switch Notification

#### 8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

#### 8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the TA Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Tag ID Pointer IE is also included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

If the UE History Information IE and is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it for MRO optimization for LTM cell switch. If the ping-pong indicator IE is also incuded, the gNB-DU shall, if supported, consier it to determine and resolve ping-pong LTM cell switch issue based on the UE history information received.

---------------------------------------------------------------next change-----------------------------------------------------------------

8.11.1	Access and Mobility Indication

8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Beam Failure Recovery Information IE, the gNB-DU shall, if supported, consider that a Beam Failure Recovery after successful LTM Cell Switch has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Cell Switch Failure due to Wrong Beam Information IE, the gNB-DU shall, if supported, consider that an LTM Cell Switch Failure due to Wrong Beam has occurred in the target DU for the UE identified by the gNB-DU UE F1AP ID IE, and may take it into account for optimisation of target beam selection.

If the ACCESS AND MOBILITY INDICATION message contains the TA Information IE (name is FFS), the gNB-DU may take it into account for optimisation of Timing Advance related parameters for the UE identified by the gNB-DU UE F1AP ID IE.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Failure information IE, the gNB-DU may take it into account for optimization of LTM cell switch related paramenters.

8.11.1.3	Abnormal Conditions

Not applicable.

---------------------------------------------------------------next change-------------------------------------------------------------------

9.2.2	UE Context Management messages

[skip]

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| UE History Information | O |  | 9.3.1.x |  | YES | ignore |
| Ping-pong indicator | O |  |  | Indicates that a pontential ping-pong occurrence detected by the gNB-CU. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

---------------------------------------------------------------next change-------------------------------------------------------------------

9.2.10	Self Optimisation Support Messages

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU. This IE is included in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| LTM Cell Switch Failure due to Wrong Beam Information | O |  |  |  | YES | ignore |
| >Target Beam Information | M |  | FFS | FFS if it corresponds to the target beam index or TCI state ID. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| TA Information (name is FFS) | O |  |  |  | YES | ignore |
| >TA Value | M |  | INTEGER (0..4095) | Indicates the TA value used at successful Random Access, as defined in TS 38.213 [31]. | - |  |
| >gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | - |  |
| LTM Failure information | O |  | 9.3.1.z | Includes information of a LTM cell switch failure event. | YES | ignore |

Editor’s note: FFS if a common structure is needed for UE F1AP IDs.

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

---------------------------------------------------------------next change-------------------------------------------------------------------

9.3	Information Element Definitions

9.3.1	Radio Network Layer Related IEs

[skip]

#### 9.3.1.x	UE History Information

The UE History Information IE contains information about cells that a UE has been served by in active state prior to the target cell. The overall mechanism is described in TS 36.300 [20].

NOTE:	The definition of this IE is aligned with the definition of the UE History Information IE in TS 38.413 [3].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Last Visited Cell List |  | 1..<maxnoofCellsinUEHistoryInfo> |  | Most recent information is added to the top of this list |
| >Last Visited Cell Information | M |  | 9.3.1.y |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinUEHistoryInfo | Maximum number of last visited cell information records that can be reported in the IE. Value is 16. |

#### 9.3.1.y	Last Visited Cell Information

The Last Visited Cell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited Cell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited NG-RAN Cell Information | M |  | OCTET STRING | Defined in TS 38.413 [3]. |

#### 9.3.1.z	LTM failure Information

The LTM failure Information IE contains information about a LTM failure event.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UE Assistant Identifier | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  |
| C-RNTI | M |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU |
| LTM failure type | M |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell, ...) |  |
| Source cell CGI | M |  | NR CGI 9.3.1.12 | NR CGI of source cell of the LTM cell switch procedure or failed cell of the gNB-DU. |
| Target cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of target cell for LTM cell switch procedure |
| Re-establishment cell CGI | O |  | NR CGI 9.3.1.12 | NR CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the LTM failure |