3GPP TSG-RAN WG3 Meeting #129	R3-255220

Bengaluru, India, 25 – 29 August 2025

Agenda item:	18.2

Source:	Nokia

Title:	[TP for LP-WUS BL CR TS 38.423] Conclusions for LP-WUS open points

WID/SID:	NR_LPWUS

Document for:	Discussion and Decision

# 1	Introduction

At the last RANP#102 meeting, the work item of LP-WUS was agreed in [1] including the following objective:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

The work item has progressed in previous RAN3 meetings reaching agreements and working assumptions while some points remain open. This paper reviews the current agreements and proposes conclusions for the remaining open points.

# 2	Discussion

The LP-WUS feature has been designed to optimize further energy consumption of the device. It provides similar benefits as prior low power features based on wake-up signals such as PEI but with some differences. Even though the feature applies to both idle, inactive and connected state the main RAN3 impact is related to idle/inactive states.

For example, due to the presence of the new WUR (wake up receiver), some further optimization is reached due to the possibility for the UE to not turn on at all the main radio to monitor the PDCCH even for monitoring the wake-up signal. In this so-called “deep sleep” mode, the wake-up signal is monitored by the WUR instead, enabling further energy saving. Moreover, the main radio could be relaxed from doing some of the measurements taken over by WUR.

A summary of the impacts with resolution of open points is listed below per interface.

ALL interfaces

At RAN1#120, RAN1 further decided one codepoint to wake up all UES: the use case can be e.g. a SIB update or the need to address multiple UES which belong to at least two different groups. For this RAN1 decided that a special codepoint value among the 32 values sent over the air interface will be reserved to “wake up all UEs”. The agreement is shown below:

RAN1#118bis:

Agreement

At least the following codepoints are supported for LP-WUS:

- One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS

- One codepoint corresponding to all the subgroups that can be indicated by LP-WUS

- FFS: Additional codepoints

RAN1#120:

Agreement

Confirm the following working assumption with the modification:

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 31 32.

From RAN3 perspective, the gNB-DU should be able to infer when to use this special codepoint over the air interface: in case of SIB Update the gNB-DU can decide, and for the case of multiple UEs to wake up the F1AP Paging message is only per UE and one UE can only belong to one group.

All this means that the “special LP-WUS codepoint” to wake up all UEs over the air interface doesn’t need to be instantiated over the NG/Xn/F1 Paging. We conclude that only 31 codepoints are needed for the LP-WUS Assistance Information over Xn interface.

The remaining question is whether to encode the 31 values from 0 to 30 or from 1 to 31:

We note that RAN2 has the following encoding for the subgroups:

LP-SubgroupConfig-r19 ::=        SEQUENCE {

lp-SubgroupsNumPerPO-r19         INTEGER (1.. maxNrofPagingSubgroupsLP-r19),

lp-SubgroupsNumForUEID-r19       INTEGER (1.. maxNrofPagingSubgroupsLP-r19)                               OPTIONAL,  -- Need S

...

}

maxNrofPagingSubgroupsLP-r19            INTEGER ::= 31      -- Maximum number of paging subgroups per paging occasion for LP-WUS

In order to align with RAN2 we therefore propose to encode from 1 to 31.

Proposal 1: Update the FFS on the range of the LP-WUS CN subgroup ID IE in the baseline CRs TS 38.473/413/423 from 1 to 31.

Xn interface

Besides, RAN3#126 took the following working assumption:

WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

This working assumption was taken under the assumption that the LP-WUS operation is not unnecessarily restricted to the last serving cell. Due to this working assumption, RAN3#127 agreed the baseline CR for TS 38.423 in R3-250913 with the following editor’s note:

Editor’s note: the inclusion of the LP-WUSPS Assistance Information IE is to be finally confirmed based on whether a UE will be configured to monitor LP-WUS, either only in the last used cell, if indication is sent in the system information, or any other cell (after cell reselection).

RAN1/RAN2 does not envision such restriction. Therefore, it is proposed to turn this working assumption into agreement and remove the editor’s note from the XnAP baseline CR.

Proposal 2: Turn the Working Assumption into agreement and remove the editor’s note concerning the presence of the LP-WUSPS Assistance Information IE in the XnAP RAN Paging message.

Emergency Call Impact

The handling of emergency call when using LP-WUS was discussed and resolved at RAN3#127bis.

Extended UE Identity Index value

This issue was solved at RAN3#128 by introducing the Further Extended UE Identity Index value.

Paging loss issue

This topic is addressed in tdoc [2].

# 3	Conclusion and Proposals

This paper has provided a summary of current agreements and has proposed conclusions for the remaining open points. It is proposed to update the baseline CRs along the following lines:

All Interfaces

Proposal 1: update the FFS on the range of the LP-WUS CN subgroup ID IE in the baseline CR TS 38.473/423/413 from 1 to 31.

Xn Interface

Proposal 2: Turn the working assumption into agreement and remove the editor’s note concerning the presence of the LP-WUSPS Assistance Information IE in the XnAP RAN Paging message.

Paging loss issue

This topic is addressed in tdoc [2].

As a result, we propose to agree the TP for LP-WUS BL CR for TS 38.423 in annex A and the TP in [2].

# 4	References

- RP-240801, Low Power Wake up signal and receiver for NR, Vivo, NTT Docomo, Ericsson, Mediatek, Samsung, Sony

- R3-255221, [TP for BL CR LP-WUS TS 38.300] Resolution of Paging Loss

# 5	TP for BL CR LP-WUS TS 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34], and for eDRX or the UE_ID based subgrouping according to TS 38.304 [33]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards the NG-RAN node2.

When available, the NG-RAN node1 shall include the E-UTRA Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the E-UTRA Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE Specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the NR Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the NR Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the NR Paging eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

When available, the NG-RAN node1 shall include the Paging Cause IE in the RAN PAGING message towards the NG-RAN node2. If the Paging Cause IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.331 [10].

When available, the NG-RAN node1 shall include the Hashed UE Identity Index Value IE in the RAN PAGING message towards the NG-RAN node2. If the Hashed UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33] or TS 36.304 [34].

If the PEIPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the NR Paging Long eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the LP-WUSPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the Further Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x	LP-WUSPS Assistance Information

This IE provides the information related to LP-WUS paging subgrouping for a particular UE, as specified in TS 38.304 [33].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUS CN Subgroup ID | M |  | INTEGER (1..31, …) |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

...,

suspend-lower-layers,

resume-lower-layers

}

LPWUSPSassistanceInformation ::= SEQUENCE {

lPWUScNsubgroupID			LPWUSCNsubgroupID,

iE-Extensions				ProtocolExtensionContainer { { LPWUSPSassistanceInformation-ExtIEs} } OPTIONAL,

...

}

LPWUSPSassistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LPWUSCNsubgroupID ::= INTEGER (1..31, ...)

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>